SA WG2 Meeting #S2-165	S2-2410762

14 - 18 October, 2024, Hyderabad, India	(revision of S2-2410179)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5065 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF Provisioning of non-3GPP device identifer information to UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_UIA_ARC conclusion for KI#4 and the UIA_ARC WID, the AF can provision Device Identifiers and their corresponding QoS/Policies parameters associated with a UE subscription to UDR. According to the agreement in the last meeting, the AF provisioning data is stored in the UDR and the Service specific parameter provisioning is used for AF provisioning. This CR reflects this agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding non-3GPP Device Identifier information to Nnef_ServiceParameter service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AF provisioning of Device Identifier of UIA_ARC is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.11.1, 5.2.6.11.2, 5.2.6.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.2.6.11	Nnef_ServiceParameter service

##### 5.2.6.11.1	General

This service is for allowing external party to provision of service specific parameters which can be used for the UE in 5GS. The detailed information is described in clause 4.15.6.7 and 4.15.6.X.

Next CHANGE

##### 5.2.6.11.2	Nnef_ServiceParameter_Create operation

Service operation name: Nnef_ServiceParameter_Create

Description: The consumer stores service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier)

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available), "PLMN ID(s) of inbound roamers", subscribedEvents, notificationDestination, Non-3GPP Device Identifier(s) Information as defined in the clause 4.15.6.X.

If identifiers of target UE(s) or a group of UEs or "PLMN ID(s) of inbound roamers" are not provided, then the Service Parameters shall correspond to any UEs of the PLMN of the NEF using the service identified by the Service Description.

Outputs, Required: Transaction Reference ID, operation execution result indication.

Outputs, Optional: None.

Next CHANGE

##### 5.2.6.11.3	Nnef_ServiceParameter_Update operation

Service operation name: Nnef_ServiceParameter_Update

Description: The consumer updates service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier), Transaction Reference ID.

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available), or "PLMN ID(s) of inbound roamers", Non-3GPP Device Identifier(s) Information as defined in the clause 4.15.6.X.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

END OF CHANGES