3GPP SA WG2 Meeting #165	S2-2410772

Hyderabad, India, 14 – 18 October 2024

Title:	    [DRAFT] LS reply on LS on RAN2 agreements and assumptions for Ambient IoT

Response to:	LS on RAN2 agreements and assumptions for Ambient IoT

Release:	Rel-19

Work Item:	FS_Ambient_IoT_solutions, FS_AmbientIoT

Source:	SA2

To:	RAN2

Cc: 	RAN3, SA3

# Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank RAN2’s LS (S2-2409573/ R2-2406150) on RAN2 agreements and assumptions for Ambient IoT.

Agreements - … - RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 - …..

For above agreement, SA2 would like to confirm there is no discussion in SA2 on per-packet QoS and no per-QoS flow QoS so far and it is not expected to open the discussion in rel-19 scope.

Agreements - As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions.

For above agreement, SA2 would like confirm Step D is needed in order to send the success ack to step C or report information as requested by step C.

Agreements ….. - From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it: - A-IoT paging message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. - Use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command) ……

For above agreement, it is unclear security feasibility for this use case which needs SA3 to confirm. If SA3’s confirmation is positive, from SA2 point of view it is feasible to support “•	A-IoT paging message from the reader contains the command”.

- RAN2 will study the following cases for A-IoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

For above agreement, SA2 would like to confirm both AIoT device and Group ID can be used to filter AIoT devices, but there is also some other filter information which is different from per device ID and group ID but captures the information used to filter AIoT devices. Hence, the above three can be categorized as a filter information, Including the one of the following (and which is used depends on which is provided by the AF):

  - AIoT device ID list,

  - Match information, which is different from AIoT device ID can be used to filter one or multiple AIoT devices.

SA2 also would like RAN2 to feedback the following aspects:

  - What information is required by the CN for NG RAN to allocate resource for the UE reader.

  - whether the following AF provided information is useful for RAN to perform AIoT operation or not:

    - The characteristics of the target AIoT devices, e.g. whether UL and/ DL amplification are supported by AIoT device, whether the device’s UL transmission is generated internally by the device, or be backscattered on a carrier wave provided externally.

    - The frequency supported by the reader

2. Actions:

RAN2:

ACTION: 	SA2 kindly asks RAN2 to consider the above information and provide feedback on above question.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, FL, US

TSG-SA2 Meeting #166AH-e	20 - 24 January, 2025	Electronic meeting

TSG-SA2 Meeting #167		17 - 21 February, 2025	Athens, Greece