3GPP TSG SA WG2#165	            S2-2410821

Hyderabad, India, October 14-18 2024 	(revision of S2-2409974)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5685 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE served by a MWAB: QoS aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This CR introduces the descriptions on how the MWAB supports QoS for the UEs it serves |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added to describe how the MWAB is supporting QoS for the UEs it serves |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MWAB QoS feature cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of CHANGES

### 5.49.1.x	Support of QoS for UEs served by a MWAB

The MWAB-UE may establish and modify the BH PDU Sessions according to the QoS requirements identified by the MWAB-gNB, e.g. based on the QoS information of the PDU Sessions context received at the MWAB-gNB during the PDU Session establishment/modification procedure for UEs served by the MWAB-gNB, and/or based on OAM configuration. The MWAB-UE indicates to the MWAB-gNB the current BH PDU Session's QoS Flow level QoS parameters, e.g. for admission control of the QoS-Flows of the UEs served by the MWAB. Any QoS requirements for the BH PDU Session used for OAM (if different from the one used for the UEs served by the MWAB-gNB), are configured in the MWAB-gNB so these can be indicated to the MWAB-UE when the MWAB-gNB needs a BH PDU session to communicate with the OAM server.

Based on configuration and traffic mapping specified in TS 38.414 [195] and clause 5.7.1.5, the MWAB-gNB and the UPFs in the MWAB Broadcasted PLMN/SNPN set proper marking information (DSCP) for the N3 GTP-U traffic for the UE(s) served by the MWAB-gNB.  More specifically, the following applies:

1)	The MWAB-gNB takes the traffic mapping specified in TS 38.414 [195] into account (i.e. the mapping of DSCP values used for N3 traffic in the MWAB Broadcasted PLMN/SNPN to 5QIs to be used in the BH PLMN/SNPN) to determine the required Packet Filter Set for filtering of packets with the DSCP value associated with the 5QI to be used in the MWAB Broadcasted PLMN/SNPN for the BH PDU sessions, and it provides these to the MWAB-UE. The MWAB-UE may initiate PDU session modification procedure to update the BH PDU session(s) for N3 traffic accordingly. As defined in clause 5.7.1.5, the MWAB-UE SMF may then accordingly configure/update the DL traffic mapping (DL PDRs) in the MWAB-UE’s UPF.

2) 	For UL traffic from the UE served by the MWAB-gNB, the MWAB-gNB sets the DSCP value for the corresponding N3 GTP-U traffic in the MWAB Broadcasted PLMN/SNPN based on configuration and traffic mapping specified in TS 38.414 [195] and clause 5.7.1.5.  The MWAB-UE uses  the QoS requirements defined in bullet 1  provided by the MWAB-gNB to associate the N3 traffic in the MWAB Broadcasted PLMN/SNPN to the corresponding QoS Flows of the BH PDU Sessions.

3) 	For DL Traffic for the UE served by the MWAB-gNB, the UEs UPF in the MWAB-Broadcasted PLMN/SNPN sets the DSCP value for the corresponding N3 GTP-U traffic based on configuration and traffic mapping specified in clause 5.7.1.5. The MWAB-UEs UPF classifies the DL N3 traffic into the corresponding QoS Flows of the BH PDU Session(s) using the DL PDR(s) from bullet 1 above.

End of CHANGES