SA WG2 Meeting #165 	S2-2410869

14 - 18 October 2024, Hyderabad, India

Title:	LS Reply to Service Consumers of Nnwdaf, Ndccf, Nmfaf related services

Response to:	S2-2409586/C3-244437 LS on Service Consumers of Nnwdaf, Ndccf, Nmfaf related services

Release:	Rel-18

Work Item:	     eNA_Ph3

Source:	SA2

To:	CT3

Cc:	-

Contact Person:	Yuang Feng

E-mail Address:	feng.yuang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  S2-2401780/S2-2401781

1. Overall Description:

SA2 would like to thank CT3 for the LS on Service Consumers of Nnwdaf, Ndccf, Nmfaf related services.

Here are the answers of questions:

Question 1:	Is every analytics consumer also a consumer of the Ndccf_DataManagement and Nmfaf_3caDataManagement services or not?

Answer: The analytics consumer requests historical analytics through DCCF by invoking the Ndccf_DataManagement_Subscribe service operation. The DCCF, base on the local configuration, may configure MFAF (via Nmfaf_3daDataManagement_Configure) to map notifications to the analytics consumer. After that, the configured MFAF may use the Nmfaf_3caDataManagement_Notify service operation to send historical analytics to the analytics consumer. Therefore, each analytics consumer of Ndccf_DataManagement may also be a consumer of Nmfaf_3caDataManagement, depending on whether DCCF configures MFAF. Please check clause 6.1.4.5 of TS 23.288 for more details.

Question 2:	If the answer to Question is "Yes", then why are the LMF, OAM and CEF not included in the consumers of Ndccf_DataManagement and Nmfaf_3caDataManagement services in TS 23.288 Table 8.1-1 and Table 9.1-1?

Answer:  SA2 has no consensus about analytics subscription between OAM/CEF and DCCF. However, SA2 has agreed LMF was missing in the table 8.1-1 and table 9.1-1. Therefore, SA2 amended the table in TS 23.288 to resolve this issue. See agreed CRs attached.

Question 3:	Is the ADRF the consumer of Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services? Is Nnwdaf_AnalyticsSubscription/Nnwdaf_AnalyticsInfo service used by the ADRF for collecting analytics in the procedure of Historical Data and Analytics Storage via Notifications?

Answer: The ADRF is a consumer of  Nnwdaf_AnalyticsSubscription service. In step 6b of clause 6.2B.3, in addition to the Nnwdaf_DataManagement_Subscribe, the ADRF may use Nnwdaf_AnalyticsSubscription to subscribe to NWDAF for analytics. SA2 has amended the procedure, see the agreed CRs attached.

2. Actions:

To SA3

ACTION:  SA2 kindly asks CT3 to take the above information into account and update their specifications if necessary.

3. Date of Next SA2 Meeting:

SA2#166			November 18th – 22nd, 2024			Orlando, US

SA2#166-Ad Hoc-e                     January 20th – 24th, 2025                                      Online meeting