SA WG2 Meeting #165	S2-2410879

# Hyderabad, India, 14 – 18 October 2024	(was S2-2410588)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0375 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of eCall over IMS with CEN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson, Nokia, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN, TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CEN has requested alignment of eCall over IMS with CEN requirements for 5 cases where 3GPP and CEN requirements for eCall over IMS differ as described in their LS to SA#104 in SP-240522. SA has agreed alignment for cases 3 and 4 in LS SP-240522 according to their LS reply in S2-2409582/SP-241421. Case 3 concerns an eCall over IMS attempt by the UE where the UE receives a SIP response code of the form 4xx/6xx from the PSAP containing a positive MSD ACK, and where the UE is then required by CEN to not reattempt the eCall but instead wait for a PSAP callback, whereas TS 23.167 Annex H.6 requires an eCall reattempt . Case 4 concerns an eCall over IMS attempt by the UE where the UE receives a SIP 200 OK response from the PSAP containing a negative MSD ACK, and where the UE is then required by CEN to not resend the MSD inband, whereas TS 23.167 clause 7.7.2 requires inband MSD sending. There is a third more subtle case not in the CEN LS where a UE performs domain selection according to Table H.2 in TS 23.167. For rows B, C, D and E in Table H.2, a UE that is not in eCall only mode might receive an eCall request when attached to, registered on or camped on a PLMN with one or more RATs in the PS domain that provide no support for eCall over IMS (ECL=N). Table H,2 allows use of such RATs for a first eCall attempt if CS is not available or for a second eCall attempt if a first eCall attempt over CS fails. However, CEN EN 17184:2024 clause 7.13.1.2 and CEN EN 17905:2024 clause 7.5.4 do not allow an eCall over the PS domain with no support of eCall over IMS if there is another PLMN with a PS RAT that supports eCall over IMS or with a CS RAT that was not yet attempted. The stage 3 change in CP-242256 supports the changes for cases 3 and 4. Changes to TS 23.167 are needed to align eith these cases and to support alignment with CEN for the third case above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for the three cases above to align with CEN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP suppprt for eCall over IMS will not align with CEN which may degrade service and reliability of an eCall over IMS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.2, Annex H.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 24.229 CR 6670 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.7.2	MSD Transfer/Acknowledgement during Session Establishment with a PSAP Not supporting NG-eCall (inband means)

Figure 7.7.2-1 illustrates a high level call flow for an interworking scenario between a UE and a PSAP via the CS domain.

This scenario may occur in the case, when PS access is available, but the UE does not detect that the NG-eCall is supported by the IP-CAN and there is no CS access available. In this case, the UE shall establish a regular IMS emergency call.

A similar scenario may occur in the case, when the UE detects that the IP_C AN supports NG-eCall and initiates an NG eCall but the session is handled by a PSAP not supporting NG-eCall. The only exception would be step 2 and where the SIP INVITE shall contain the initial MSD and the eCall type of emergency service indicator (automatic, manual).

Figure 7.7.2-1: eCall Scenario with PSAP not supporting NG-eCall

1.	The UE performs steps 1 to 6 of the procedure shown in Figure 7.1.

2.	The UE sends an initial emergency SIP INVITE to the IMS core. The SIP INVITE shall be setup as follows:

-	if the UE has not received the "eCall supported" indication, the UE shall include the eCall type of emergency service indication (automatic, manual) and shall not include the initial MSD if the PS access is available;

-	if the UE has received the "eCall supported" indication, the UE shall include the initial MSD and the eCall type of emergency service indicator (automatic, manual).

3.	The IMS core routes the emergency SIP INVITE towards the appropriate PSAP. In this call flow, the appropriate PSAP is accessed over the MGCF and the CS domain after translating the eCall type of emergency service indication into the corresponding MSISDN.

3a.	The SIP INVITE is sent to an MGCF/MGW for interfacing to the CS domain.

3b.	The MGCF sends an IAM towards the appropriate PSAP for the emergency voice call.

4.	The emergency voice call establishment is completed with a voice path only.

5.	If the UE has sent the MSD in step 2 and did not receive either a positive or negative acknowledgement for the initial MSD included in the SIP INVITE, or the UE did not send the MSD in step 2 the UE shall attempt to transfer the MSD to the PSAP via the eCall Inband Modem, as defined in TS 26.267 [42].

# * * * * Next change * * * *

# H.6	eCall over IMS

This clause details the domain priority and selection (see clause 7.3) for a UE that attempts to make an eCall over IMS session using E-UTRAN or NG-RAN radio access networks based on the availability of the CS or PS domains and the network support for IMS emergency, eCall over IMS and IMS voice over PS.

The following table (Table H.2) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an eCall over IMS session is initiated.

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN or NG-RAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | PS on another PS RAT if available with EMS=Y and ECL=Y or CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) (NOTE 2) |
| C | Y | Y or N | N | N | CS if available | PS on another PS RAT if available with EMS=Y or EMS unknown (NOTE 2) |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS (NOTE 2) |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) (NOTE 2) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28] and TS 23.502 [49]. For the purposes of this table, a UE in limited service state shall assume that Voice over IMS over PS sessions is supported when EMS=Y or ECL=Y. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28] and TS 23.501 [48] and TS 23.502 [49]. A UE in limited service state shall use the ims-EmergencySupport broadcast bit (TS 36.331 [55]; TS 38.331 [56]) instead of the Emergency Service Support indicator. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28] and TS 23.501 [48]. NOTE 1: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE, can make a third attempt using CS. NOTE 2: Fallback to the PS domain with ECL=N shall only occur after the UE has attempted but failed to select another PLMN in the PS domain with ECL=Y or in the CS domain according to PLMN selection rules for a UE in eCall only mode in TS 23.122 clause 4.4.3.1.1 [41]. |  |  |  |  |  |  |

NOTE:	If the E-UTRAN and NG-RAN cells available to the UE have different settings, the UE assumes "PS Available" and "ECL" apply to whichever cell is indicated (as defined in TS 23.401 [28] and TS 23.501 [48]) as providing eCall over IMS support. When support by more than one cell is indicated, a UE may select any cell to attempt eCall over IMS according to the UE implementation.

If a UE that is not in limited service state, receives a response from the PSAP for an attempt over PS indicating non-acceptance of the eCall over IMS by the PSAP and a positive acknowledgment for the MSD, the UE shall not reattempt the eCall using another PS RAT or the CS domain and shall instead wait for a callback from the PSAP.

# * * * * End of changes * * * *