3GPP SA WG2 Meeting #165	 S2-2410898

# Hyderabad, India, 14-18 October 2024									                 (was S2-2410080)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3821 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update table on CIoT functionalities for UE specific DRX for NB-IOT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NB_IOTenh3, CIoT_ext |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In rel.16 RAN and SA2 introduced support for UE specific DRX procedures for NB-IoT. The SA2 CR that introduced the functionality in TS 23.401 (CR3558) did not update the tables in Annex M which continue show that UE-specific DRX is not supported in NB-IoT RAT which is incorrect from rel.16 onwards. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the table on Annex M to show the correct information for UE-specific DRX in NB-IoT RAT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR applies to previous versions of the specification (starting from rel.16) but it is not considered FASMO since the normative text is already updated |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

Annex M (informative):
Functions and procedures over NB-IoT RAT

In the case of conflict between the information in this Annex and other information in the main body of the present document, the information in the main body takes precedence.

The following tables list the functions and procedures that are:

-	Supported or not supported over NB-IoT RAT, including whether for CP CIoT EPS Optimisation only, UP CIoT EPS Optimisation only or both.

-	Optional for the UE and/or network when using NB-IoT RAT.

NOTE:	The tables M-1 to M-5 are ordered by clause number according to the present specification. The table M-6 is ordered by clause number according to TS 23.682 [74]. The notation "CP/UP/Both" indicates whether a particular item is supported for CP CIoT EPS Optimisation only, UP CIoT EPS Optimisation only or both.

Table M-1: Clause 4 Concepts

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 4.3.3 | IP header compression |  |  |  |  |
|  | RoHC (NAS) | Yes | CP | Optional | Optional |
|  | RoHC (AS) | Yes | UP | Optional | Optional |
| 4.3.5 | Mobility management functions | Yes | Both |  |  |
|  | Inter-RAT idle mode NAS mobility (WB-E_UTRAN, UTRAN, GERAN) | Yes | Both |  |  |
|  | Reachability management | Yes | Both | ISR Optional | ISR Optional |
|  | TA list management (2) | Yes | Both |  |  |
|  | ISR | Yes | Both |  | Optional |
|  | Mobility restrictions | Yes | Both |  |  |
|  | IMS voice over PS session supported indication | No | N/A | N/A | N/A |
|  | Voice domain preference and UE usage setting | No | N/A | N/A | N/A |
|  | Preferred and supported network behaviour | Yes | Both |  |  |
| 4.3.7.1a.2 | GTP-C Overload Control | Yes | CP | N/A |  |
| 4.3.7.4 | MME control of overload | Yes | Both | N/A |  |
|  | Extended Access Barring | No | N/A | N/A | N/A |
| 4.3.7.4.2.7 | Control Plane data specific NAS level congestion control | Yes | CP |  |  |
| 4.3.12 | IMS emergency session | No | N/A | N/A | N/A |
| 4.3.12a | Support of Restricted Local Operator Service | No | N/A | N/A | N/A |
| 4.3.13 | Closed Subscriber Group | No | N/A | N/A | N/A |
| 4.3.17 | Support for MTC |  |  |  |  |
|  | Low access priority | Yes | Both |  | Optional |
|  | Non-IP data delivery (NIDD) | Yes | Both |  |  |
|  | NIDD SCEF | Yes | CP | N/A (1) | Optional (3) |
|  | NIDD SGi | Yes | Both | N/A (1) | Optional (3) |
| 4.3.18 | Multimedia Priority Service | No | N/A | N/A | N/A |
| 4.3.22 | Power Save Mode | Yes | Both | Optional | Optional |
| 4.3.27a | Restriction of use of Enhanced Coverage for voice centric UE | No | N/A | N/A | N/A |
| 4.3.27b | Restriction of use of Enhanced Coverage for data centric UE | No | N/A | N/A | N/A |
| 4.3.28 | Restriction of use of Enhanced Coverage | Yes | Both |  | Optional |
| 4.3.29 | 3GPP PS Data Off | Yes | Both | Optional | Optional |
| 4.7 | QoS |  |  |  |  |
|  | PDN connection to SCEF | Yes | CP |  | Optional |
|  | Dedicated bearer | No | N/A | N/A | N/A |
|  | GBR bearer | No | N/A | N/A | N/A |
|  | Non-GBR bearer | Yes | UP |  |  |
|  | Rate control (Serving PLMN) | Yes | CP |  | Optional |
|  | Rate control (APN) | Yes | Both |  | Optional |
| 4.7.8 | Inter-UE QoS for NB-IoT UEs using Control Plane CIoT EPS Optimisation | Yes | Yes | N/A | Optional |
| 4.10 | CIoT EPS Optimisation |  |  |  |  |
|  | UP | Yes | UP | Optional | Optional |
|  | CP | Yes | CP |  |  |
| NOTE 1: Whether the non-IP PDN connection is provided via SCEF or over SGi is transparent to the UE. NOTE 2: Required for inter-RAT idle mode mobility. NOTE 3: At least one of NIDD SCEF and NIDD SGi is required. |  |  |  |  |  |

Table M-2: Clause 5.3 Authentication, security and location management

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.3.1 | IP address allocation (1) | Yes | Both | Not required for NIDD |  |
| 5.3.2 | Attach procedure | Yes | Both |  |  |
|  | Normal attach | Yes | Both | Optional (2) |  |
|  | Attach without PDN connectivity | Yes | Both | Optional (2) | Optional |
|  | Emergency attach | No | N/A | N/A | N/A |
|  | SMS transfer without Combined Attach | Yes | Both |  | Optional |
| 5.3.3 | TAU procedure | Yes (3) | Both |  |  |
| 5.3.4 | Service request procedures | Yes | Both (4) |  |  |
| 5.3.4A | Connection Suspend procedure | Yes | UP |  |  |
| 5.3.4B | Data transport in CP CIoT EPS Optimisation | Yes | CP |  |  |
|  | Reliable data delivery with hop-by-hop acknowledgement | Yes | CP | N/A | Optional |
| 5.3.5 | S1 release procedure | Yes (5) |  |  |  |
| 5.3.5A | Connection resume procedure | Yes | UP |  |  |
| 5.3.6A | PDN GW pause of charging procedure | Yes | Both | N/A | Optional |
| 5.3.7 | GUTI reallocation procedure | Yes | Both |  |  |
| 5.3.8 | Detach procedure | Yes (6) | Both |  |  |
| 5.3.10 | Security function | Yes | Both |  |  |
|  | AS security | Yes | UP |  |  |
|  | NAS security | Yes | Both |  |  |
| 5.3.14 | UE radio capability match request | No (7) | N/A | N/A | N/A |
| NOTE 1: Incl. attach without PDN connectivity. NOTE 2: At least one of Normal attach and Attach without PDN connectivity is required. NOTE 3: Incl. inter RAT. NOTE 4: Also supported with CP CIoT EPS Optimisation when the UE/MME also support S1-U data transfer or UP CIoT EPS Optimisation. NOTE 5: Releases the S11-U bearer in the case of CP CIoT EPS Optimisation. RRC Connection release is used with the UE. NOTE 6: ISR aspects optional. NOTE 7: IMS Voice not supported over NB-IoT. |  |  |  |  |  |

Table M-3: Clause 5.4 Session management, QoS and interaction with PCC

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.4.1 | Dedicated bearer activation | No | N/A | N/A | N/A |
| 5.4.2 | Bearer modification with QoS update | Yes | UP | Optional | Optional |
| 5.4.3 | PDN GW initiated bearer modification without bearer QoS update | Yes | UP |  |  |
| 5.4.4 | Bearer deactivation | Yes | UP |  |  |
| 5.4.5 | UE-requested bearer resource modification | Yes | Both | Optional with CP CIoT EPS Optimisation |  |
| 5.4.7 | E-UTRAN initiated ERAB modification procedure | No (1) | N/A | N/A | N/A |
| 5.4.8 | E-UTRAN initiated UE context modification procedure | No (1) | N/A | N/A | N/A |
| NOTE 1: Dual connectivity not supported over NB-IoT RAT. |  |  |  |  |  |

Table M-4: Clauses 5.5 and 5.6 Handover and NACC

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.5 | Handover | No |  |  |  |
| 5.5.1 | Intra-E-UTRAN handover | No | N/A | N/A | N/A |
| 5.5.2 | Inter RAT handover | No | N/A | N/A | N/A |
| 5.6 | Network assisted cell change | No | N/A | N/A | N/A |

Table M-5: Misc.

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 5.8 | MBMS (3) | (3) | (3) | (3) | (3) |
| 5.10 | Multiple PDN support and PDN activation for UEs supporting Attach without PDN connectivity | Yes (1) | Both |  |  |
| 5.11 | UE capability handling | Yes | Both |  |  |
| 5.12 | Warning message delivery | No | N/A | N/A | N/A |
| 5.13 | DRX and UE-specific DRX parameter handling | Yes | Both |  |  |
| 5.13a | Extended idle mode DRX | Yes | Both | Optional | Optional |
| 5.15 | RAN information management procedures | No | N/A | N/A | N/A |
| 5.16 | MME-initiated procedure on UE's CSG membership change | No | N/A | N/A | N/A |
| 5.17 | HeNB multicast packet forwarding function | No | N/A | N/A | N/A |
| NOTE 1: Multiple PDN optional. NOTE 2: void. NOTE 3: See Table M-6. |  |  |  |  |  |

Table M-6:MTC/CIoT-related items in TS 23.682 [74]

| Clause | Clause title | Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NB-IoT RAT | CP/UP/Both | UE | NW |
| 4.5.14.3 | Reliable Data Service | Yes | CP | Optional | Optional |
| 4.5.15, 5.14 | Support of Packet Flow Description management via SCEF | Yes | Both | N/A | Optional |
| 4.5.16 | MSISDN-less MO-SMS via T4 | Yes | Both | Optional | Optional |
| 4.5.18 | MBMS user service for UEs using power saving functions | Yes | Both | Optional | Optional |
| 4.5.19 | Enhancements to Location Services for CIoT | Yes | Both | Optional | Optional |
| NOTE: This table lists MTC/CIoT-related items in TS 23.682 [74] not otherwise specified in the present specification. |  |  |  |  |  |

# >>>> NEXT CHANGES<<<<

# >>>>END OF CHANGES<<<<