3GPP TSG SA WG2 #165	            S2-2410966

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-2410619)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1480 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Architecture for avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE, vivo, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#8 conclusion in TR 23.700-77, the architecture for avatar communications needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of IMS DC based avatar communication is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS DC based avatar communication cannot be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

AC.X	Support of Avatar Communication

AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) may be locally configured in the UE or fetched from the Base Avatar Repository (BAR) by DCAS, and is transferred to the UE through bootstrap data channel or application data channel.

Editor's note:	It is FFS the Avatar related information (e.g. the personal assets such as accessesories, guarmets, and the data required for the user to select one Avatar to use in a call such as thumbnails, location, time, country) that can be provided together with the Avatar IDs in the list.

NOTE:	When the Avatar ID List is downloaded from bootstrap data channel, the DC AS URL is configured in DCSF. The DCSF could also configure user specific DC AS URL in the DCSF service specific data. How the DC AS URL is configured is out of the scope of 3GPP.

After selection of Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar - IDs is provided from the BAR to MF, and it is transferred to the local UE (if not locally available) and/or to the remote UE according to the rendering mode via application data channel if needed.

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the local UE, the remote UE, and/or the network according to the rendering mode.

-	The animation data may be generated by the local UE, the remote UE, or the network (e.g. from audio, video, and/or metadata such as action and facial expression data).

The details of enhanced IMS data channel architecture and procedures are described in following sub-clauses.

Editor's note:            The fetching of Avatar ID List from the BAR is FFS.

Editor's note:            The completion of this clause AC.X.1 is FFS.

AC.X.2	Architecture

Figure AC.X.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Represenation and its associated Avatar ID. One or more Avatar Representation(s) is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR may provide Avatar ID List to the DC Application Server, and the DC Application Server may further provide to DCSF.

-	The BAR sends Avatar Representation to the MF directly.

Editor's note:	The interface(s) between the BAR and the DC Application Server is FFS.

DCSF:

-	The DCSF may request the Avatar ID List to DCAS to fetch it from the BAR, and sends it to MF via MDC1.

MF:

-	The MF sends the Avatar ID List to the UE through bootstrap data channel or application data channel.

-	The MF receives the Avatar Representation from the BAR by using MDC4.

-	The MF supports Avatar media processing and transcoding (e.g. ASR/TTS).

DC AS:

-	The DC AS retrieves the Avatar IDList from BAR and may send it to DCSF via DC3 or DC4 during bootstrap data channel setup procedure for Avatar communication.

-	The DC AS supports Avatar media negotiation and provides the instruction for Avatar media processing.

# * * * * End of changes * * * *