SA WG2 Meeting #S2-165	S2-2411040

14 - 18 October, 2024, Hyderabad, India

Title:	LS Response on slice mapping between the HPLMN and EHPLMN

Response to:	S2-2409590/C4-243606

Release:	Rel-18

Work Item:	TEI18, 5GS_Ph1, 5GProtoc17

Source:	SA2

To:	CT1, CT4

Cc:

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501CR5682

1. Overall Description:

SA2 thanks for CT4 for the LS (C4-243606) on slice mapping between the HPLMN and EHPLMN. SA2 also received the LS (C1-239320) from CT1 and noted the LS at S2-160-Ad Hoc-e . In the CT4 LS the following scenarios are considered

1. The UE enters an EHPLMN whose PLMN code is not derived from the IMSI.

2. The UE doesn’t have S-NSSAI(s) applicable in the EHPLMN and the UE includes HPLMN S-NSSAI(s) in the Requested mapped NSSAI IE.

3. The network maps the HPLMN S-NSSAI(s) to the S-NSSAI(s) of EHPLMN and returns the mapping of the S-NSSAI(s) of EHPLMN and HPLMN S-NSSAI(s) to the UE.

SA2 discussed these LSs and the scenarios above. SA2 agreed that if the UE selects an EHPLMN (including the case if the PLMN ID derived from SUPI is within the EHPLMN list) the network considers this case as non-roaming, and the S-NSSAI of the EHPLMN can be also applicable for HPLMN.

SA2 would like to provide the answers as below:

Q1: Is it necessary for the AMF to perform the mapping of the S-NSSAI(s) of EHPLMN and HPLMN S-NSSAI(s) and provide mapped HPLMN S-NSSAI corresponding to the S-NSSAI(s) of EHPLMN to the UE?

SA2 answer: Yes. In order to avoid the impact on the UE side, when the UE selects a PLMN which is not derived from the SUPI, the network shall provide mapping information to the HPLMN to the UE together with the Configured S-NSSAI or Allowed NSSAI if different values are used.

Q2: If the answer of Q1 is yes, which S-NSSAI(s) (i.e. either HPLMN S-NSSAI or S-NSSAI of EHPLMN, or both) should be used for I-SMF, A-SMF or other NFs selection?

SA2 answer: SA2 agreed that the S-NSSAI of the Serving PLMN is used to select the I-SMF and S-NSSAI of the mapped S-NSSAI in the HPLMN is used to select the SMF for the PDU Session if a mapped S-NSSAI value of the HPLMN is present in the Allowed NSSAI.

Q3: If the answer of Q1 is yes, how is the admission control of the HPLMN S-NSSAI and the mapped S-NSSAI of EHPLMN performed in the 5GC? Are there two NSACFs, i.e. one for HPLMN and the other for EHPLMN?

SA2 answer: When different S-NSSAI values are used for HPLMN and EHPLMN, the (I-)SMF performs NSAC procedures separately for EHPLMN S-NSSAI and HPLMN S-NSSAI. Whether to deploy one NSACF (supporting both S-NSSAIs) or two NSACFs is per operator deployment.

Please see the agreed CR from Rel-18 as attached.

2. Actions:

To CT1 and CT4

ACTION:

SA2 kindly asks CT1 and CT4 to take the information above into account.

3. Date of Next SA2 Meetings:

SA2#166		Nov 18-22, 2024 				Orlando , US

SA2#166-Adhoc-e	Jan 20-24, 2025 				E-meeting