3GPP TSG-SA2 Meeting #165	S2-2411194

# Hyderabad, India, Oct 14 – 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 9 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE, OPPO, China Telecom, ETRI, Huawei, HiSilicon, Nokia, Futurewei, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper adding procedure of Vertical Federated Learning when NWDAF is acting as VFL server |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause for procedure of Vertical Federated Learning when NWDAF is acting as VFL server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3 (new), 6.2H.2.3.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CRs shall be implemented in the following order: CR 1198, CR 1246, CR1235, CR1208, CR1134 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 6.2H.2.3 Training Procedure for Vertical Federated Learning

#### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server.

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

Editor’s Note:	It is FFS if the term “backward local ML model training information” needs to be changed into “intermediate results”.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE: VFL Server determines to perform VFL training based on the internal trigger or local configuration.

The steps 2-6 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server allocates VFL correlation ID. The VFL server sends a request to start the VFL training to each of the selected VFL clients. The VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_VFLTraining_Subscribe or Naf_VFLTrainingInfo_Request to the selected AF clients(s). The request includes VFL correlation ID, VFL capability type (VFL Client).

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the backward local ML model training information to each of the VFL clients for next round of VFL training.

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

Editor’s Note:	It is FFS which service is used between VFL server and VFL clients for VFL training.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and based on possible backward local ML model training information distributed by the VFL server in the previous training iteration, and computes and reports the client intermediate training result of the local ML model to the VFL server.

Editor’s note:	The report may include ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on the client intermediate training result received in step 4, its own local intermediate results and the label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether weight of feature is computed by VFL server is FFS.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor’s Note: The content of the VFL status report is FFS.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed

7.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process. The VFL Server and each VFL Client, stores the VFL correlation ID and latest information related to their locally trained Models

NOTE: The VFL correlation ID is used later for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

# >>>>END OF CHANGES<<<<