3GPP TSG-WG2 Meeting #165	S2-2411257

# Hyderabad, India,  14th - 18th October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0597 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 - Addressing Editor´s Note: Procedure for Data Collection for AI/ML based positioning from NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Xiaomi, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolution of the following Editor´s Note in clause 5.18, see DP, S2-2410215. Editor’s note: The procedure of LMF collects input data from NG-RAN for ML model training is FFS. Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: Further details on user consent for data collection for model training will be aligned with SA WG3. Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS. Other aspects: The current description of the performance monitoring in clause 5.18 is not aligned with the conclusions in the technical report: Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. - Principle #4.1 The result of model performance monitoring may trigger the LMF to change the positioning method, e.g. from AI/ML based positioning to the legacy positioning, or vice versa. - Principle #4.2 The result of model performance monitoring may trigger ML model retraining in the training entity. When the training entity is LMF, the performance monitoring is also performed by LMF as per Principle#4.2. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | A new clause for Data collection from NG-RAN is added, using Network Assisted Positioning Procedure. The LMF uses the same procedure to retrieve and enforce user consent as defined in Rel-18 in TS 23.288, except that NWDAF is not the consumer but the LMF. The LMF triggers data collection based on internal trigger, and requests AMF(s) to provide the list of SUPIs in an area of interest. When the training entity is LMF, the performance monitoring is performed by LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML based positining is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, 6.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0584, CR074 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 6.x.2. The LMF collects input data from NG-RAN for ML model training is described in clause 6.x.3.

LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF. LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37]. LMF requests ML model for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest. NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.x.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37].

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model.  When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model.  LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

Editor's note:	The Performance monitoring procedures are FFS.

* * * Next Change * * *

### 6.x.3	Data collection from NG-RAN for LMF-based AI/ML positioning

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.x.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs.

- The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model for a PRU UE, in this case step 5 follows. If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest that may include a list of TAIs to identify a more concrete list of AMFs.

- For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

- The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

- The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM, alternatively if the UE has PRU functionality, the LMF does not need to check user consent. If user consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 6 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training for LMF-based AI/ML Model positioning will be aligned with SA WG3.

- The LMF request input data for the target UE.

- The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training and the ground truth data is used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 6 may occur in paralell for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

* * * End of Changes * * *