SA WG2 Meeting #S2-165	S2-2409586

14 - 18 October, 2024, Hyderabad, India

3GPP TSG CT WG3 Meeting #136	C3-244437

Maastricht, NL, 19 - 23 August, 2024

Title:	LS on Service Consumers of Nnwdaf, Ndccf, Nmfaf related services

Response to:	-

Release:	Rel-18

Work Item:	eNA_Ph3

Source:	CT3

To:	SA2

Cc:

Contact person:	Xiaojian Yan

yan.xiaojian@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1	Overall description

In TS 23.288,

#### 6.1.4.4	Analytics Exposure via Messaging Framework

The procedure as depicted in Figure 6.1.4.4-1 is used by analytics consumer(s) (e.g. NFs/OAM) to subscribe/unsubscribe to NWDAF analytics and be notified of analytics information, using Ndccf_DataManagement_Subscribe service operation. The 3GPP DCCF Adaptor (3da) Data Management service and 3GPP Consumer Adaptor (3ca) Data Management service of the Messaging Framework Adaptor Function (MFAF) are used to interact with the 3GPP Network and the Messaging Framework. Whether a NWDAF service consumer directly contacts the NWDAF or goes via the DCCF is based on NWDAF service consumer configuration.

…

4.	The DCCF sends an Nmfaf_3daDataManagement_Configure (Analytics Consumer Information, MFAF Notification Information, Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the NWDAF to outgoing notifications sent to endpoints and to instruct the MFAF how to format and process the outgoing notifications.

"Analytics Consumer Information" contains for each notification endpoint, the analytics consumer Notification Target Address (+ Analytics Consumer Notification Correlation ID) to be used by the MFAF when sending notifications in step 7.

…

7.	The MFAF uses Nmfaf_3caDataManagement_Notify to send the analytic to all notification endpoints indicated in step 4. Notifications are sent to the Notification Target Address(es) using the Analytics Consumer Notification Correlation ID(s) received in step 4. Analytics sent to notification endpoints may be processed and formatted by the MFAF, so they conform to delivery requirements specified by the analytics consumer. The MFAF may store the information in the ADRF if requested by consumer or if required by DCCF configuration, using procedure as specified in clause 6.2B.3.

8.	If a Nmfaf_3caDataManagement_Notify contains a fetch instruction, the notification endpoint sends a Nmfaf_3caDataManagement_Fetch request to fetch the analytics from the MFAF.

As per the text above, there are different understanding in CT3 on whether every analytics consumer is also a consumer of Ndccf_DataManagement and Nmfaf_3caDataManagement services or not.

We noticed that LMF is an analytics consumer (i.e. consumer of Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services) but not the consumer of Ndccf_DataManagement and Nmfaf_3caDataManagement services, as shown in the following tables from TS 23.288.

We also noticed that OAM and CEF are analytics consumers but not the consumers of Ndccf_DataManagement and Nmfaf_3caDataManagement services as defined in TS 23.288.

Question 1:	Is every analytics consumer also a consumer of the Ndccf_DataManagement and Nmfaf_3caDataManagement services or not?

Question 2:	If the answer to Question is "Yes", then why are the LMF, OAM and CEF not included in the consumers of Ndccf_DataManagement and Nmfaf_3caDataManagement services in TS 23.288 Table 8.1-1 and Table 9.1-1?

TS 23.288, clause 5A.1

NOTE:	Nnwdaf_DataManagement service can be used to collect historical data or runtime data. For collecting historical and runtime analytics, Nnwdaf_AnalyticsSubscription service is used.

TS 23.288, clause 5B.1

Data may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive data or analytics for storage. The ADRF then subscribes to the NWDAF or DCCF for data or analytics.

However, in the procedure of Historical Data and Analytics Storage via Notifications in clause 6.2B.3 of TS 23.288, only Nnwdaf_DataManagement_Subscribe service operation is supported in step 6b for the ADRF subscribes to the NWDAF for data/analytics, Nnwdaf_AnalyticsSubscription_Subscribe service operation is missing. In addition, ADRF is not the consumer of Nnwdaf_AnalyticsSubscription/Nnwdaf_AnalyticsInfo services as shown in above Table 7.1-1.

Question 3:	Is the ADRF the consumer of Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services? Is Nnwdaf_AnalyticsSubscription/Nnwdaf_AnalyticsInfo service used by the ADRF for collecting analytics in the procedure of Historical Data and Analytics Storage via Notifications?

# 2	Actions

To: SA2

ACTION: 	CT3 kindly asks SA2 to answer the above question and amend 3GPP TS 23.288 if needed.

# 3	Dates of next TSG CT WG 3 meetings

CT3#137	October 10– 14, 2024		Hefei, CN

CT3#138	November 18– 22, 2024		Orlando, US