SA WG2 Meeting #S2-165	S2-2409587

14 - 18 October, 2024, Hyderabad, India

3GPP TSG CT WG3 Meeting #136 		C3-244442

Maastricht, NL, 19 - 23 August, 2024

Title:	LS on Multiple Queries related to untrusted AF influence on Traffic Routing

Response to:	N/A

Release:	Rel - 19

Work Item:	TEI19, eEDGE_5GC, 5GS_Ph1_CT

Source:	CT3

To:	SA2

Cc:

Contact person:	Anusuya Balasubramanian

anu@cewit.org.in

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

In clause 4.3.6 of TS 23.502 the procedure for Application Function Influence on traffic routing is defined. CT3 has queries on two procedural parts related to untrusted AF invoking the Traffic Influence(TI) feature.

## 1.1 Multiple Traffic Influence Request

The TI request from AF can have attributes related to traffic flow and N6 routing information. The attributes "trafficFilters" or "ethTrafficFilters" (as per the stage 3 definition of the Nnef_TrafficInfluence API in TS 29.522) carry the information about traffic flow. The attribute "trafficRoutes" carry N6 routing information. These attributes are defined as arrays which enables the AF to request for one routing requirement in a request.

To enable AF to request for multiple sets of traffic filters and the corresponding traffic routing requirement via a single request, e.g., one traffic flow to DNAI(s) and another traffic flow to other DNAI(s), CT3 agreed the optimization CR (CR#1277, C3-243667) for an ongoing PDU session in CT#135 for Rel-18 as a stage 3 protocol/interface enhancement. The related discussion paper can be found in C3-243064.

Currently in CT3#136, CT3 discussed and agreed in principle to extend this optimization for future PDU session(s) for Rel-19, which will have impacts on the UDR.

CT3 would hence like to ask the following questions:

Question 1:	Whether there are any impacts of these optimizations on the traffic influence functionality from architectural and/or procedural point of view?

Question 2:	Whether CT3 can proceed with this optimization for future PDU session(s) as a pure stage 3 enhancement in Rel-19?

## 1.2 User Plane Path Change Event Notification to untrusted AF

In TI request an untrusted AF can subscribe for User Plane (UP) path change event notification  to NEF. The NEF in turn can subscribe to SMF to receive UP path change notifications. The SMF sends a notification corresponding to this event when the "UE IP address" / "Prefix" and/or "DNAI" and/or "N6 traffic routing information" changes, according to definition of Nsmf_EventExposure Service in clause 5.2.8.3 of TS 23.502.

In a specific use case, when the untrusted AF has requested for routing of multiple traffic flows and subscribes for UP path change notification, there can be two possible scenarios as described below:

Scenario 1: In case enforcement of the requested traffic flow(s) routing are successful for some flows but fails for the rest, then the notification report from SMF conveys NEF that the UP path has changed. It also has provision to indicate traffic flows which are successfully routed. But a subsequent notification from NEF to AF is not provisioned to indicate these traffic flows. So, AF ,on receiving this notification, do not get the right information about the success/failure of routing of various flows given in the request.

Scenario 2: In case if all requested traffic flows fail to get routed, the AF is not notified about this. Even in case of single flow requested by AF, if its failed to get routed, the notification is not provided to AF.

The clause 4.3.6 of TS 23.502 does not provide clarity on whether the User Plane management event notification shall convey the outcome of traffic routing to AF. The failure in enforcement of routing the requested traffic flows is reported from SMF to PCF. According to stage 3 definition of the Npcf_PolicyAuthorization API in TS 29.514, NF service consumer is provisioned to subscribe and get notification for the failure for which NEF and trusted AF are consumers. Based on this, CT3 has agreed other CRs (CR#0752, C3-213551) and (CR#0325, C3-213461) where, UP path change failure event has been added for a trusted AF from PCF in 3GPP CT3#116-e meeting.

Currently in CT3#136 meeting, CT3 is discussing on the CR (CR#1320, C3-244029) to extend the UP path change failure notification to an untrusted AF from NEF in Release-19.

CT3 would hence like to ask the following questions:

Question 3:	Whether the outcome of the Traffic Influence request like success or failure be notified to an AF by NEF so that it can take appropriate actions. If yes, can SA2 indicate the signaling path, the success or failure notification will take?  Like, would it be the existing UP path change notification procedure itself?

Question 4:	Is it within the scope of CT3 to extend the existing notifications to convey both success and failure from NEF to an untrusted AF ?

# 2	Actions

To SA2

ACTION: 	CT3 kindly asks SA2 to answer the above questions and update their relevant specifications accordingly (if needed).

# 3	Dates of next TSG CT WG 3 meetings

CT3#137	October 14, 2024 – October 18, 2024	Hefei, China