3GPP TSG-WG SA2 Meeting #165	S2-2409638

# 14 - 18 October, 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5628 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Energy Efficiency and Energy Saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Orange, SK Telecom, Rakuten Mobile, NEC, Tencent, Toyota, LG Electronics, ETRI, Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This paper provides the basic framework and function design to support energy related information collection, energy consumption information calculation, and exposure. New NF EECF is introduced to support the above functionality. The methods of calculation for the Energy consumption information of different granularities are provided. The Energy Consumption information of the different granularities are derived from user plane traffic transmission, i.e. the combination of Energy Consumption information in NG-RAN, I-UPF(s) and PSA-UPF(s), and the data volume is the main contributor. The Energy Consumption Assisted information include: Node-level energy consumption information, Node-level data volume, and data volume of the required granularities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NF EECF for architecture and function design to support energy consumption information collection and calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy saving features cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AI/ML	Artificial Intelligence/Machine Learning

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AoI	Area of Interest

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

CQRCI	Clock Quality Reporting Control Information

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DetNet	Deterministic Networking

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

EECF	Energy Efficiency and Consumption Function

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FL	Federated Learning

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

L4S	Low Latency, Low Loss and Scalable Throughput

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPQUIC	Multi-Path QUIC

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

MWAB	Mobile gNB with wireless access backhauling

N3IWF	Non-3GPP InterWorking Function

N3QAI		Non-3GPP QoS Assistance Information

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NAT	Network Address Translation

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NS-AoS	Network Slice Area of Service

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PDV	Packet Delay Variation

PEGC	PIN Element with Gateway Capability

PEI	Permanent Equipment Identifier

PEMC	PIN Element with Management Capability

PER	Packet Error Rate

PFD	Packet Flow Description

PIN	Personal IoT Network

PINE	PIN Element

PLR	Packet Loss Rate

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSIHI	PDU Set Integrated Handling Information

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QMC	QoE Measurement Collection

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

RTT	Round Trip Time

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SF	Service Function

SFC	Service Function Chain

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAC	TSC Assistance Container

TSCAI	Traffic Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TSS	Timing Synchronization Status

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * * Second change (all new text) * * * *

5.X	Support of Energy Efficiency and Energy Saving

5.X.1	General

This clause provides an overview of 5GS functionalities to support of energy related information collection, energy consumption information calculation, and exposure. Based on the UE subscription and Energy Consumption information, the 5GS may perform energy saving policies, NF discovery and (re-)selection, and UP path adjustment.

-	The 5GS supports energy related information collection, energy consumption information calculation and exposure, which are specified in clause 5.X.2;

-	The policy control with considering Energy Consumption information is described in clause 5.X.3;

-	Enhancements of NF discovery and (re-)selection considering energy-related information are described in clause 5.X.4;

-	The user plane path adjustment for a PDU session(s) based on Energy Consumption information are described in clause 5.X.5;

5.X.2	Energy Consumption Information calculation and exposure

5.X.2.1	General

The Energy Efficiency and Consumption Function (EECF) is defined to collect and store the Energy Consumption Assisted information, calculate the Energy Consumption information, and expose the Energy Consumption information to the authorized consumer NF(s) subject to operator’s policy.

NOTE: The EECF can be deployed standalone or co-located with NWDAF.

When the authorized consumer is AF, the granularities of Energy Consumption information include: per UE, per UE per application, per PDU session. When the authorized consumer NF is a 5GC NF (e.g., NWDAF, PCF), the granularities of Energy Consumption information include: per UE, per-UE-per-QoS flow, per PDU session.

- AF or 5GC NF (e.g., NWDAF, PCF) subscribe the EECF for Energy Consumption information of the above granularities, with providing parameters including e.g., UE GPSI, UE IP address(es), Flow description(s), reporting frequency, reporting threshold, as described in clause 5.2.X in TS 23.502[x]

The Energy Consumption information of the above granularities are derived from user plane traffic transmission, i.e. the combination of Energy Consumption information in NG-RAN, I-UPF(s) and PSA-UPF(s), and the data volume is the main contributor.

The Energy Consumption Assisted information include: Node-level energy consumption information, Node-level data volume, and data volume of the required granularities. The Node-level Energy Consumption Assisted information received from OAM and SMF/UPF, could be used by EECF for all the UEs serving by the NF Node.

- EECF requests OAM to provide the NG-RAN and UPF Node-level energy consumption information based on a measurement time period T (e.g., with including start/end time) as specified in SA5 TS 28.541.

- EECF requests OAM to provide the NG-RAN Node-level data volume which serving the UE.

- EECF gets the overall data volume of the UPF(s) from SMF/UPF.

- EECF gets the data volume of the granularities e.g., QoS flow or PDU session, from SMF/UPF. And for the same QoS flow or PDU session, the data volume in NG-RAN and UPF(s) are the same.

- When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be sent to the new functionality through AMF/SMF as described in clause 4.3.5.X in TS 23.502[x].

NOTE X1: It is assumed that the measurement time period T is aligned over the UPF/NG-RAN nodes for which information is collected, i.e. the start/stop of the time periods is time-synchronized.

EECF calculates the Energy Consumption information for UPF(s) as described in 5.X.2.2 and calculates the Energy Consumption information for NG-RAN as described in 5.X.2.3.

Editor’s NOTE: Details of OAM expose Node level Energy Consumption information needs the coordination with SA WG2 and SA WG5.

Editor's NOTE:	Details of the exposure and collection of renewable energy information needs the coordination with SA WG2 and SA WG5.

5.X.2.2	Energy Consumption information for UPF(s)

To enable the calculation of Energy Consumption information per UPF(s) in the given granularity level, the EECF needs to get the UPF Node-level energy consumption information from OAM, the Node-level data volume from SMF/UPF, and the data volume of the required granularities (as described in 5.X.2.1) from SMF/UPF.

The data volume of the required granulairities (as described in 5.X.2.1) are obtained as follows:

- Case A) For per UE granularity, the consumer NF (e.g., AF or NWDAF) provides UE ID to EECF, and EECF subscribes the UDM through Nudm_UECM_Get service operation to retrieve the SMF ID(s) which serving the UE. Then EECF gets the UE’s traffic data volume through invoking Nsmf_EventExposure service. SMF requests the corresponding UPF(s) which serving the UE to report the data volume of the UE.

- Case B) For per PDU session granularity, the consumer NF (e.g., AF or NWDAF) provides UE address to EECF. EECF reaches the PCF as described in clause 4.3.6.4 in TS23.502[x] and PCF reports the SMF ID to EECF. Then EECF gets the PDU session’s traffic data volume through invoking Nsmf_EventExposure service. SMF requests the corresponding UPF(s) which serving the PDU session to report the data volume of the PDU session.

- Case C) For per-UE-per-QoS flow granularity, the consumer NF (e.g., NWDAF) provides UE address, Flow Description(s) to EECF. EECF gets the per-UE-per-QoS flow’s data volume with the same procedure as Case B).

- Case D) For per-UE-per-application granularity, the consumer NF (i.e. AF) provides UE address, Flow Description(s) to EECF. The Flow description(s) belong to the per-UE-per-application. EECF gets the per-UE-per-application’s data volume with the same procedure as Case B).

During the above A-D), SMF(s) request the corresponding UPFs to report the Node-level data volume of the UPF node, and then provides to the EECF as well as the UPF ID(s)

EECF requests the UPF node level energy consumption information from OAM based on the UPF node ID reported from SMF.

When (I-)UPF is relocated, the SMF gets the new (I-)UPF ID and update to EECF.

NOTE: For example, the potential formula for Energy Consumption of UE in UPF(s) over a time period T is 
EUE,UPFs= UPFi∈UPFUETEUE, UPFi, where UPFUET are all (I-)UPFs used by the UE in a time period T, and EUE, UPFi is the energy consumption the UE in i-th UPF. One potential approach to estimate is

EUE,UPFi=EUPFiDVUE,UPFiDVUPFi

Where EUPFi is the total energy consumption of the UPF (provided by OAM), DVUE,UPFi is the data volume correspond to the UE in the UPF and DVUPFi is the total data volume of the UPF.

5.X.2.3	Energy Consumption information for gNB

Energy Consumption information at the given granularity level per gNB is calculated based on the gNB Node-level energy consumption information from OAM, gNB Node-level data volume from OAM, and the data volume of the required granularities (as described in 5.X.2.1) which are assumed to be the same value as the data volume in the UPF.

The EECF gets the SMF ID(s) based on the Case A-D, and requests the SMF(s) to report the gNB ID which serving the UE. During the handover, the target gNB ID will be updated in SMF and SMF reports this new gNB ID to EECF.

The EECF requests the OAM to report the Node-level data volume and the Node-level Energy Consumption information with providing the gNB ID.

NOTE: For example, one potential formula for Energy Consumption of UE in gNB over a time period T is EUE,gNBs=gNBi∈gNBUETEUE, gNBi, where gNBUET are all gNBs used by the UE a time period T, and EUE, gNBi is the energy consumption the UE in i-th gNB. One potential approach to estimate it is

EUE,gNBi=EgNBiDVUE,gNBiDVgNBi

Where EgNBi is the total energy consumption of the gNB (provided by OAM), DVUE,gNBi is the data volume correspond to the UE in the gNB and DVgNBi is the total data volume of the gNB.

Editor’s NOTE: It is based on RAN WGs decision whether the NG-RAN support to report UE level energy consumption information.

# * * * * Third change (all new text) * * * *

5.X.3	Policy control with considering energy-related information

Editor’s Note: This clause will specify the enhancements in the policy controls considering energy-related information.

# * * * * Forth change (all new text) * * * *

5.X.4	NF (re)selection with considering energy-related information

Editor’s Note: This clause will specify the enhancements in NF selection and reselection considering the energy-related information.

# * * * * Fifth change (all new text) * * * *

5.X.5	UP Path Adjustment with considering energy-related information

Editor’s Note: This clause will specify adjustment of UP path of PDU sessions considering the energy-related information.

# * * * * End of changes * * * *