3GPP TSG SA WG2 #165	            S2-2409666

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5634 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Energy Saving per UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#2 and 3 conclusion in TR 23.700-66, the subscription data to allow the network energy saving for the user needs to be supported, and the corresponding policy control needs to be performed based on the subscription data and the UE request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New subscription data for the user to allow network energy saving and the corresponding network behaviour are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy saving and energy efficiency cannot be supported completely. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

5.X	Support for Network Energy Saving and Efficiency (NESE)

5.X.Y	Network Energy Saving per UE

The network may use the subscription data to control the network energy saving operations and the policy decisions.

The Network Energy Saving Allowed Indication may be included in the Access and Mobility Subscription data of the UE to indicate whether the network energy saving operation is allowed to be applied for the UE.

When the network energy saving is required, the AMF determines to activate the network energy saving for UEs who are allowed for the network energy saving operations. When the network energy saving is not required anymore, the AMF determines to deactivate the the network energy saving for the UEs.

When the AMF decided to activate or deactivate the network energy saving operations for the UE, the AMF indicates the NES Status to the NG-RAN, and PCF. The network energy saving operations are applied for the UE whose NES Status is set to ‘activated’ in the NG-RAN or PCF.

The AMF indicates the NES Status to NG-RAN during Registration procedure as described in clause 4.2.2.2.2 of TS 23.502 [3], or via UE Configuration Update procedure as described in clause 4.2.4.2 of TS 23.502 [3] for the UE. The AMF also indicate the NES Status to PCF during AM Policy Association Establishment procedure as described in clause 4.16.1.2 of TS 23.502 [3]. The AMF notifies the change of NES Status to PCF to trigger corresponding policy control procedures (e.g. Network Slice Replacement, RFSP index re-allocation) in accordance to the clause 6.1.2.5 of TS 23.503 [45].

# * * * * End of changes * * * *