SA WG2 Meeting #165	S2-2409687

Hyderabad, India, 14 - 18 October, 2024

# Title:	[DRAFT] Reply LS on Service Consumers of Nnwdaf, Ndccf, Nmfaf related services

# Response to:	LS on Service Consumers of Nnwdaf, Ndccf, Nmfaf related services (S2-2409586 / C3-244437)

# Release:	Rel-18

# Work Item:	eNA_Ph3

Source:	vivo [to be SA WG2]

To:	CT3

CC:

Contact person:	Vivian Chong

vivian.chong@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.288 CR#xxxx

1	Overall description

SA2 thanks CT3 for its question on service consumers of NWDAF related services. SA2 would like to provide the answers to the following questions:

Question 1:	Is every analytics consumer also a consumer of the Ndccf_DataManagement and Nmfaf_3caDataManagement services or not?

Answer 1: Yes, every analytics consumer is also a consumer of the Ndccf_DataManagement and Nmfaf_3caDataManagement services. For Ndccf_DataManagement service, OAM, CEF and LMF can be the service consumers to obtain the historical analytics (see clause 6.1.4). For Nmfaf_3caDataManagement service, the DCCF utilizes Nmfaf_3daDataManagement_Configure to configure MFAF to map notification endpoints to the analytics consumers and instruct the MFAF how to send the notifications to the analytics consumers (see clause 6.1.4.4 and 6.1.4.5). In other words, the consumers of Ndccf_DataManagement service can also be the consumers of Nmfaf_3caDataManagement service.

Question 2:	If the answer to Question is "Yes", then why are the LMF, OAM and CEF not included in the consumers of Ndccf_DataManagement and Nmfaf_3caDataManagement services in TS 23.288 Table 8.1-1 and Table 9.1-1?

Answer 2: TS 23.288 does have some errors, some refinements will be done with a CR.

Question 3:	Is the ADRF the consumer of Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services? Is Nnwdaf_AnalyticsSubscription/Nnwdaf_AnalyticsInfo service used by the ADRF for collecting analytics in the procedure of Historical Data and Analytics Storage via Notifications?

Answer 3: SA2 confirms the ADRF is the consumer of Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services. ADRF can use Nnwdaf_AnalyticsSubscription service to subscribe to the NWDAF to receive notifications in clause 6.2B.3.

2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above information into consideration.

3	Dates of next SA WG2 meetings

SA2#166:	18 - 22 November, 2024	Orlando, USA

SA2#166AH-e:	20 - 24 January, 2025	Electronic meeting

SA2#167:	17 - 21 February, 2025	Athens, Greece