3GPP TSG-SA2 Meeting #165	S2-2409700

# Hyderabad, India, October 14-18, 2024          			                 		 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0138 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the USS triggered network-assisted DAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#164 meeting, CR (S2-2409506) was agreed to update the conclusion for Key Issue#2 “Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information” in TR 23.700-59 to use the relative location of UAVs provided by ranging service to enable NWDAA. This conclusion is not reflected in the procedure for USS triggered network-assisted DAA in clause 5.14.4. So it is proposed to update the procedure accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the USS triggered network-assisted DAA procedure to add the feature of using relative location of UAVs provided by Ranging service to enable NWDAA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of using relative location of UAVs provided by ranging service to enable NWDAA is not reflected in the USS triggered network-assisted DAA procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.14.4	Procedure for USS Triggered Network-assisted DAA

Figure 5.14.4-1: Procedure for USS Triggered Network-assisted DAA

1.	The UAV and UAV-C establish PDU Session for communication with the USS as described in clause 5.2.3.

2.	The USS may decide to trigger NWDAA based on application layer request.

3.	The USS requests absolute location of UAV(s) based on the identifiers received in step 2 by invoking the 5GC-MT-LR procedure for the commercial location service in clause 6.1.2 or Initial and Reporting of Location Events in clause 6.3.1 of TS 23.273 [8].

The USS may also request GMLC service via NEF for Ranging/Sidelink Positioning location results, i.e. the relative positioning between UAV UEs as described in clause 6.20.3 of TS 23.273 [8] for one notification, or clause 6.20.4 of TS 23.273 [8] for notifications.

4.	[Optional] The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [20] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [20]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)).

5.	[Optional] If the step 4 is performed, after NEF receive the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [20]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

6.	[Optional] If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [20] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [20].

7.	The USS performs step 7a and 7b or step 7C.

7a.	If the USS detects the potential collision based on the information received in step 3 and/or step 6, the USS notifies the potential collision to UAV-C.

7b.	[Conditional] If the step 7a is performed, the UAV-C further notifies the potential collision to UAV.

7c.	If the USS detects the potential collision based on the information received in step 3 and/or step 6 is performed, the USS notifies the potential collision to UAV UE.

8.	If the USS detects the potential collision based on the information received in step 3 and/or step 6, the USS notifies to potential collision to AAM.

NOTE:	The notification in steps 7a, 7b and 8 is out of 3GPP scope. As an example, the notification may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

9.	[Conditional] If the step 8 is performed, to avoid confliction, steps 6-8 in high-level procedure for Ground-based DAA for an Area in clause 5.7.2 are performed.

10.	[Conditional] If the step 7b or step 7c is performed, to avoid confliction, steps 4a-7 in DAA procedure based on PC5 in clause 5.6.1 are performed.

* * * End of Change * * *