SA WG2 Meeting #165	S2-2409706

Hyderabad, India, 14 – 18 October 2024	(was S2-24XXXXX)

Source:	vivo

Title:	Conclusion on Topology2 Architecture, protocol stack, CP & UP

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: it proposes to the Topology2 Architecture, protocol stack, CP & UP.

1 Discussion

1.1 Solution summary for CP/UP data forwarding methods for Topology 2

In TR 23.700-13, the following solutions related with AIoT CP and UP data forwarding methods for Topology 2 are summarized in Table 1.

Table 1: Solution summary for CP/UP data forwarding methods for Topology 2

| Solution # | Supported Topologies | CP-based or UP based, or selection (for T2) | New NF or other NFs needed | Transmission Path and Protocol aspects | UL Data aggregation |
| --- | --- | --- | --- | --- | --- |
| Solution #3: | Topology 1 & 2 | UP-only | Standalone AIoT controller; For T2: UPF, AMF, SMF. | PDU session between UE reader and UPF; Reader Application Protocol between UE reader and AIoT controller |  |
| Solution #4: | Topology 1 & 2 | CP-based; UP-based; CP and UP selection during AIoT service procedures. | AMF with AIoT Function functionality; CP-based: AMF, UP-based: AMF, SMF, UPF | CP; UP: PDU session between UE reader and UPF; UPF->AS |  |
| Solution #10: | Topology 1 & 2 | CP-based | AMF with AIoT Function functionality; AMF, UDM, NEF |  | Don't suggest that UE Reader and RAN reader perform a response aggregation operation. |
| Solution #11: | Topology 2 | CP-based | AIoT Function; AMF, UDM, NEF, AF | AIoT devices do not have NAS layer. Similar to L-3 relay, not clearly descripted. |  |
| Solution #17: | Topology 1 & 2 | CP-based | New AIoT Function Function; AMF, UDM, AAA-Proxy, AUSF, NEF, CHF, AF, AAA server | AIoT device has NAS layer with AIOTF |  |
| Solution #21: | Topology 2 only | CP-based | New AIoT Function, AIoT Function interacts with RAN via AMF; AMF, NEF | New “AIoT-NAS layer” between AIoT device and AIoT Function. |  |
| Solution #22: | Topology 2 only | CP-based; UP-based; CP and UP selection by AF (not clear mentioned) | No new network function or interfaces. AMF, SMF, UPF, UDM, NEF, AF | CP: utilizing the Control Plane CIoT 5GS Optimisation. AF<->NEF<-> SMF<->AMF<->NG-RAN<-> UE reader. UP: PDU session between UE reader and UPF. UE AIoT layer: between AF and UE reader. Command layer: between device and AF. | UE reader |
| Solution #28: | Topology 1 & 2 | CP- based | AIoTMF/AMF; UDM, NEF |  |  |
| Solution #30: | Topology 2 only | CP- based | New AIoT Function: AIoT Function interacts with RAN via AMF; UDM, AMF, NEF | New “AIoT UE-NAS layer” over UE NAS between UE Reader and AIoT Function. New “AIoT Device NAS layer” between device and AIoT Function. | AIoT Function. |
| Solution #31: | Topology 1 & 2 | CP- based | AIoT Function or AMF. AIoT Function interacts with RAN via AMF; UDM, NEF, AF |  | BS reader and UE reader |
| Solution #32: | Topology 2 only | UP- based | AIoT Function. AIoT Function interacts with RAN via UPF; AMF, UPF, SMF, UDM, PCF, NEF | New “AIoT NAS layer” between device and AIoT Function. PDU session between UE reader and UPF. |  |
| Solution #33: | Topology 1 & 2 | CP- based | New AIoT Control Function (can be integrated with AMF) AIoTCF interacts with RAN directly. | New “AIoT NAS layer” between device and AIoT Function. New AIoT NGAP btweeen RAN and AIoTCF. |  |
| Solution #42: | Topology 2 only | CP-based; UP-based; Selection during AIoT service procedure. Option1:AIoT Function based on the AIoT service requirements and the UE capability. Option 2: The gNB selects the AIoT data forwarding method. | AIoT Function. CP: interacts with RAN via AMF. UP: UE reader<->RAN<->AIoT Function; Others: AMF, NEF,AF，UPF, SMF |  |  |

1.2 Main issues and proposals for CP/UP data forwarding methods for Topology 2

For the solutions in Table 1, following issues related with CP and UP data forwarding methods for Topology 2 need to be discussed, evaluated and concluded.

- Issue 1：Topology 2 control plane data forwarding architecture

- Option 1: AIoT data forwarding path: AIoT Function interacts with RAN via AMF.

Figure 1.1 CP data forwarding method- Option 1

- Option 2: AIoT data forwarding path: AIoT Function interacts with RAN directly.

Figure 1.2 CP data forwarding method- Option 2

- Option 3: AIoT data forwarding path: Control Plane CIoT 5GS Optimization: AF<->NEF<->SMF<->AMF<->RAN<->UE reader. Without AIoT Function

Figure 1.3 CP data forwarding method- Option 3

Regards to the above options, the following evaluation is provided.

Table 2 Evaluation on CP data forwarding method options

| Options | Pros | Cons | Sol# |
| --- | --- | --- | --- |
| 1 | With little impact on the interface between RAN and CN. Reusing the N1 NAS messages for UE reader registration and AIoT service operation. | - CP based method is not efficient to support great amount data. | #4, 10，11，17，21, 28, 31, 42, S2- Note: The solutions in which AIoT Function is colocated with AMF are categraded into option 1a. |
| 2 | AIoT Function interacts with RAN directly to reduce the latency of the AIoT services and complexity of the deployment. | - CP based method is not efficient to support great amount data. - New interface between RAN and AIoT Function. - AIoT Function has to support AMF similar fucntionalities, which is simillar as a co-located case of AIoT Function and AMF of option1 | #33 |
| 3 | Reusing the existing CIoT procedures. | Not necessary to couple CIoT and AIoT | #22 |

Considering that AIoT Function would be supported in Topology 1, it would be reused in Topology 2. Regards to the CP data forwarding method for AIoT service, Option 1 has minimal impacts on RAN and CN. So it is proposed to support option 1 for CP data forwarding for AIoT services.

Proposal 1: It’s proposed to support CP data forwarding method for AIoT services. In the CP data forwarding, the AIoT Function interacts with RAN via AMF.

- Issue 2：Topology 2 user plane data forwarding architecture

- Option 1: user plane based data forwarding between the UE reader and h AIoT Funciton.

Figure 2.1 UP data forwarding method- Option 1

- Option 2: user plane based data forwarding between the UE reader and AF.

Figure 2.3 UP data forwarding method- Option 2

Regards to the above options, the following evaluation is provided.

Table 3 Evaluation on UP data forwarding method options

| Options | Pros | Cons | Sol# |
| --- | --- | --- | --- |
| 1 | - user plane based data forwarding between the UE reader and With AIoT Funciton. - it is more aligned with and compatible with T1 since T1 has agreed to have a AIoT Function |  | #3, #32, #42 |
| 2 | - user plane based data forwarding between the UE reader and AF - The AIoT function is no necessary. | - It is not compatible with T1 which has agreed to have an AIoT Function | #4 |

Considering that both CP and UP solutions could satisfy the data forwarding requirements of AIoT service, it is proposed to support both CP and UP solutions for AIoT data forwarding.

Regards to the UP solutions, Option 1 which is more compatible  with T1 design is proposed to be supported.

Proposal 2: It is proposed to support both CP and UP data forwarding methods for AIoT services.

2 Proposal

It is proposed to add the following conclusions on AIoT data forwarding methods for Topology 2 to TR 23.700-13.

* * * * Start of Changes (all text new)* * * *

8.x Conclusion for Topology 2

8.x.y Conclusion on Architecture for Topology 2

The following aspect are agreed for the architecture to support topology 2(CP and UP agnostic):

-	A new core network function AIOT Function is introduced to support Ambient IoT for Topology 2, which is responsible for the AIoT operation granularity management.

-  The AMF is responsible for the per UE reader management, including: authorizing a registered UE to act as UE Reader and reader configuration.

-  The UDM stores the UE subscription data for UE reader.

The AIoT data forwarding between UE reader and AIoT Function could be CP-based or UP-based and the UE may support CP based and/or UP based method.

-  The CP based architecture is as shown in Figure 8.x.y-1 for topology 2:

Figure 8.x.y-1: CP Based architecture for Topology 2

-  The UP based architecture is as shown in Figure 8.x.y-1 for topology 2:

Figure 8.x.y-1: UP Based architecture for Topology 2

* * * * Second change (all text new)* * * *

8.x.z Conclusion on the Protocol Stack and AIoT data forwarding methods for Topology 2

8.x.z.1 Common part (CP and UP agnostic)

There is an AIoT NAS layer between UE reader and AIoT Function as shown in Figure 8.x.z.2-1 and Figure 8.x.z.3-1.

8.x.z.2 Conclusion on the Protocol Stack for CP based method

For the AIoT control-plane based data forwarding method, followings are supported:

  - The AIoT Function interacts with AMF directly via service-based interface.

  - The AIoT data forwarding between UE reader and AMF should be over N1 NAS message.

The Protocol Stack for control plane AIoT data forwarding is as shown in the following figure:

Figure 8.x.z.2-1: Control plane for AIoT CP-based data forwarding for topology 2

8.x.z.3 Conclusion on the Protocol Stack for UP based method

For the AIoT user plane data forwarding method, followings are supported:

- The UE reader is configured with the DNN/S-NSSAI for PDU session for AIoT data forwarding in the registration accept message or by pre-configuration.

The Protocol Stack for control plane AIoT data forwarding is as shown in the following figure:

Figure 8.x.z.3-1: User plane for AIoT UP-based data forwarding for topology 2

* * * * End of Changes * * * *