3GPP SA WG2 Meeting #165	S2-2409716

# Hyderabad, India, 14 – 18 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1205 | rev | x | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedure of sample alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in Conclusions for KI#2: 5GC Support for Vertical Federated Learning, the NWDAF acting as VFL Server should obtain the samples from the VFL clients, and generates the intersection of samples. P#2.3.1: For NWDAF acting as VFL Server, NWDAF triggers sample alignment, may query NWDAFs or AFs acting as VFL clients to query availability of samples, and generates the intersection of samples. P#2.3.2 In case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information), then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training. P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples. NOTE 4: How NEF assists sample alignment in P#2.3.3 will be determined in normative phase. NOTE 5: For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase. Following the above conclusion, this CR proposes to add new clauses to introduce the procedure for sample alignment when the NWDAF and AF acting as VFL Server and how NEF supports the sample alignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses for sample alignment when NWDAF as VFL server and AF as VFL Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature of sample alignment when the NWDAF or AF acts as VFL Server is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y (new), 6.2X.Z (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (All new text) * * *

6.2X.Y	Procedure for sample alignment when NWDAF acts as the VFL Server

Figure 6.2X.Y-1: Sample alignment procedure when NWDAF acts as the VFL server

1a. [Conditional] The NWDAF as VFL server starts samples alignment procedure with other NWDAF(s) as VFL client(s) using Nnwdaf_VFLSampleAlign_Request service with Analytics ID, Feature spaces information, and optional analytics filter information, then it will obtain sample IDs for the VFL process from other NWDAF(s) as VFL client.

Editor’s Note: Whether and how to transports feature space information is FFS.

1b.	[Conditional] The NWDAF as VFL server starts samples alignment procedure with trusted AF(s) as VFL client(s) using Naf_VFLSampleAlign_Request service with Analytics ID, Feature spaces information, optional analytics filter information to obtain sample IDs for the VFL process from the AF(s) as VFL Client.

1c.	[Conditional] If an AF as VFL client is an untrusted AF, the NWDAF as VFL server sends sample alignment request to the NEF by invoking Nnef_VFLSampleAlign_Request service with Analytics ID (i.e. internal Analytics ID), AF ID(s) obtained after VFL clients discovery as described in clause X, Feature spaces information, optional sample IDs (i.e. internal UE IDs) selected by the NWDAF as VFL server and analytics filter information. The NEF maps the Analytics ID to an external one and sample IDs to external ones.

2.	[Conditional] If NEF received VFL sample alignment request in step 1c, the NEF sends a sample alignment request to the untrusted AF(s) acting as VFL client(s) corresponding to the AF ID(s) to request the sample IDs from the untrusted AF.

3.	[Conditional] The untrusted AF selects sample IDs (i.e. external UE IDs) and sends them to the NEF.

4.	[Conditional] Based on the sample IDs from each untrusted AF as VFL client received in step 3 and the sample IDs from the NWDAF as the VFL server received in step 1c, the NEF converts the sample IDs into one format, and generates an intermediate intersection of sample IDs.

5a. [Conditional] The NWDAF as VFL client returns the selected sample IDs to the NWDAF as VFL server by using Nnwdaf_VFLSampleAlign_Request Response service.

5b.	[Conditional] If an AF as VFL client is a trusted AF, the AF as VFL client returns its selected sample IDs (i.e. external UE IDs) to the NWDAF as VFL server by using Naf_VFLSampleAlign_Request Response service. The NWDAF as VFL server may interact with the NEF to convert the external UE IDs into internal UE IDs.

5c.	[Conditional] If an AF is an untrusted AF, the NEF returns the intermediate intersection of samples (i.e. internal ones) to the NWDAF as VFL server by using Nnef_VFLSampleAlign_Request Response service.

6.	The NWDAF as VFL server determines final sample IDs based on the intermediate intersection of samples received in step 5c and/or selected sample IDs in step 5a and 5b.

7a.	[Conditional] The NWDAF as VFL server sends the final samples to each NWDAF as VFL client by using the Nnwdaf_VFLSampleNotify service.

7b.	[Conditional] The NWDAF as VFL server sends the final samples to each AF as VFL client by using the Naf_VFLSampleNotifyt service.

7c.	[Conditional] If an AF is an untrusted AF, the NWDAF as VFL server sends the final samples to the AF(s) via NEF by invoking Nnef_VFLSample_Notify service.

8.	[Conditional] The NEF sends the final samples to the untrusted AF(s) as VFL client(s).

* * * Next Change (all new text) * * *

6.2X.Z	Procedure for sample alignment when AF acts as the VFL Server

Figure 6.2X.Z-1: Sample alignment procedure when AF acts as the VFL server

1. 	[Conditional] If the VFL Server is an untrusted AF, the AF as VFL server starts samples alignment procedure with NWDAF(s) as VFL client via NEF. The AF sends the Nnef_VFLSampleAlign_Request service to the NEF with temporary NWDAF ID(s) obtained after VFL clients discovery as described in clause X, Analytics ID (i.e. external Analytics ID), sample IDs (i.e. external UE IDs) of VFL Server, Feature spaces information, and optional analytics filter information.

Editor’s Note: Whether and how to transports feature space information is FFS.

[Conditional] If the VFL Server is a trusted AF, The AF as VFL server starts samples alignment procedure with each NWDAF as VFL client using Nnwdaf_VFLSampleAlign_Request service with Analytics ID (i.e. internal Analytics ID), Feature spaces information, optional sample IDs (i.e. internal UE IDs) of VFL Server, and optional analytics filter information, then it will obtain sample IDs for the VFL process from each NWDAF as VFL client.

2.	[Conditional] If NEF received VFL sample alignment request in step 1, for the each NWDAF as VFL client corresponding to the temporary NWDAF ID, the NEF sends the Nnwdaf_VFLSampleAlign_Request service with Analytics ID (i.e. mapped internal Analytics ID), Feature spaces information, optional sample IDs (i.e. mapped internal UE IDs) of VFL Server, and optional analytics filter information to request sample IDs.

3.	[Conditional] If the VFL Server is an untrusted AF, each NWDAF as VFL client returns the selected sample IDs (i.e. internal UE IDs) to the NEF by using Nnwdaf_VFLSampleAlign_Request Response service.

[Conditional] If the VFL Server is a trusted AF, each NWDAF as VFL client returns the selected sample ID(s) to the AF as VFL server by using Nnwdaf_VFLSampleAlign_Request Response service.

4.	[Conditional] Based on the sample IDs from each NWDAF as VFL client and the ones from the AF as the VFL server, the NEF generates an intermediate intersection of sample IDs received samples in step 1 and 3.

5.	[Conditional]The NEF sends the intermediate intersection of sample IDs (i.e. external ones) to the AF as VFL server.

6.	The AF as VFL server determines the final sample IDs based on the ones from NEF in step 5, or from NWDAF in step 3.

7.	[Conditional] If the VFL Server is an untrusted AF, it sends the final sample IDs to the NEF by using the Nnef_VFLSampleNotification_request service with final sample IDs (i.e. external ones), and temporary NWDAF ID(s).

[Conditional] If the VFL Server is a trusted AF, it sends the final samples to each NWDAF(s) as VFL client by using the Nnwdaf_VFLSampleNotification_request service.

8．[Conditional] The NEF sends the final sample IDs to each NWDAF as VFL client by using the Nnwdaf_VFLSampleNotification_request service with final sample IDs (i.e. internal ones).

* * * End of Changes * * *