SA WG2 Meeting #165	S2-2409720

# Hyderabad, India, 14 – 18 October, 2024                        (revision of S2-240xxxx)

Source: 	CATT

Title: 	Conclusions for KI#1

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper makes some conclusions for KI#1.

1	Discussion

Key Issue #1 in TR 23.700-13 is to address the architecture aspect of Ambient IoT as followings:

5.1	Key Issue #1: Architecture support of Ambient IoT Devices

5.1.1	Description

This key issue will address the system architecture to support Ambient IoT Devices, especially on the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

-	Authentication and authorization for the Ambient IoT Device;

-	Validation of the Ambient IoT Device identifier;

NOTE 1:	Format of the Ambient IoT Device identifier is addressed in KI#2.

-	Whether and how to secure device operations and services for an Ambient IoT Device or a group of Ambient IoT Devices;

NOTE 2:	This key issue will take into account the outcome of RAN study in TR 38.769 [8].

NOTE 3: 	The security aspects related to this key issue, including the enable/disable device operation, requires coordination with SA WG3.

In the last SA2#164 meeting, conclusions for KI#1 was initially discussed and one solution principle was agreed:

8.1	Conclusion on Key Issue #1

8.1.1	Architecture to Support Topology 1

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

At least the following principles are agreed for the architecture to support topology 1:

-	A new core network function is introduced to support Ambient IoT.

Editor's note:	Whether the new core network function also applies to topology 2 is FFS.

This paper further discussed the architecture options and made some conclusion proposals.

1) Interaction between AIoTF and RAN Reader/UE Reader

Based on the SA2#164 agreed conclusions, a new core network function is introduced to support Ambient IoT for Topology 1. Assume the new core network function is named as AIoT Function (AIoTF), the interface between AIoTF and RAN Reader for Topology 1 needs to be decided.

For Topology 1, in summary of the solutions in TR 23.700-13, there are two options: one is AIoTF interacts with RAN Reader directly (Option A), and the other is AIoTF interacts with RAN Reader via AMF (Option B).

Figure 1a: Topo 1 Option A                         Figure 1b: Topo 1 Option B

For Option A (Topo 1), AIoTF interacts with RAN Reader directly via a new defined Nxx interface and the AIoTF is responsible for AIoT services handling. The Option A has less dependency on the existing 5G architecture (e.g. no AMF involved) and facilitates fast deployment of AIoT services, however the Option A architecture is difficult to accommodate Topology 2 (in which AMF is mandatory) and to support other services (e.g. location service in R20).

For Option B (Topo 2), AIoTF interacts with RAN Reader via AMF and the AIoT services handling may be transparent to AMF or not. The Option B relies more on the existing 5G architecture and is easy to accommodate Topology 2 and to support other services (e.g. location service in R20).

From SA2 point of view both Option A and Option B are feasible for Topology 1 and it is operator choice for deployment option.

Proposal 1a: It is proposed to conclude that for Topology 1, AIoTF may interact with RAN Reader directly via a new defined Nxx interface or AIoTF may interact with RAN Reader via AMF.

For Topology 2, the AIoTF is also necessary to handle the AIoT Services. Based on the solutions of TR 23.700-13, the architecture for Topology 2 can be categorized into UP based architecture (Figure 2a) and CP based architecture (Figure 2b).

Figure 2b: CP based Topo 2 architecture

Figure 2a: UP based Topo 2 architecture

For UP based solution, the AIoTF interacts with UE Reader via User Plane signalling. For CP based solution, the AIoTF interacts with UE Reader via NAS signalling. Both UP based solution and CP based solution are feasible for Topology 2 and one solution can be selected based on, e.g. AIoT services data volume.

Proposal 1b: It is proposed to conclude that for Topology 2, AIoTF may interact with UE Reader via user plane signalling or NAS signalling.

2) Interaction between AIoTF and AIoT Device

As RAN2 has assumed no AS security is support between AIoT Device and RAN Reader/UE Reader, it is necessary to support security mechanism at NAS layer to fulfil the SA1 requirements and to make 3GPP defined AIoT solution as competitive to RFID solution. Considering the nature of the AIoT Devices (e.g. ultra-low complexity power, cost and resource-constrained), the AIoT NAS message is a light weight NAS message.

For Topology 1, the protocol stacks for Topology 1 Option A and Option B are shown in Figure 3a and Figure 3b respectively.

Figure 3a: Protocol Stack for Topology 1 Option A

Figure 3b: Protocol Stack for Topology 1 Option B

For Topology 2, the protocol stacks for UP based architecture and CP based architecture are shown in Figure 4a and Figure 4b respectively.

Figure 4a: Protocol Stack for Topology 2 UP based architecture

Figure 4b: Protocol Stack for Topology 2 CP based architecture

Proposal 2: It is proposed to conclude that AIoTF interacts with AIoT Device via AIoT NAS message.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1	Conclusion on Key Issue #1

8.1.1	Architecture to Support Topology 1

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

At least the following principles are agreed for the architecture to support topology 1:

-	A new core network function is introduced to support Ambient IoT.

-	AIoTF may interact with RAN Reader directly via a new defined Nxx interface or AIoTF may interact with RAN Reader via AMF.

-	AIoTF interacts with AIoT Device via AIoT NAS message.

8.1.X	Architecture to Support Topology 2

At least the following principles are agreed for the architecture to support topology 2:

-	A new core network function is introduced to support Ambient IoT.

-	AIoTF may interact with UE Reader via user plane signaling or NAS signaling.

-	AIoTF interacts with AIoT Device via AIoT NAS message.

# * * * * End of Changes * * * *