SA WG2 Meeting #165	S2-2409721

# Hyderabad, India, 14 – 18 October, 2024                        (revision of S2-240xxxx)

Source: 	CATT

Title: 	Conclusions for KI#2

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper makes some conclusions for KI#2.

1	Discussion

The Key Issue #2 is documented in TR 23.700-13 as followings.

5.2	Key Issue #2: Identification, Subscription, Registration and Connection management

5.2.1	Description

This Key Issue pertains to the authorization and management of Ambient IoT Devices to support Ambient IoT services.

Considering that Ambient IoT Devices are a new type of reduced capabilities devices, the existing subscription model may not be suitable. Specifically, there is the need to study the device identification method to support Ambient IoT devices which are under operator control.

Based on the above consideration, the aspects to be studied in this key issue include:

-	Study whether subscription management, registration management and/or connection management are necessary for an Ambient IoT Device or a group of Ambient IoT Devices, and if so identify the necessary state machine(s), procedures and functionality considering the Ambient IoT Devices capability and characteristics.

-	Study whether and how reachability and paging apply to Ambient IoT Device(s) considering the Ambient IoT devices capability and characteristics, and if so, what are the impacts.

-	Study how to identify Ambient IoT Device or group of devices and how to format the identifier.

NOTE:	NAS based Congestion control are not in the scope of this study.

In the last SA2#164 meeting, some interim conclusions were made for KI#2 as followings.

8.2	Interim Conclusion on Key Issue #2

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

Editor's note:	How it is used is FFS.

In this paper, the following aspects are discussed and the conclusion proposals are made.

1) Subscription management of AIoT Device

Based on the solutions documented in TR 23.700-13, the subscription data stored in UDM is mainly used for authentication/validation of AIoT Device.

Proposal 1: It is proposed to conclude that subscription data stored in UDM is used for authentication/validation of AIoT Device.

2) Registration management of AIoT Device

For the DT and DO-DTT traffic studied in Rel-19, no Device initiated registration procedure can be performed. Some solutions of TR 23.700-13 proposed network triggered registration procedure mainly for authenticating AIoT Device or allocating temporary ID for AIoT Device. Considering the constraints of AIoT Device, no additional registration procedure is favoured besides Inventory and Command procedures.

Proposal 2: It is proposed to conclude that Registration management procedure is not supported for AIoT Device.

3) Connection management of AIoT Device

As the NAS signalling connection comprises AN connection and N2 connection, and RRC states are not supported, so the NAS signalling connection can’t be supported which means Connection Management states are not supported.

Proposal 3: It is proposed to conclude that Connection management procedure is not supported for AIoT Device.

4) Paging and reachability management of AIoT Device

Per the RAN2 agreements in the incoming LS (R2-2406150), the assumption is that AIoT service operation request (e.g. Inventory Request) from CN will trigger RAN Reader to perform paging with AIoT Devices. Most of the solutions about AIoT service procedures in the TR 23.700-13 are aligned with this assumption.

Proposal 4: It is proposed to conclude that AIoT service operation request (e.g. Inventory Request) from CN triggers RAN/UE Reader to perform paging with AIoT Devices.

5) Identifier and Identification Management

In the last SA2#164 meeting, it was agreed an ambient IoT Device Identifier can be assigned by an operator or by a third party, but how it is used is FFS. Based on the related solutions in the TR 23.700-13, it can be seen that in general there are two usages of AIoT Device ID: one is to construct the mask/filter for Inventory/Command Request, and the other is to be used for authentication of AIoT Device.

Figure 1: Illustration of Usage of AIoT Device ID

Proposal 5: It is proposed to conclude that AIoT Device ID is used for construction of mask/filter for Inventory/Command Request, and is used for authentication of AIoT Device.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.2	Interim Conclusion on Key Issue #2

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

AIoT Device ID is used for construction of mask/filter for Inventory/Command Request, and is used for authentication of AIoT Device.

8.2.2	Subscription management

Subscription data stored in UDM is used for authentication/validation of AIoT Device.

8.2.3	Registration management

Registration management procedure is not supported for AIoT Device.

8.2.4	Connection management

Connection management procedure is not supported for AIoT Device.

8.2.5	Paging and reachability management

AIoT service operation request (e.g. Inventory Request) from CN triggers RAN/UE Reader to perform paging with AIoT Devices.

# * * * * End of Changes * * * *