3GPP TSG-SA2 Meeting #165	S2-2409730

# Hyderabad, India, 14-18 October 2024		                                               (was S2-2408051)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1421 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#5: Support of Data off feature for data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of features to support "Handling of PS data off exemption for services over IMS DC" based on the conclusions for KI#5 in TR 23.700-77 and the LS reply S2-2407389 from SA1, wherein the following requirement is confirmed: "Depending on the business models and/or network deployment, flexibility is allowed for operators to further control the usage of individual applications on the IMS Data Channel when the PS Data Off Exempt of IMS Data Channel is on". |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution add a new chapter to TS 23.228 based on the conclusions for KI#5 in TR 23.700-77. - Add “Data Channel Serivces in IMS” in annex X - Add a new charpter to Annex AC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed "Handling of PS data off exemption for services over IMS DC" is not in the TS 23.228. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.1, X.2.3, X.3.2, AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

Annex X (normative):
IMS 3GPP PS Data Off Service Accessibility

X.1	General

3GPP PS Data Off is an optional feature. When activated by the user and when the network supports the feature, this feature allows control of the IMS services and more broadly SIP based services using the IMS framework that the user is allowed to access, for both originating and terminating sessions.

The list of 3GPP PS Data Off Exempted Services is configured by the HPLMN in the UE and in the network for enforcement. The list of SIP based service can include any one or any combination of the following services:

-	MMTel Voice;

-	SMS over IMS;

-	MMTel Video;

-	USSI;

-	Data Channel Serivces in IMS;

-	Particular IMS Services not defined by 3GPP, where each such IMS service is identified by an IMS communication service identifier.

* * * Next Changes * * *

X.2	UE Behaviour

X.2.3	UE Enforcement of 3GPP SIP-Based 3GPP PS Data Off Exempt Services

When the UE changes its 3GPP PS Data Off status from inactive to active, the UE shall ensure that only the 3GPP PS Data Off Exempted Services in the provisioned list are allowed to be transported, and the corresponding IP uplink packets shall be sent accordingly as follows:

-	The UE shall prevent sending of UE-originating SIP requests which are for services other than the 3GPP PS Data Off Exempted Services configured in the UE via device management or in the UICC.

-	The UE shall prevent sending of SDP offers and SDP answers with media streams for the media types other than those related to the 3GPP PS Data Off Exempted Services configured via device management or in the UICC.

-	A UE shall immediately stop sending any media packets and shall terminate all ongoing sessions for all SIP-based services that are not 3GPP PS Data Off Exempt. Specifically, when Data Channel Serivces in IMS is not in the list of 3GPP PS Data Off Exempted services:

-	If the ongoing session is an IMS DC session including audio/video media while audio and/or video media is exempted, UE shall terminat all the IMS Data Channels as specified in clause AC.7.6.

-	If the ongoing session is a standalone IMS DC session, the UE shall terminate the standalone IMS DC session

-	A UE provisioned with an updated list shall enforce the updated list immediately. To that effect, the UE shall immediately stop sending any media packets and terminate all ongoing SIP-based sessions handling the 3GPP PS Data Off Exempt Service(s) removed from the list.

X.3	Network Behaviour

X.3.2	Network Enforcement of SIP-Based 3GPP PS Data Off Exempted Services

Application Servers implementing the SIP-based services shall enforce the SIP based 3GPP PS Data Off Exempted services for all UEs.

Each Application Server shall be configured with up to two lists of 3GPP PS Data Off Exempt Services, one list for non-roaming users, and the other list for users roaming in the various VPLMNs with whom roaming agreements exist.

The AS shall become aware of the UE 3GPP Data Off status (active/inactive) at IMS (re-)Registration through third party registration. If the UE has changed its 3GPP PS Data Off status from inactive to active, the AS shall ensure that only SIP-based services which are part of the SIP-based 3GPP PS Data Off Exempt Services are permitted. Specifically, when Data Channel Serivces in IMS is not in the list of 3GPP PS Data Off Exempted services:

-	If the ongoing session is an IMS DC session including audio/video media while audio and/or video media is exempted, the IMS AS shall ensure that all the IMS Data Channels are terminated.

-	If the ongoing session is a standalone IMS DC session, the IMS AS shall ensure that the standalone IMS DC session is terminated.

In the context of IMS Data Channel, the AS may further notify the DCSF about the UE 3GPP PS Data Off status.

If the UE has changed its 3GPP PS Data Off status from active to inactive, the AS shall also let through the terminating requests to the UE for services that were not Data Off exempt.

NOTE 1:	The AS could be implemented in an existing AS e.g. SCC AS or distributed across other AS's.

NOTE 2:	The operator needs to ensure coordinated lists of 3GPP Data Off Exempt services provisioned in the UE and configured the network.

NOTE X:	The notification about the UE 3GPP PS Data Off status to the DCSF can be used by operators for further control, which is specific to implementation.

* * * Next Changes * * *

AC.X	PS Data Off

"Services over IMS Data Channel" is configured as a whole service in the list of 3GPP PS Data Off Exempted Services.

For more information refer to clause Annex X.

* * * End of Changes * * *