SA WG2 Meeting #165	S2-2409739

# Hyderabad, India, Oct 14th–Aug 18th, 2024        			(was S2-)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5642 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing LLDP TLV types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current LLDP functionality of the 5GS bridge is not compliant to IEEE Std 802.1AB-2016. IEEE Std 802.1AB-2016 specifies the support of TLV categories in clause 8.3 which are only partially supported. It is proposed to solve this issue by addition of missing LLDP TLV types to UMIC/PMIC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of missing LLDP TLV types to UMIC/PMIC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dependencies to other Ethernet standards which use LLDP according to IEEE Std 802.1AB-2016 cannot be resolved, e.g. - Management Address TLV, System Capabilities TLV (IEC/IEEE 60802) - Additional Ethernet Capabilities TLV (IEEE Std 802.3-2018). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1, K.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.28.1	5GS bridge management for TSN

5GS acts as a Layer 2 Ethernet Bridge. When integrated with IEEE TSN network, 5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the port on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the 5GS TSN bridge is per UPF for each network instance or DNN/S-NSSAI. The bridge ID of the 5GS TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side and a PDU Session during reporting of 5GS TSN bridge information. The TSN AF also stores the information about ports on the UPF/NW-TT side. The UPF/NW-TT forwards traffic to the appropriate egress port based on the traffic forwarding information. From the TSN AF point of view, a 5GS TSN bridge has a single NW-TT entity within UPF and the NW-TT may have multiple ports that are used for traffic forwarding.

NOTE 1:	How to realize single NW-TT entity within UPF is up to implementation.

NOTE 2:	Ethernet PDU Session type in this release of the specification may be subject to the constraint that it supports a single N6 interface in a UPF associated with the N6 Network Instance.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single 5GS bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5GS Bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 3:	It is assumed that all PDU Sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based 5GS bridge

NOTE 4:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple 5GS TSN bridges.

In order to support IEEE 802.1Q features related to TSN, including TSN scheduled traffic (clause 8.6.8.4 in IEEE Std 802.1Q [98]) over 5GS Bridge, the 5GS supports the following functions:

-	Configure the bridge information in 5GS.

-	Report the bridge information of 5GS Bridge to TSN network after PDU Session establishment.

-	Receiving the configuration from TSN network as defined in clause 5.28.2.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS Flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge ID

Bridge ID is to distinguish between bridge instances within 5GS. The Bridge ID can be derived from the unique bridge MAC address as described in IEEE Std 802.1Q [98], or set by implementation specific means ensuring that unique values are used within 5GS;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in IEEE Std 802.1Q [98]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	VLAN Configuration Information.

NOTE 5:	This Release of the specification does not support the modification of VLAN Configuration Information at the TSN AF.

-	Topology of 5GS Bridge as defined in IEEE Std 802.1AB [97]:

-	LLDP Configuration Information.

-	Chassis ID subtype and Chassis ID of the 5GS Bridge.

-	Optional TLV types.

-	LLDP Discovery Information for each discovered neighbor of each NW-TT port and DS-TT port.

-	Traffic classes and their priorities per port as defined in IEEE Std 802.1Q [98].

-	Stream Parameters as defined in clause 12.31.1 in IEEE Std 802.1Q [98], in order to support PSFP:

-	MaxStreamFilterInstances: The maximum number of Stream Filter instances supported by the bridge;

-	MaxStreamGateInstances: The maximum number of Stream Gate instances supported by the bridge;

-	MaxFlowMeterInstances: The maximum number of Flow Meter instances supported by the bridge (optional);

-	SupportedListMax: The maximum value supported by the bridge of the AdminControlListLength and OperControlListLength parameters.

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

DS-TT and NW-TT report txPropagationDelay to the TSN AF relative to the time base of the TSN GM clock (identified by the TSN time domain number received in PMIC). If the TSN AF has subscribed for notifications on txPropagationDelay and if the difference to the previously reported txPropagationDelay is larger than the txPropagationDelayDeltaThreshold received in PMIC, the corresponding DS-TT or NW-TT informs the TSN AF about the updated txPropagationDelay using PMIC signalling.

NOTE 6:	Configuration of TSN time domain number and txPropagationDelayDeltaThreshold via PMIC is optional for NW-TT. NW-TT can instead be pre-configured with the threshold and the single time domain that is used by the CNC for bridge configuration and reporting.

Bridge ID of the 5GS Bridge, port number(s) of the Ethernet port(s) in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC as described in clause 6.3.3.3.

This release of the specification requires that each DS-TT port is assigned with a globally unique MAC address.

NOTE 7:	The MAC address of the DS-TT port must not be used in user data traffic; it is used for identification of the PDU Session and the associated bridge port within the 3GPP system.

When there are multiple network instances within a UPF, each network instance is considered logically separate. The network instance for the N6 interface (clause 5.6.12) may be indicated by the SMF to the UPF for a given PDU Session during PDU Session establishment. UPF allocates resources based on the Network Instance and S-NSSAI and it is supported according to TS 29.244 [65]. DNN/S-NSSAI may be indicated by the SMF together with the network instance to the UPF for a given PDU Session during PDU Session establishment procedure.

The TSN AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the CNC.

# * * * * Next change * * * *

# K.1	Standardized port and user plane node management information

Table K.1-1 and Table K.1-2 list standardized port management information and user plane node management information, respectively.

Table K.1-1: Standardized port management information

| Port management information | Applicability (see NOTE 6) |  | Supported operations by TSN AF | Supported operations by TSCTSF | Reference |
| --- | --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT | (see NOTE 1) | (see NOTE 1) |  |
| General |  |  |  |  |  |
| Port management capabilities (see NOTE 2) | X | X | R | R |  |
| Bridge delay related information |  |  |  |  |  |
| txPropagationDelay | X | X | R | - | IEEE Std 802.1Q [98] clause 12.32.2.1 |
| txPropagationDelayDeltaThreshold (see NOTE 23) | X | X | RW |  |  |
| Traffic class related information |  |  |  |  |  |
| Traffic class table | X | X | RW | - | IEEE Std 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |  |
| queueMaxSDUTable |  |  |  |  | IEEE Std 802.1Q [98], clause 12.29.1 |
| > queueMaxSDU | X | X | RW |  | IEEE Std 802.1Q [98], clause 12.29.1 |
| > TransmissionOverrun (see NOTE 3) | X | X | R |  | IEEE Std 802.1Q [98], clause 12.29.1 |
| GateEnabled | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| AdminGateStates | X | X | RW |  | IEEE Std 802.1Q [98] Table 12-32 |
| AdminBaseTime | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| AdminControlList | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| AdminCycleTime (see NOTE 3) | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| AdminControlListLength (see NOTE 3) | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| AdminCycleTimeExtension | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| Tick granularity | X | X | R | - | IEEE Std 802.1Q [98] Table 12-32 |
| SupportedListMax | X | X | R | - | IEEE Std 802.1Q [98] Table 12-32 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |  |
| adminStatus | D | X | RW | - | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| OtlvSet (Note 28) |  | X | RW |  |  |
| DS-TT port neighbor discovery configuration |  |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D |  | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| OtlvSet (Note 28) | D |  | RW |  |  |
| Neighbor discovery information for each discovered neighbor of NW-TT (NOTE 26) |  |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| TTL |  | X | R | - | IEEE Std 802.1AB [97] clause 8.5.4 |
| OtlvSet (Note 28) |  | X | R |  |  |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| TTL | D |  | R | - | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| OtlvSet (Note 28) | D |  | R |  |  |
| Information for deterministic networking for each NW-TT port (NOTE 27) |  |  |  |  |  |
| Interface information |  |  |  |  |  |
| Interface type |  | X |  | R | IETF RFC 8343 [151] |
| Interface enabled status |  | X |  | R | IETF RFC 8343 [151] |
| phys-address |  | X |  | R | IETF RFC 8343 [151] |
| IPv4 information |  |  |  |  |  |
| IPv4 enabled status |  | X |  | R | IETF RFC 8344 [152] |
| IPv4 forwarding status |  | X |  | R | IETF RFC 8344 [152] |
| IPv4 MTU |  | X |  | R | IETF RFC 8344 [152] |
| Information for each IPv4 address |  |  |  |  |  |
| IPv4 address |  | X |  | R | IETF RFC 8344 [152] |
| prefix-length |  | X |  | R | IETF RFC 8344 [152] |
| netmask |  | X |  | R | IETF RFC 8344 [152] |
| origin |  | X |  | R | IETF RFC 8344 [152] |
| Information for each IPv4 neighbor |  |  |  |  |  |
| IPv4 address |  | X |  | R | IETF RFC 8344 [152] |
| link-layer-address |  | X |  | R | IETF RFC 8344 [152] |
| origin |  | X |  | R | IETF RFC 8344 [152] |
| IPv6 information |  |  |  |  |  |
| IPv6 enabled status |  | X |  | R | IETF RFC 8344 [152] |
| IPv6 forwarding status |  | X |  | R | IETF RFC 8344 [152] |
| IPv6 MTU |  | X |  | R | IETF RFC 8344 [152] |
| Information for each IPv6 address |  |  |  |  |  |
| IPv6 address |  | X |  | R | IETF RFC 8344 [152] |
| prefix-length |  | X |  | R | IETF RFC 8344 [152] |
| origin |  | X |  | R | IETF RFC 8344 [152] |
| status |  | X |  | R | IETF RFC 8344 [152] |
| Information for each IPv6 neighbor |  |  |  |  |  |
| IPv6 address |  | X |  | R | IETF RFC 8344 [152] |
| link-layer-address |  | X |  | R | IETF RFC 8344 [152] |
| origin |  | X |  | R | IETF RFC 8344 [152] |
| is-router |  | X |  | R | IETF RFC 8344 [152] |
| state |  | X |  | R | IETF RFC 8344 [152] |
| Stream Parameters (NOTE 11) |  |  |  |  |  |
| MaxStreamFilterInstances | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.1 |
| MaxStreamGateInstances | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.2 |
| MaxFlowMeterInstances | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.3 |
| SupportedListMax | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.4 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | - | IEEE Std 802.1Q [98] Table 12-35 |
| > StreamFilterInstanceIndex | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-35 |
| > Stream Identification type | X | X | RW | - | IEEE 802.1CB [83] clause 9.1.1.6 |
| > Stream Identification Controlling Parameters | X | X | RW | - | IEEE 802.1CB [83] clauses 9.1.2, 9.1.3, 9.1.4 (NOTE 12) |
| > PrioritySpec | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-35 |
| > StreamGateInstanceID | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-35 |
| Stream Gate Instance Table (NOTE 9) |  |  |  |  | IEEE Std 802.1Q [98] Table 12-33 |
| StreamGateInstanceIndex | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-36 |
| StreamGateAdminBaseTime | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-36 |
| StreamGateAdminControlList | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-36 |
| StreamGateAdminCycleTime | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-36 |
| StreamGateTickGranularity | X | X | R | - | IEEE Std 802.1Q [98] Table 12-36 |
| StreamGateAdminCycleTimeExtension | X | X | R | - | IEEE Std 802.1Q [98] Table 12-36 |
| Time Synchronization Information |  |  |  |  |  |
| TSN Time domain number (NOTE 24) | X | X | RW |  |  |
| Supported PTP instance types (NOTE 13) | X |  | R | R | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| Supported transport types (NOTE 14) | X |  | R | R |  |
| Supported delay mechanisms (NOTE 15) | X |  | R | R | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| PTP grandmaster capable (NOTE 16) | X |  | R | R |  |
| gPTP grandmaster capable (NOTE 17) | X |  | R | R |  |
| Supported PTP profiles (NOTE 18) | X |  | R | R |  |
| Number of supported PTP instances | X |  | R | R |  |
| PTP instance specification |  |  |  |  |  |
| PTP Instance ID (NOTE 25) | X | X | RW | RW |  |
| > PTP profile (NOTE 19) | X |  | RW | RW |  |
| > Transport type (NOTE 20) | X |  | RW | RW |  |
| > Grandmaster enabled (NOTE 21) | X |  | RW | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 22) |  |  |  |  |  |
| > defaultDS.clockIdentity | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.2.2 |
| > defaultDS.clockQuality.clockClass | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.2 |
| > defaultDS.clockQuality.clockAccuracy | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.4 |
| > defaultDS.priority1 | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.1 |
| > defaultDS.priority2 | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.2 |
| > defaultDS.domainNumber | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.3 |
| > defaultDS.sdoId | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.5 |
| > defaultDS.instanceEnable | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.2 |
| > defaultDS.instanceType | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| > portDS.portIdentity | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.2.1 |
| > portDS.portState | X | X | R | R | IEEE Std 1588 [126] clause 8.2.15.3.1 |
| > portDS.logMinDelayReqInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.3.2 |
| > portDS.logAnnounceInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.1 |
| > portDS.announceReceiptTimeout |  | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.2 |
| > portDS.logSyncInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.3 |
| > portDS.delayMechanism | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| > portDS.logMinPdelayReqInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.5 |
| > portDS.versionNumber | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.6 |
| > portDS.minorVersionNumber | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.7 |
| > portDS.delayAsymmetry | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.8 |
| > portDS.portEnable | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.5.1 |
| > timePropertiesDS.currentUtcOffset | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.4.2 |
| > timePropertiesDS.timeSource | X |  | RW | RW | IEEE Std 1588 [126] clause 8.2.4.9 |
| > externalPortConfigurationPortDS.desiredState |  |  | RW | RW | IEEE Std 1588 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 22) |  |  |  |  |  |
| > defaultDS.clockIdentity | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| > defaultDS.clockQuality.clockClass | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| > defaultDS.clockQuality.clockAccuracy | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| > defaultDS.priority1 | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| > defaultDS.priority2 | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| > defaultDS.timeSource | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| > defaultDS.domainNumber | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| > defaultDS.sdoId | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.instanceEnable | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| > portDS.portIdentity |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| > portDS.portState |  | X | R | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| > portDS.ptpPortEnabled | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| > portDS.delayMechanism | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| > portDS.isMeasuringDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| > portDS.asCapable | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| > portDS.meanLinkDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| > portDS.meanLinkDelayThresh | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| > portDS.delayAsymmetry | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| > portDS.neighborRateRatio | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| > portDS.initialLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| > portDS.currentLogAnnounceInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| > portDS.useMgtSettableLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| > portDS.mgtSettableLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| > portDS.announceReceiptTimeout |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| > portDS.initialLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| > portDS.currentLogSyncInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| > portDS.useMgtSettableLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| > portDS.mgtSettableLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| > portDS.syncReceiptTimeout |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| > portDS.syncReceiptTimeoutTimeInterval |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| > portDS.initialLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| > portDS.currentLogPdelayReqInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| > portDS.useMgtSettableLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| > portDS.mgtSettableLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| > portDS.initialLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| > portDS.currentLogGptpCapableMessageInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| > portDS.useMgtSettableLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| > portDS.mgtSettableLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| > portDS.initialComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| > portDS.currentComputeNeighborRateRatio | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| > portDS.useMgtSettableComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| > portDS.mgtSettableComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| > portDS.initialComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| > portDS.currentComputeMeanLinkDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| > portDS.useMgtSettableComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| > portDS.mgtSettableComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| > portDS.allowedLostResponses | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| > portDS.allowedFaults | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| > portDS.gPtpCapableReceiptTimeout | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| > portDS.versionNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| > portDS.nup | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| > portDS.ndown | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| > portDS.oneStepTxOper | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| > portDS.oneStepReceive | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| > portDS.oneStepTransmit | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| > portDS.initialOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| > portDS.currentOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| > portDS.useMgtSettableOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| > portDS.mgtSettableOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| > portDS.syncLocked | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| > portDS.pdelayTruncatedTimestampsArray | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| > portDS.minorVersionNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| > timePropertiesDS.currentUtcOffset | X |  | RW | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| > externalPortConfigurationPortDS.desiredState |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access; ― = not supported. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime, AdminControlListLength and TransmissionOverrun are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT using the User Plane Node Management Information Container (refer to Table K.1-2) and NW-TT performs neighbor discovery on behalf on DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. It is mandatory that the general neighbor discovery configuration is identical for all DS-TTs and the NW-TTs that belongs to the bridge. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. TSN AF indicates the neighbor discovery information for each discovered neighbor of DS-TT port to CNC. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, using the User Plane Node Management Information Container (refer to Table K.1-2), the NW-TT performing neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information and by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. When available, TSN AF uses the PSFP information for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in clause 8.6.5.2.1 of IEEE Std 802.1Q [98]. NOTE 11: TSN AF composes a Stream Parameter Table towards the CNC. It is up to TSN AF how it composes the Stream Parameter Table based on the numerical values as received from DS-TT and NW-TT port(s) and for the bridge for each individual parameter. NOTE 12: The set of Stream Identification Controlling Parameters depends on the Stream Identification type value as defined in IEEE Std 802.1CB [83] Table 9-1 and clauses 9.1.2, 9.1.3, 9.1.4. NOTE 13: Enumeration of supported PTP instance types. Allowed values as defined in clause 8.2.1.5.5 of IEEE Std 1588 [126]. NOTE 14: Enumeration of supported transport types. Allowed values: IPv4 (as defined in Annex C of IEEE Std 1588 [126]), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 15: Enumeration of supported PTP delay mechanisms. Allowed values as defined in clause 8.2.15.4.4 of IEEE Std 1588 [126]. NOTE 16: Indicates whether DS-TT supports acting as a PTP grandmaster. NOTE 17: Indicates whether DS-TT supports acting as a gPTP grandmaster. NOTE 18: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 19: PTP profile to apply, identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 20: Transport type to use. Allowed values: IPv4 (as defined in Annex C of IEEE Std 1588 [126]), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 21: Indicates whether to act as grandmaster or not, i.e. whether to send Announce, Sync and optionally Follow_Up messages. NOTE 22: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588 [126] data sets apply. NOTE 23: Indicates how much the txPropagationDelay needs to change so that DS-TT/NW-TT report a change in txPropagationDelay to TSN AF. This is optional for NW-TT. NOTE 24: Indicates the gPTP domain (identified by a domain number) that is assumed by the CNC as the reference clock for time information in the scheduled traffic (gate control) information, PSFP information and bridge delay related information. This is optional for NW-TT. NOTE 25: PTP Instance ID uniquely identifies a PTP instance within the user plane node. NOTE 26: TSN AF indicates the neighbor discovery information for each discovered neighbor of NW-TT port to CNC. NOTE 27: Applicable in case of interworking with IETF Deterministic Networking. NOTE 28: OtlvSet consists of concatenated list of the TLVs as defined in clause 8.5 and clause 8.6 of IEEE Std 802.1AB [97]. |  |  |  |  |  |

Table K.1-2: Standardized user plane node management information

| User plane node management information | Supported operations by TSN AF | Supported operations by TSCTSF | Reference |
| --- | --- | --- | --- |
|  | (see NOTE 1) | (see NOTE 1) |  |
| Information for 5GS Bridge/Router |  |  |  |
| User plane node Address | R | R |  |
| User plane node ID | R | R |  |
| NW-TT port numbers | R | R |  |
| Traffic forwarding information |  |  |  |
| Static Filtering Entry (NOTE 3) | RW | - | IEEE Std 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 2) |  |  |  |
| adminStatus | RW | - | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration for DS-TT ports (NOTE 4) |  |  |  |
| >DS-TT port neighbor discovery configuration for each DS-TT port |  |  |  |
| >> DS-TT port number | RW | - |  |
| >> lldpV2LocPortIdSubtype | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> OtlvSet (Note 18) | RW |  |  |
| Discovered neighbor information for DS-TT ports (NOTE 4) |  |  |  |
| >Discovered neighbor information for each DS-TT port (NOTE 4) |  |  |  |
| >> DS-TT port number | R | - |  |
| >> lldpV2RemChassisIdSubtype | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> TTL | R | - | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| >> OtlvSet (Note 18) | R |  |  |
| Stream Parameters (NOTE 5) |  |  |  |
| MaxStreamFilterInstances | R | - | IEEE Std 802.1Q [98] Table 12-34 |
| MaxStreamGateInstances | R | - | IEEE Std 802.1Q [98] Table 12-34 |
| MaxFlowMeterInstances | R | - | IEEE Std 802.1Q [98] Table 12-34 |
| SupportedListMax | R | - | IEEE Std 802.1Q [98] Table 12-34 |
| Time synchronization information |  |  |  |
| Supported PTP instance types (NOTE 6) | R | R |  |
| Supported transport types (NOTE 7) | R | R |  |
| Supported delay mechanisms (NOTE 8) | R | R |  |
| PTP grandmaster capable (NOTE 9) | R | R |  |
| gPTP grandmaster capable (NOTE 10) | R | R |  |
| Supported PTP profiles (NOTE 11) | R | R |  |
| Number of supported PTP instances | R | R |  |
| Time synchronization information for PTP instances (NOTE 16) |  |  |  |
| > PTP instance specification |  |  |  |
| >> PTP Instance ID (NOTE 17) | RW | RW |  |
| >> PTP profile (NOTE 12) | RW | RW |  |
| >> Transport type (NOTE 13) | RW | RW |  |
| >> Grandmaster candidate enabled | RW | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 15) |  |  |  |
| >> defaultDS.clockIdentity | RW | RW | IEEE Std 1588 [126] clause 8.2.1.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.4 |
| >> defaultDS.priority1 | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.1 |
| >> defaultDS.priority2 | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.2 |
| >> defaultDS.domainNumber | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.3 |
| >> defaultDS.sdoId | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.5 |
| >> defaultDS.instanceEnable | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.2 |
| >> defaultDS.externalPortConfigurationEnabled | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.3 |
| >> defaultDS.instanceType | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| >> timePropertiesDS.currentUtcOffset | RW | RW | IEEE Std 1588 [126] clause 8.2.4.2 |
| >> timePropertiesDS.timeSource | RW | RW | IEEE Std 1588 [126] clause 8.2.4.9 |
| IEEE Std 802.1AS [104] data sets (NOTE 15) |  |  |  |
| >> defaultDS.clockIdentity | RW | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| >> defaultDS.priority1 | RW | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| >> defaultDS.priority2 | RW | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| >> defaultDS.timeSource | RW | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| >> defaultDS.domainNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| >> defaultDS.sdoId | RW | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| >> defaultDS.externalPortConfigurationEnabled | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.instanceEnable | RW | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| >> timePropertiesDS.currentUtcOffset | RW | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| Time synchronization information for DS-TT ports |  |  |  |
| > Time synchronization information for each DS-TT port |  |  |  |
| > DS-TT port number | RW | RW |  |
| >> Time synchronization information for each PTP Instance |  |  |  |
| >> PTP Instance ID (NOTE 17) | RW | RW |  |
| >> Grandmaster on behalf of DS-TT enabled (NOTE 14) | RW | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 15) |  |  |  |
| >> portDS.portIdentity | RW | RW | IEEE Std 1588 [126] clause 8.2.15.2.1 |
| >> portDS.portState | R | R | IEEE Std 1588 [126] clause 8.2.15.3.1 |
| >> portDS.logMinDelayReqInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.3.2 |
| >> portDS.logAnnounceInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.1 |
| >> portDS.announceReceiptTimeout | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.2 |
| >> portDS.logSyncInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.3 |
| >> portDS.delayMechanism | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| >> portDS.logMinPdelayReqInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.5 |
| >> portDS.versionNumber | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.6 |
| >> portDS.minorVersionNumber | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.7 |
| >> portDS.delayAsymmetry | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.8 |
| >> portDS.portEnable | RW | RW | IEEE Std 1588 [126] clause 8.2.15.5.1 |
| >> externalPortConfigurationPortDS.desiredState | RW | RW | IEEE Std 1588 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 15) |  |  |  |
| >> portDS.portIdentity | RW | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| >> portDS.portState | R | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| >> portDS.ptpPortEnabled | RW | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| >> portDS.delayMechanism | RW | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| >> portDS.isMeasuringDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| >> portDS.asCapable | R | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| >> portDS.meanLinkDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| >> portDS.meanLinkDelayThresh | RW | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| >> portDS.delayAsymmetry | RW | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| >> portDS.neighborRateRatio | R | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| >> portDS.initialLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| >> portDS.currentLogAnnounceInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| >> portDS.useMgtSettableLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| >> portDS.mgtSettableLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| >> portDS.announceReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| >> portDS.initialLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| >> portDS.currentLogSyncInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| >> portDS.useMgtSettableLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| >> portDS.mgtSettableLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| >> portDS.syncReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| >> portDS.syncReceiptTimeoutTimeInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| >> portDS.initialLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| >> portDS.currentLogPdelayReqInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| >> portDS.useMgtSettableLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| >> portDS.mgtSettableLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| >> portDS.initialLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| >> portDS.currentLogGptpCapableMessageInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| >> portDS.useMgtSettableLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| >> portDS.mgtSettableLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| >> portDS.initialComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| >> portDS.currentComputeNeighborRateRatio | R | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| >> portDS.useMgtSettableComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| >> portDS.mgtSettableComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| >> portDS.initialComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| >> portDS.currentComputeMeanLinkDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| >> portDS.useMgtSettableComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| >> portDS.mgtSettableComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| >> portDS.allowedLostResponses | RW | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| >> portDS.allowedFaults | RW | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| >> portDS.gPtpCapableReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| >> portDS.versionNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| >> portDS.nup | RW | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| >> portDS.ndown | RW | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| >> portDS.oneStepTxOper | R | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| >> portDS.oneStepReceive | R | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| >> portDS.oneStepTransmit | R | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| >> portDS.initialOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| >> portDS.currentOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| >> portDS.useMgtSettableOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| >> portDS.mgtSettableOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| >> portDS.syncLocked | R | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| >> portDS.pdelayTruncatedTimestampsArray | RW | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| >> portDS.minorVersionNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| >> externalPortConfigurationPortDS.desiredState | RW | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| Time synchronization status (TSS) information |  |  |  |
| > Synchronization state | - | R | Table 5.27.1.12-1 |
| > Clock quality | - | R | Table 5.27.1.12-1 |
| >> Traceable to UTC | - | R | Table 5.27.1.12-1 |
| >> Traceable to GNSS | - | R | Table 5.27.1.12-1 |
| >> Frequency stability | - | R | Table 5.27.1.12-1 |
| >> Clock accuracy | - | R | Table 5.27.1.12-1 |
| > Parent time source | - | R | Table 5.27.1.12-1 |
| NOTE 1: R = Read only access; RW = Read/Write access; ― = not supported. NOTE 2: General neighbor discovery information is included only when NW-TT performs neighbor discovery on behalf of DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. NOTE 3: If the Static Filtering Entry information is present, UPF/NW-TT can use Static Filtering Entry information for forwarding TSC traffic, as specified in clause 5.8.2.5.3. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. TSN AF indicates the discovered neighbor information for each DS-TT port to CNC. NOTE 5: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge have indicated support of PSFP. The support of PSFP at the NW-TT ports is expressed by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. NOTE 6: Enumeration of supported PTP instance types. Allowed values as defined in clause 8.2.1.5.5 of IEEE Std 1588 [126]. NOTE 7: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588 [126] Annex C), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 8: Enumeration of supported PTP delay mechanisms. Allowed values as defined in clause 8.2.15.4.4 of IEEE Std 1588 [126]. NOTE 9: Indicates whether NW-TT supports acting as a PTP grandmaster. NOTE 10: Indicates whether NW-TT supports acting as a gPTP grandmaster. NOTE 11: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 12: PTP profile to apply, identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 13: Transport type to use. Allowed values: IPv4 (as defined in Annex C of IEEE Std 1588 [126]), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 14: Indicates whether to act as grandmaster on behalf of a DS-TT port or not if 5GS is determined to be the grandmaster clock, i.e. whether to send Announce, Sync and optionally Follow_Up messages on behalf of DS-TT. NOTE 15: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise, the IEEE Std 1588 [126] data sets apply. NOTE 16: Specifies the default data set for each PTP instance identified by PTP instance ID within the user plane node. NOTE 17: PTP Instance ID uniquely identifies a PTP instance within the user plane node. NOTE 18: OtlvSet consists of concatenated list of the TLVs as defined in clause 8.5 and clause 8.6 of IEEE Std 802.1AB [97]. |  |  |  |

# * * * End of changes * * * *