3GPP TSG-WG SA2 Meeting #165	S2-2409753

# Hyderabad, IN, 14th Oct – 18th Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5650 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAT functionality in the UPF of BH PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During during PDU Session Establishment of Normal UE: - The N3 UPF receives the AN Tunnel information of MWAB-gNB (including IP address of MWAB-UE ). - The AN Tunnel information is included in the N2 SM container, which is transparent to the BH PLMN . Considering different possible deployment scenarios as described in the Annex S of TS 23.501, it is possible that the NAT on N6 is supported when the N3 message which carries the data traffic of normal UE is transferred via BH PDU session. If MWAB-UE IP is a private IP address and NAT functionality in the UPF of BH PDU Session is used, when N3 message which carries the data traffic of UE is routed by BH UPF via N6, the source IP address of N3 GTP-U payload will be NATed, which is different from the AN tunnel information of MWAB-gNB provided to UE SMF during BH PDU Session establishment/modification. And that may cause the UE UPF fail to identify the traffic from MWAB-gNB. It is needed to clarify the IP routing between BH PLMN and MWAB broadcasted PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the IP routing aspect between BH PLMN and MWAB broadcasted PLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If NAT functionality in the UPF of BH PDU Session is used, the reachability of IP routing between BH PLMN and broadcasted PLMN may not be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.49	Support for Mobile gNB with Wireless Access Backhauling (MWAB)

5.49.1	General

5.49.1.1	Principles and functional entities

Mobile gNB with Wireless Access Backhauling (MWAB) provides an NR access link to UEs in proximity and connects to the 5GC serving the UE through an IP connectivity provided by a Backhaul PDU session(s), as illustrated in figure 5.49.1.1-1. The MWAB consists of a gNB component (MWAB-gNB) and a UE component (MWAB-UE). The MWAB-gNB is based on the gNB functionality specified in TS 38.300 [27] and TS 38.401 [42]. The MWAB may be mounted on a moving vehicle and may serve UEs inside or outside the vehicle.

The MWAB-UE establishes the IP connectivity for the backhaul links for the MWAB-gNB, via NR Uu, using the existing registration procedure and PDU session establishment procedure. The backhaul links are between the MWAB-gNB and entities of the network (e.g. AMF, UPF, other gNBs and OAM server) that a MWAB-gNB cell serves. The IP connectivity provided by the MWAB-UE may be either via the same PLMN/SNPN that the MWAB-gNB serves or a different PLMN/SNPN, depends on the MWAB-UE PLMN selection mechanism as specified in TS 23.122 [17]. Different possible deployment scenarios are presented in Annex S.

NOTE x:	The reachability of the IP routing needs to be guaranteed between the BH PLMN and broadcasted PLMN, and the entities of the network (e.g., AMF, UPF, other gNBs and OAM server) belonging to the MWAB broadcasted PLMN should be able to identify the MWAB when receiving this IP address from backhaul links. To this end, the BH SMF/UPF may consider the dedicated S-NSSAI/DNN when allocating the IP address for the MWAB-UE.

MWAB operation supports both PLMN and SNPN cases. When the MWAB-gNB is serving a PLMN, the UEs served by MWAB may be non-roaming or roaming in the MWAB Broadcasted PLMN. In case the MWAB-gNB is serving a SNPN, the subscribed SNPN of the UEs may be different from the MWAB Announced SNPN. The UEs served by the MWAB are not aware of the network serving the MWAB-UE.

Figure 5.49.1.1-1: MWAB architecture for 5GS

NOTE 1:	In this Release, the CU/DU split of the MWAB-gNB is not supported.

NOTE 2:	The interface between MWAB-UE and MWAB-gNB is implementation based and not in scope of this specification.

MWAB-UE has a single NR Uu hop to the NG-RAN, using either TN or NTN access technology. NR access is used for the radio link between a MWAB-gNB and the served UEs. The NR Uu access link between the MWAB-gNB and the served UE(s) does not use NTN access technology. 5G MOCN can be supported by the MWAB-gNB.

NOTE 3:	The use of multiple PDU Sessions for N2, N3, Xn, OAM access, follows the existing URSP logic as defined in TS 23.503 [45].

The details of the MWAB configuration/provisioning process are described in clause 5.49.2.

In order to operate as MWAB, the MWAB-gNB component and MWAB-UE component are authorized and controlled separately. The detailed procedures for the authorization of MWAB are described in clause 5.49.3.

Service continuity for the UE(s) served by the MWAB-gNB is supported, when the MWAB moves and the UE(s) move or do not move together with the MWAB. The detailed procedures for the support of mobility are described in clause 5.49.5.

The MWAB shall be able to serve UE(s) without any MWAB-specific enhancements. For some operations, the MWAB may be configured to provide access to only certain UEs. Existing access control mechanisms, e.g. CAG control, can be used to manage the UE(s)'s access to the MWAB-gNB. The details of the access control of the UEs served by MWAB are described in clause 5.49.6.

The LCS framework as defined in TS 23.273 [87] is used for providing the location service to the UE(s) served by MWAB. Details on supporting the LCS over MWAB are described in clause 5.49.7.

Regulatory services (e.g. emergency services, priority services) can be supported by the MWAB, and the details are provided in clause 5.49.8.

End of CHANGEs