TSG SA WG2 Meeting #165	S2-2409759

Hyderabad, India, October 14-18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5653 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of QoS for UEs served by MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed in TR 23.700-60’s conclusion that, - If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is the used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures. In this contribution, the alignment for the QoS control of UE QoS flow and BH QoS flow is considered, including the mapping of 5QI and DSCP and PDB coordination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following enhancement are introduced, - MWAB-gNB is configured with the mapping of the 5QI to the DSCP field and provides the mapping within the packet filter and required QoS to MWAB-UE. MWAB-UE triggers PDU session modfication procedure by providing the packet filter (e.g., with DSCP) and the required QoS (e.g., 5QI) to MWAB-SMF, which enables the alignment for the QoS control of UE QoS flow and BH QoS flow. - MWAB-UE binds UE QoS flow to BH QoS flow by taking QoS requirements (e.g., PDBs) of both QoS flows into consideration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The alignment for the QoS control of UE QoS flow and BH QoS flow is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.49	Support for Mobile gNB with Wireless Access Backhauling (MWAB)

### 5.49.1	General

#### 5.49.1.1	Principles and functional entities

Mobile gNB with Wireless Access Backhauling (MWAB) provides an NR access link to UEs in proximity and connects to the 5GC serving the UE through an IP connectivity provided by a Backhaul PDU session(s), as illustrated in figure 5.49.1.1-1. The MWAB consists of a gNB component (MWAB-gNB) and a UE component (MWAB-UE). The MWAB-gNB is based on the gNB functionality specified in TS 38.300 [27] and TS 38.401 [42]. The MWAB may be mounted on a moving vehicle and may serve UEs inside or outside the vehicle.

The MWAB-UE establishes the IP connectivity for the backhaul links for the MWAB-gNB, via NR Uu, using the existing registration procedure and PDU session establishment procedure. The backhaul links are between the MWAB-gNB and entities of the network (e.g. AMF, UPF, other gNBs and OAM server) that a MWAB-gNB cell serves. The IP connectivity provided by the MWAB-UE may be either via the same PLMN/SNPN that the MWAB-gNB serves or a different PLMN/SNPN, depends on the MWAB-UE PLMN selection mechanism as specified in TS 23.122 [17]. Different possible deployment scenarios are presented in Annex S.

MWAB operation supports both PLMN and SNPN cases. When the MWAB-gNB is serving a PLMN, the UEs served by MWAB may be non-roaming or roaming in the MWAB Broadcasted PLMN. In case the MWAB-gNB is serving a SNPN, the subscribed SNPN of the UEs may be different from the MWAB Announced SNPN. The UEs served by the MWAB are not aware of the network serving the MWAB-UE.

Figure 5.49.1.1-1: MWAB architecture for 5GS

NOTE 1:	In this Release, the CU/DU split of the MWAB-gNB is not supported.

NOTE 2:	The interface between MWAB-UE and MWAB-gNB is implementation based and not in scope of this specification.

MWAB-UE has a single NR Uu hop to the NG-RAN, using either TN or NTN access technology. NR access is used for the radio link between a MWAB-gNB and the served UEs. The NR Uu access link between the MWAB-gNB and the served UE(s) does not use NTN access technology. 5G MOCN can be supported by the MWAB-gNB.

NOTE 3:	The use of multiple PDU Sessions for N2, N3, Xn, OAM access, follows the existing URSP logic as defined in TS 23.503 [45].

The details of the MWAB configuration/provisioning process are described in clause 5.49.2.

In order to operate as MWAB, the MWAB-gNB component and MWAB-UE component are authorized and controlled separately. The detailed procedures for the authorization of MWAB are described in clause 5.49.3.

Service continuity for the UE(s) served by the MWAB-gNB is supported, when the MWAB moves and the UE(s) move or do not move together with the MWAB. The detailed procedures for the support of mobility are described in clause 5.49.5.

The MWAB shall be able to serve UE(s) without any MWAB-specific enhancements. For some operations, the MWAB may be configured to provide access to only certain UEs. Existing access control mechanisms, e.g. CAG control, can be used to manage the UE(s)'s access to the MWAB-gNB. The details of the access control of the UEs served by MWAB are described in clause 5.49.6.

The LCS framework as defined in TS 23.273 [87] is used for providing the location service to the UE(s) served by MWAB. Details on supporting the LCS over MWAB are described in clause 5.49.7.

Regulatory services (e.g. emergency services, priority services) can be supported by the MWAB, and the details are provided in clause 5.49.8.

#### 5.49.1.x	Support of QoS for UEs served by MWAB

Based on configuration and traffic mapping specified in TS 38.414 [195], the MWAB-gNB sets proper marking information (e.g. DSCP) for the N3 traffic going over the Backhaul PDU Session of the MWAB-UE. The MWAB-gNB provides packet filter (e.g., with DSCP) and Requested QoS (e.g., 5QI) to the MWAB-UE. The MWAB-UE triggers PDU session modification procedure by providing packet filter (e.g., with DSCP) and Requested QoS (e.g., 5QI) towards the SMF of the Backhaul PDU session(s), which enables the SMF takes the traffic mapping specified in TS 38.414 [195] into account when generating the QoS Rules and PDRs for the BH PDU session(s).

MWAB-UE binds the UE QoS flow to BH QoS flow, by taking the QoS parameters of the UE QoS flow and BH QoS flow into account, e.g., to make sure that PDB of the BH QoS flow is smaller than the CN PDB for the UE.

Upon receiving dedicated (DNN, S-NSSAI), the SMF of BH PDU session(s) set the PDU session AMBR and MBDV with pre-configured values, which are defined and used for BH PDU sessions.

# * * * End of changes * * * *