3GPP TSG-WG SA2 Meeting #165 	S2-2409765

Hyderabad, India, October 14th – 18th, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: I-SMF selection for local offloading

Document for:	Approval

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract: there are two proposals on I-SMF insertion in S2#164 meeting, this document compares the two approaches and proposes a way forward for I-SMF selection.

1. Introduction

Two types of solution were proposed for I-SMF selection in S2#164 meeting:

- AMF triggered I-SMF selection (S2-2407903, S2-2407720). AMF triggers I-SMF selection based on subscription data and UE location.

- SMF triggered I-SMF selection (S2-2408562). SMF triggers AMF to select I-SMF based on subscription data (and optional the target DNAI received in PCC rule).

This paper tries to compare the impact of these two solutions and give a suggestion on which solution is to be selected.

2. Discussion

Option 1: AMF based solution:

SMF

PCF

I-SMF

AMF

AF/NEF

UDM

1. Registration procedure in clause 4.2.2.2 of TS 23.502, AMF receives local offloading allowed indication in the SMF selection subscription data, and local offloading service area (a list of TA(s))

11. Nudm_SDM_Get (local offloading allowed indication per DNN, S-NSSAI)

7.Nsmf_PDUSession_CreateSMContext Request

9. Select and configure ULCL/BP, L-PSA, EASDF based on UE location

8.Nsmf_PDUSession_CreateSMContext Response

RAN

UE

12. remaining steps for PDU session establishment with I-SMF

3. PDU Session Establishment Request (Requested DNN, S-NSSAI)

4. Determine the DNN/S-NSSAI is allowed for local offloading

5. Determine the current UE location is in the local offloading service area

2. local configured with local offloading service area

6. Select both an I-SMF and a SMF supporting local offloading

10.Nsmf_PDUSession_Create Request

SMF

PCF

I-SMF

AMF

AF/NEF

UDM

1. Registration procedure in clause 4.2.2.2 of TS 23.502, AMF receives local offloading allowed indication in the SMF selection subscription data, and local offloading service area (a list of TA(s))

11. Nudm_SDM_Get (local offloading allowed indication per DNN, S-NSSAI)

7.Nsmf_PDUSession_CreateSMContext Request

9. Select and configure ULCL/BP, L-PSA, EASDF based on UE location

8.Nsmf_PDUSession_CreateSMContext Response

RAN

UE

12. remaining steps for PDU session establishment with I-SMF

3. PDU Session Establishment Request (Requested DNN, S-NSSAI)

4. Determine the DNN/S-NSSAI is allowed for local offloading

5. Determine the current UE location is in the local offloading service area

2. local configured with local offloading service area

6. Select both an I-SMF and a SMF supporting local offloading

10.Nsmf_PDUSession_Create Request

Figure 1: AMF triggered I-SMF insertion during PDU session establishment

As shown in Figure 1, when the AMF receives Local Offloading Allowed Indicator (per DNN/S-NSSAI granularity) from subscription data and the UE location is in the local offloading service area, the AMF triggers local I-SMF insertion.

NOTE: the local offloading service area is the service area supported by the Local I-SMF.

This solution has been discussed at the TR phase. The existing HR-SBO solution can be used as a reference to design the I-SMF based Local Offloading Management solution.

[Observation 1]:   AMF based approaches is similar as the HR-SBO solution. Based on the subscription data and UE location AMF determine whether the local I-SMF is to be inserted.

Option 2: SMF based solution:

SMF

PCF

I-SMF

AMF

5.PCC rule (target DNAI)

AF/NEF

4. AF influence info provisioning as clause 4.3.6.2 of TS 23.502 (target DNAI)

UDM

1. step 1-3 of PDU session establishment procedure in clause 4.3.2.2.1 of TS 23.502

2. Nudm_SDM_Get (local offloading allowed indication per DNAI)

10. Nsmf_PDUSession_SMContextStatusNotify (target DNAI)

12.Nsmf_PDUSession_CreateSMContext Request (target DNAI)

14. select and configure ULCL/BP, L-PSA based on target DNAI

3. step 5-7a of PDU session establishment procedure in clause 4.3.2.2.1 of TS 23.502

UDR

7. If current UE location is associated with the target DNAI (i.e. UE location in the TA(s) corresponding to the target DNAI)

step 8-9 are skipped

11. I-SMF selection based on target DNAI (assume this I-SMF can support local offloading)

6. Deteremine the target DNAI   which has local offloading allowed indication

13.Nsmf_PDUSession_ContextRequest

I-SMF insertion as clause 4.23 of TS 23.502

15.Nsmf_PDUSession_Create Request

16. PSA UPF reconfiguration via N4 session modification

17.Nsmf_PDUSession_Create Response

18.Nsmf_PDUSession_CreateSMContext Response

19.Namf_Communication_N1N2Message Transfer

RAN

UE

20. N2 PDU Session Request

21. RRC reconfiguration

22. remaining steps for PDU session establishment

SMF

PCF

I-SMF

AMF

5.PCC rule (target DNAI)

AF/NEF

4. AF influence info provisioning as clause 4.3.6.2 of TS 23.502 (target DNAI)

UDM

1. step 1-3 of PDU session establishment procedure in clause 4.3.2.2.1 of TS 23.502

2. Nudm_SDM_Get (local offloading allowed indication per DNAI)

10. Nsmf_PDUSession_SMContextStatusNotify (target DNAI)

12.Nsmf_PDUSession_CreateSMContext Request (target DNAI)

14. select and configure ULCL/BP, L-PSA based on target DNAI

3. step 5-7a of PDU session establishment procedure in clause 4.3.2.2.1 of TS 23.502

UDR

7. If current UE location is associated with the target DNAI (i.e. UE location in the TA(s) corresponding to the target DNAI)

step 8-9 are skipped

11. I-SMF selection based on target DNAI (assume this I-SMF can support local offloading)

6. Deteremine the target DNAI   which has local offloading allowed indication

13.Nsmf_PDUSession_ContextRequest

I-SMF insertion as clause 4.23 of TS 23.502

15.Nsmf_PDUSession_Create Request

16. PSA UPF reconfiguration via N4 session modification

17.Nsmf_PDUSession_Create Response

18.Nsmf_PDUSession_CreateSMContext Response

19.Namf_Communication_N1N2Message Transfer

RAN

UE

20. N2 PDU Session Request

21. RRC reconfiguration

22. remaining steps for PDU session establishment

Figure 2: SMF triggered I-SMF insertion during PDU session establishment

This is the new proposal and not been discussed in the TR phase before.

As shown in Figure 2, AMF selects the SMF first. When the SMF receives the Local Offloading Allowed Indicator (per DNAI granularity) from subscription data, the SMF notifies the target DNAI to the AMF. Based on that information, AMF triggers the local I-SMF insertion. The SMF notifies the AMF of the target DNAI when it is included in the subscription data and also optional AF traffic influence include the target DNAI.

For this approach, it is assumed that when the SMF notifies the Target DNAI to AMF, the UE is in the TA corresponding to the target DNAI. Otherwise, this notification to the AMF is useless.

[Observation 2]:  SMF based approach is based on subscription data and needs SMF to be aware the UE location to notify AMF trigger I-SMF selection.

For the SMF based solution, on how to notify the AMF aspect some clarification is needed. If there are more than one DNAI recieved by the SMF, it is unclear whether the SMF notification to AMF is just one target DNAI or it can be more than one Target DNAI. If it is more than one target DNAI, it is unclear how the SMF is aware how DNAI can be grouped together, e.g. some DNAI are not suitable to be grouped together due to applied in different place.

[Observation 3]: For the SMF based solution, how the SMF notify the AMF of the target DNAI need some more clarification.

The motivation of the KI#1 is to reduce the impact of the central SMF. The impact on the central SMF on two solution can be listed as following:

- For AMF based solution, as shown in Figure 1, the information corresponding to I-SMF service area can be learnt/stored by AMF same as before, and the selection/insertion of I-SMF are controlled by AMF per subscription data. The central SMF only needs authorize the request for local offloading per subscription data. The impact to central SMF is minimum.

- For SMF based solution as shown in Figure 2, the SMF needs to determine when to notify the AMF to do the I-SMF insertion.  To support this the central SMF need be aware two additional information: 1) the mapping informatoin between the location (e.g. TA/cell) and DNAI; 2) the UE current location.

Hence the central SMF need be always keep updated with the information of mapping beween location and DNAI, e.g. a new DNAI is added to the network. Also the central SMF need subscribe the UE location information, e.g. AOI information. Due to the different DNAI may be associated with different AOI, due to UE mobility central SMF also need determine whether the AOI need be updated and if yes update the UE location subscripton information.

Above two information are also required by the AMF based approaches. For AMF based approaches, the mapping between location and DNAI is maintained by the I-SMF and reflected as the service area of the I-SMF. And the UE current location is aware by the AMF. Hence there are no impact on the central SMF on this aspect.

[Observation 4]: Comparing above two solutions, it is clear that the AMF based approaches has less impact to the central SMF.

3. Proposal

Based on the above-mentioned analysis, it is clear that the SMF based apporches need some more clarification and big impact on the central SMF, which is conflict with the intention KI#1. Hence we suggest to go with AMF based approaches.

[Conclusion]: For the I-SMF insertion, it is suggested to go with the HR-SBO similar mechanism, i.e. AMF based mechanism.