3GPP TSG-WG SA2 Meeting #165	S2-2409766

# Hyderabad, India, 14th Oct – 18th Oct, 2024        	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0257 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: EDI retrieval of the local SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion as in the cluase 8.1 of TR 23.700-49: I-SMF is introduced to reduce impact on central 5GC NFs and is support of EDI management. For I-SMF which deployed locally, it may only care about part of the EDI (e.g., corresponding to its supported DNAI(s)). So, it brings unnecessary impact on the local SMF if EDI that the local SMF does not care about is also notified to it. Hence, it is proposed to add DNAI(s) in the Data SubKey for EDI subscription/notification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add calrification that if the SMF only cares about EDI related with some DNAI(s), it can also indiacte DNAI(s) when subscribing EDI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The impact on central NFs are transferred to local NFs, and the key issue’s requirement cannot be satisified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5017 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.4.3	EAS Deployment Information Management in the SMF or I-SMF

The SMF may receive the EAS Deployment Information from NEF via Subscribe /Notify procedure defined in this clause. NEF may have stored the information in UDR.

When I-SMF based Local Offloading Management applies, the I-SMF may receive the EAS Deployment Information from NEF as described in this clause.

NOTE:	When I-SMF based Local Offloading Management applies, both the SMF and I-SMF may receive the EDI from NEF.

Figure 6.2.3.4.3-1: EAS Deployment Information management in the SMF procedure

1-2.	As pre-requisite condition, the SMF subscribes to EAS Deployment Information Change Notification from the NEF by sending Nnef_EASDeployment_Subscribe message. The SMF may indicate that the current status of EAS Deployment Information shall be notified immediately (if available). The SMF may indicate for which (list of) DNN and/or S-NSSAI and/or application identifier and/or Internal Group Identifier (if available) and/or DNAI(s) it subscribes.

If the (I-)SMF only cares about EAS Deployment Information related to some specific DNAI(s) (e.g., I-SMF based Local Traffic Offloading management case), the (I-)SMF also indicates DNAI(s) (e.g., DNAI(s) supported by the (I-)SMF) to the NEF to retrieve EAS Deployment Information corresponding to the indicated DNAI(s).

3-4.	The NEF invokes Nnef_EASDeployment_Notify (EAS Deployment Information) to the SMF(s) to which the EAS Deployment Information shall be provided. If there is EAS Deployment Information available and immediate report is required, the NEF notifies the SMF(s) or I-SMF for local offloading management with such information.

For EAS Deployment Information management in the I-SMF based Local Traffic Offloading management case, the SMF in clause 6.2.3.4.3 is replaced by I-SMF.

# * * * * End of changes * * * *