3GPP TSG-WG SA2 Meeting #165	S2-2409787

# Hyderabad, India, Oct 14 – 18, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1212 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the High level feature description for VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Vertical Federated Learning, when one NWDAF acting as a VFL server, there may be one or multiple NWDAF(s)/AF(s) acting as VFL Client(s), so the VFL can be available among NWDAF(s) and AF(s) within a single PLMN. However, this case is missing in current clause 5.4, which only indludes “Vertical Federated Learning is available among NWDAFs within a single PLMN or between an AF and NWDAF(s) in a single PLMN.” Regarding the interoperability among NWDAF(s) and/or AF(s), considering that the vendor ID of AF is hard to define, so it is proposed to define a VFL Group ID to identify the VFL Group, which could be pre-configured by the OAM. Entities within the same VFL Capability Group can collaborate in VFL tasks as introduced in TS23.288 CR 1213. When the VFL server selects VFL client(s), the VFL client(s) may not have the initial ML model, so the VFL server needs to provide the initial ML Model(s) to the VFL client(s) based on request from the VFL client(s). The VFL server also needs to train ML Model with the available local data set, and the VFL server will aggregate intermediate results from VFL client(s) and VFL server, which are not captured in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the VFL can be available among NWDAF(s) and AF(s) within a single PLMN. Introduce the VFL Capability Group ID for the interoperability among NWDAF(s) and/or AF(s). Clarify that the VFL server may provide the initial ML Model(s) to the VFL client(s) based on request from the VFL client(s). Clarify that the VFL server also needs to train ML Model with the available local data set, and aggregate intermediate results from VFL client(s) and VFL server. In addition, we clarify that the FL server’s data set has label, and FL client’s data set has no label. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how to perform interoperability among NWDAF(s) and/or AF(s), and the VFL client may not have the initial ML model and cannot perform local model training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning techinique without exchanging/sharing local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AF.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAF(s) and AF(s) within a single PLMN or between an AF and NWDAF(s) in a single PLMN. If the NWDAF(s) and/or AF(s) collaborate in the same VFL task, the NWDAF(s) and/or AF(s) shall share the same VFL Group ID as described in clause 5.2.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure. The VFL server may provide the initial ML Model(s) to the VFL client(s) based on request from the VFL client(s).

Editor's note:	Support for AF acting as VFL server selecting multiple NWDAFs is FFS.

-	requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL model correlation ID, and it requests to report intermediate results.

-	locally trains ML Model with the available local data set (i.e. data set with label), aggregates intermediate results from VFL client(s) and VFL server and computes intermediate training results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training results towards VFL clients involved in the joint VFL training process.

-	It initiates the VFL inference process using VFL model correlation ID.

-	It aggregates local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

VFL client:

-	locally trains ML Model with the available local data set (i.e. data set without label), which includes the data that may not be allowed to be shared with other VFL clients due to e.g. data privacy, data security, data access rights.

-	computes the intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	performs inference based on the local model and local data and provides inference results to VFL server.

Editor's note:	Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS.

Editor's note:	Accuracy monitoring in VFL is FFS.

Editor's note:	For an NWDAF impacts of the split into AnLF and MTLF are FFS.

# * * * * End of changes * * * *