3GPP TSG-WG SA2 Meeting #165	S2-2409794

# Hyderabad, India, Oct 14 – 18, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0574 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF enhancements for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding the following EN in clause 5.18, we introcude a new procedure of Location Measurement data collection from LMF for NWDAF side AI/ML positioning ML model training. Editor's note: The procedure of NWDAF containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS. Regarding the following EN, we refer to clause V.3 of TS 33.501: “Any NF that is deemed an enforcement point for user consent shall support to retrieve the user consent parameters from the UDM”. When the LMF collects input data from UE, the LMF is the enforcement point, so the LMF should check the user consent. Editor's note: Further details on user consent for data collection for model training will be aligned with SA WG3. Clause 5 is about the High Level Features, and clause 6 is about the Location Service Procedures, so it would be better to move the procedure in clause 5.18.1 to clause 6. Clarify that the LMF can request/subscribe to ML model from NWDAF based on the conclusions for KI#1 in TR 23.700-84. When the NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning, NWDAF needs to get data from LMF. However, currently, the LMF does not have services for exposing the positioning measurement data. Therefore, we propose to define new Nlmf_Location_DataExposureSubscribe, Nlmf_Location_DataExposureNotify, and Nlmf_Location_DataExposureUnSubscribe services for the data collection for NWDAF side AI/ML positioning ML model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the following ENs: Editor's note: The procedure of NWDAF containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS. Editor's note: Further details on user consent for data collection for model training will be aligned with SA WG3. Introduce a new procedure of Location Measurement data collection from LMF for NWDAF side AI/ML positioning ML model training. Change clause 5.18.1 to clause 6.X.1. Enhance the procedure for NWDAF assistance to location services to support ML model provisioning from NWDAF to LMF. Define new LMF services for the data collection for NWDAF side AI/ML positioning ML model training. Other enhancements based on conclusions for KI#1 in TR 23.700-84. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The exposure of LMF Location Measurement data and other enhancements based on KI#1 conclusions in TR 23.700-84 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X (new), 4.4.13, 5.18, 6.X (new), 6.X.1, 6.X.2 (new), 6.21.2, 6.21.X (new), 8.3.1, 8.3.2.1, 8.3.2.X (new), 8.3.2.Y (new), 8.3.2.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 1214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (new) * * * *

4.3.X	Network Data Analytics Function, NWDAF

NWDAF may collect data from LMF to perform ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

NWDAF may provide ML model to LMF for performing AI/ML Positioning.

NWDAF may provide Location Accuracy analytics to LMF.

# * * * * Next change * * * *

4.4.13	NL8 Reference Point

The NL8 reference point supports LMF to receive location related analytics from NWDAF, data collection request sent by NWDAF to an LMF, or ML model provisioning from NWDAF to an LMF, as defined in TS 23.288 [37].

# * * * * Next change * * * *

5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 6.X.1.

Editor's note:	The procedure of LMF collects input data from NG-RAN for ML model training is FFS.

LMF may also request a ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF. LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37]. LMF requests ML model for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest. NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.X.2, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37].

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

Editor's note:	The Performance monitoring procedures are FFS.

# * * * * Next change * * * *

6.X	Procedures of Location Measurement data collection for LMF-based AI/ML Positioning

6.X.1	Data collection at LMF to train the LMF-based AI/ML Positioning model to perform positioning based on UE measurements

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Editor's note:	The procedure to collect the input data for AI/ML based positioning from NG-RAN is FFS.

Figure 6.X.1-1: Data collection by LMF to train the LMF-based AI/ML Positioning model using UE measurements

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

Editor's note:	Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

3.	The LMF checks the user consent for data collection for model training.

Editor's note:	Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the LMF-based AI/ML Positioning model.

# * * * * Next change (new) * * * *

6.X.2	Location Measurement data collection by NWDAF for AI/ML positioning ML model training

Figure 6.X.2-1: Procedure of Location Measurement data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF, the service operation includes an AoI. If the NWDAF wants to collect the Location Measurement data of PRUs, the NWDAF also includes a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

NOTE: 	The Location Measurement data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to Location Measurement data from LMF by invoking Nlmf_Location_DataExposureSubscribe / Nlmf_Location_DataExposureUnSubscribe service operation. The NWDAF includes an AoI to request the Location Measurement data from LMF. The NWDAF may also include an PRU indication to indicate obtaining the PRU information if the NWDAF wants to collect the Location Measurement data of PRUs.

4.	The LMF performs steps 1 to 4 of the measurements data collection procedure defined in clause 6.X.1. After performing step 2 in clause 6.X.1, the LMF first filters out PRUs from the UE IDs collected from AMF if the PRU indication is indicated in step 3.

5.	The LMF sends the collected Location Measurement data to the NWDAF by invoking Nlmf_Location_DataExposureNotify service operation, then the NWDAF trains the ML model based on the Location Measurement data. The Location Measurement data includes the PRU location measurement(s) and associated PRU known location if the PRU indication is indicated in step 3.

# * * * * Next change * * * *

6.21.1	General

This clause specifies procedures for NF in the LCS system to interact with NWDAF for data analytics. General framework for NF in the LCS system to interact with NWDAF for data analytics refers to the clauses 6.1.1 and 6.1.2 of TS 23.288 [37].

Clause 6.21.2 describes the procedures of Location Accuracy Analytics retrieved by LMF.

Clause 6.21.3 describes the procedures of UE Mobility Analytics retrieved by AMF.

Clause 6.21.X describes the procedures of ML model for LMF-based AI/ML Positioning retrieved by LMF.

NOTE:	In this release of specification, only AMF and LMF can be NF service consumers to NWDAF.

The procedure in Figure 6.21-1 can be used by a 5GC NF involved in 5GC location services to get optional assistance from NWDAF as shown below.

Figure 6.21-1: Procedure for NWDAF assistance to location services

1.	The AMF may invoke Nlmf_Location_DeterminationLocation service operation towards the LMF to request the current location of the UE.

2.	Assistance from NWDAF for location services may be requested from the LMF and/or AMF.

3.	The LMF may request ML model from NWDAF by invoking Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operations as defined in TS 23.288 [37].

4.	The NWDAF provides the ML model to the LMF by invoking Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelInfo_Request Response service operations.

5.	The LMF may trigger UE positioning. The LMF may select LMF-based AI/ML Positioning as method to perform location calculation.

6.	The LMF may return the location estimate to AMF as specified in clause 8.3.2.

# * * * * Next change (new) * * * *

6.21.X	ML model Retrieval by LMF

LMF may subscribe/request ML model from NWDAF to perform location calculation.

The procedure to subscribe/request ML model by LMF is referred to clause 6.2A of TS 23.288 [37].

# * * * * Next change * * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify |  | AMF |
|  | DataExposureSubscribe | Subscribe/Notify | NWDAF |
|  | DataExposureNotify |  | NWDAF |
|  | DataExposureUnSubscribe |  | NWDAF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |

# * * * * Next change * * * *

8.3.2.1	General

Service description: This service enables an NF to request location determination for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally local and/or civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally local and/or civic location of a target UE for some certain events.

-	Allow the consumer NF to get notified about the geodetic and optionally local and/or civic location of a target UE when some certain events are detected.

-	Allows the consumer NF to cancel location event reporting for a target UE.

-	Allows the consumer NF to transfer location context information for location event reporting for a target UE.

-	Allows the consumer NF to request the PRU location measurement.

-	Allows the consumer NF to request to set up, modify or terminate a secure LCS-UP connection for a target UE.

-	Allows the consumer NF to get notified about status or modification of a secure LCS-UP connection for a target UE.

-	Allows the consumer NF to request the Location Measurement data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

The events to trigger location estimation notification are defined in clause 4.1a.5.1

# * * * * Next change (new) * * * *

8.3.2.X	Nlmf_Location_DataExposureSubscribe service operation

Service operation name: Nlmf_Location_DataExposureSubscribe

Description: Allow the consumer NF to subscribe the Location Measurement data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Target Address, Notification Correlation ID.

Input, Optional: AoI, PRU indication, Subscription Correlation ID (in the case of modification of the subscription).

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

# * * * * Next change (new) * * * *

8.3.2.Y	Nlmf_Location_DataExposureNotify service operation

Service operation name: Nlmf_Location_DataExposureNotify

Description: Allow the consumer NF to get notification about the Location Measurement data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Correlation ID.

Input, Optional: Location Measurement data (e.g., PRU location measurement(s) and associated PRU known location), Subscription Correlation ID (this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation ID by the LMF).

NOTE:	The Location Measurement data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * Next change (new) * * * *

8.3.2.Z	Nlmf_Location_DataExposureUnSubscribe service operation

Service operation name: Nlmf_Location_DataExposureUnSubscribe

Description: Allow the consumer NF to cancel subscription to Location Measurement data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *