3GPP TSG-WG SA2 Meeting #165	S2-249824

Hyderabad, India, Oct 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	China Mobile

Title:	Conclusions for KI#2 on Identifier and ID Management

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusions for KI#2 are proposed.

1. Introduction/Discussion

This contribution is prepared to capture some conclusions for KI#2.

2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13.

FIRST CHANGE

8.2	Conclusion on Key Issue #2

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the conclusion on Identifier  and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

Editor's note:	How it is used is FFS.

The ambient IoT device Identifier shall identify a specific Ambient IoT Device (e.g. a GS1-owned EPC or operator-owned identifier).

The 5GS shall be able to identify the operator owner of an ambient IoT device based on the network identifier and determine the credential holder based on the network identifier.

NOTE 1: The Network Identifier is an identifier to indicate the operator (e.g. MCC+MNC) who holds the subscription-like information for the Ambient IoT Device.

The 5GS shall only support read-only operations to handle ambient IoT device identifier-related management procedures.

The 5GS may support the permanent IoT device identifier (e.g., Tag ID).

END OF CHANGES