3GPP TSG-SA WG2 Meeting #165	S2-2409841

# Hyderabad, India; 14th – 18th October 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5662 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of regenerative-based satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-09-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of regenerative-based satellite access is concluded in clause 8.1 of TR 23.700-29. - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. According to the conclusion of RAN3#125 meeting: NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Further discussions on benefits to introduce Option 3 NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companie to have 2 methods for NG interface management. Inactive UE support RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. The N2 interface management should be enhanced to support the removal procedure for regenerative-based satellite access. Additionally, how AMF pages UE when NG-RAN embedded on a satellite should also be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update “Support for integrating NR satellite access into 5GS” to support regenerative-based satellite access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regenerative-based satellite access is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.1, 5.4.11.a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.4.11	Support for integrating NR satellite access into 5GS

5.4.11.1	General

This clause describes the specific aspects for NR satellite access including transparent-based satellite access and regenerative-based satellite access.

* * * Next Change * * * *

5.4.11.a	Enhancement of N2 interface management for regenerative-based satellite access

When a satellite with an on-board NG-RAN leaves an area served by a specific 5GC, the on-board NG-RAN has an imminent connection loss/deteriorated connection with the specific 5GC. The NG-RAN node and AMF shall support to remove the NG connection as described in TS 38.413 [34], to avoid the AMF sending the signaling over N2 interface to an unavailable NG-RAN, e.g. paging requests or configuration update.

Editor's note:	Whether NG suspend/resume for N2 interface is introduced will be further determined by RAN WG3.

* * * End of Changes * * * *