3GPP TSG-SA2 Meeting #165	S2-2409858

14 - 18 October, 2024, Hyderabad, India

Source:	Nokia

Title:	KI#1: Use Pseudo Analytics ID for model storage and retrieval and MTLF discovery

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution analyses how to store and retrieve models and how to discover an MTLF related to key issue 1. A related CR is in S2-2409859.

# Discussion

In TR 23.700-84, the following conclusion was agreed for KI#1

Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training.

NOTE 1: When receiving the request for a UE location, the LMF selects an appropriate method to determine the UE location and may select AI/ML positioning as method.

Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning, the trigger for data collection and for model training in LMF is up to implementation.

Principle #3: MTLF is enhanced to perform model training for AI/ML based Positioning.

-	Principle #3.1: To retrieve a model to perform location calculation, the LMF discovers a suitable MTLF via NRF.

-	Principle #3.2: NWDAF containing MTLF trains ML model for AI/ML based Positioning based on request from the LMF or internal trigger.

-	Principle #3.3: MTLF collects training data (e.g. data as defined by RAN for AI/ML based Positioning and historical data stored in ADRF) from data sources.

NOTE 2:	Whether LMF can also retrieve a trained model from OAM or not will be discussed in normative phase and need to coordinate with SA5.

This contribution provides additional analysis related to how to store and retrieve models and how to discover an MTLF

Observation 1: The LMF ML model(s) trained by NWDAF containing MTLF should be retrieved by LMF.

According to Clause 5.2 in TS 23.288:

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

Observation 2: The existing mechanism for discovery of NWDAF containing MTLF handling certain ML models are based on the analytics ID.

In Clause 6.2A.1 in the same specification:

If the NWDAF service consumer subscribes to a (set of) trained ML Model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer by invoking Nnwdaf_MLModelProvision_Notify service operation with:

-	For each Analytics ID requested by the service consumer, a set of pair(s) of unique ML Model identifier and the associated Information.

Observation 3: The NF consumer needs to use analytics ID to discover and retrieve the available set of ML model(s).

Observation 4: There will not be any analytics (statistics or predictions) that are provided by an AnLF based on the AIML model trained for LMF location determination.

The agreed text in 23.288 contains the following related editor´s notes:

Editor's note:	How to indicate supporting model training for the LMF-based AI/ML positioning as defined in TS 23.273 [39] by the MTLF to NRF, e.g., using a specific analytics ID or a new indication is FFS.

Editor's note:	When the service consumer is LMF, whether to use a specific analytics ID or a new indication to indicate to retrieve ML model for LMF based AI/ML positioning is FFS.

Considering the observations above, there are two key approaches possible for supporting training, and LMF model management in the core:

Option 1: New Identifier for LMF ML Models independent of Analytics ID

This approach introduces a new identifier for related to LMF machine learning (ML) models. This identifier helps discover the training node, provision models, and related services (like training). It can also be added to the MTLF NF profile at the NRF to enable the discovery of an MTLF handling such a model.

However, this new identifier would need to be added to multiple service operations, and there may also be backward compatibility issues when the Analytics ID as mandatory parameter is omitted. Also existing security procedures for the distribution of models would be impacted.

It has been argued that a new identifier would express the differences in required logic at NRF and MTLF more clearly, but the authors believe that there are no significant differences whatsoever.

In summary, this option requires significant modifications and enhancements to NRF, NWDAF, and MTLF and related implementation effort

It is unclear due to the backward compatibility issues whether this approach is feasible at all.

Option 2: Pseudo-Analytic ID for LMF ML Models

This option proposes a pseudo-analytic ID for LMF ML models. This approach allows reusing current model retrieval related services and discovery procedures without modifications to their parameters. The only standardization requirement is defining a new analytic ID.

Proposal 1: A pseudo-analytics ID is paired with the LMF ML model(s) enabling a backward compatible discovery of MTLF and retrieval of LMF ML model(s).

The training procedure is a computational and energy exhaustive task, and it is more practical to store the train models in a repository for the future uses. While it is possible the training node keep the trained models internally, the ADRF has already introduced for data and model repository. Clause 6.2B in TS 23.288 addresses procedures for storage, retrieval, and removal of ML model(s) in/from ADRF. These procedures are initiated by NWDAF containing MTLF, the NF performing training. Regarding LMF-based model, the following changes are required:

- NWDAF containing MTLF should be able to store LMF ML model(s) in ADRF

Observation 5: The NWDAF containing MTLF performing ML model training may store, retrieve, and remove the related ML model in the ADRF.

Input data for model training may also be stored in the ADRF by the MTLF. This concept should also be applied for input data for training LMF side models.:

Observation 6: The NWDAF containing MTLF performing the ML model training may store, retrieve, and remove related input data in the ADRF.

Summary of Proposals

Proposal 1: A pseudo-analytics ID is paired with the LMF ML model(s).