3GPP TSG-SA2 Meeting #165	S2-2409866

Hyderabad, IN  October 14-18, 2024

Source:	Nokia

Title:	KI#3: Analysis for TR Conclusions and Proposed way Forward

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution analyses key issue 3 and proposes a way forward based on the TR 23.700-84 conclusion. A related CR is in S2-2409867.

# Discussion

In TR 23.700-84, the following (in italics) was agreed for KI#3. We insert observations and provide proposals to help direct the normative work for this KI

Based on PCF request, NWDAF may provide assistance information analytics to PCF to assist the QoS determination and/or modification of QoS parameters. The PCF may take the assistance information into account to determine QoS parameters for a QoS flow, based on the operator's policy.

Observation 1: The NWDAF provides “assistance information analytics to PCF”. QoS parameters are determined by the PCF.

After the PCF determines the QoS parameters, if updated assistance information is received from NWDAF (the updated assistance information may include candidate QoS parameters, the assistance information analytics to PCF), the PCF may determine to modify the QoS parameters for a QoS flow and provide the modified QoS towards the SMF, based on the operator's policy.

Observation 2: The “updated assistance information” may include “candidate QoS parameters”. Note the candidate QoS parameters and their values were originally sent by the PCF in the analytics request (see directly below), for which “assistance information analytics” are provided to the PCF.

In particular:

-	The PCF may send a analytics request or subscription to NWDAF for QoS and policy assistance information analytics. The PCF may include an analytics ID, Filter Information, one or multiple sets of QoS parameters, the candidate value list(s) for each QoS parameter, requesting expected service experience (e.g. QoE), etc.

NOTE 1:	Details of the input parameters in the PCF request will be determined in normative stage.

NOTE 2:	Whether to introduce a new analytics ID or reuse existing one will be determined in normative stage.

Observation 3: The PCF may provide candidate values for one or more sets of QoS parameters, and requests “assistance information” analytics comprising expected service experience (e.g. QoE).

-	Based on the PCF request, the QoS and policy assistance information analytics provided by NWDAF may include: expected QoE can be achieved by the candidate QoS parameters (the values of the QoS parameters are within the candidate value list provided by the PCF).

Observation 4: The assistance information analytics output is: Expected QoE for the candidate QoS parameters / values.

NOTE 3: Whether the time window(s) associated with QoS parameters can be provided by PCF and can be then used to determine the candidate QoS will be discussed in the normative work.

NOTE 4:	Whether to introduce new or reuse existing service/ service operation for providing candidate QoS will be determined in normative phase.

-	The PCF may request one or more analytics from the NWDAF that are used within the same analytics target period, i.e. Observed service experience, QoS sustainability, Network Performance analytics, analytics of the duration and usage of the established QoS Flows.

Observation 5: These analytics (Observed service experience, QoS sustainability, Network Performance analytics, analytics of the duration and usage of the established QoS Flows) are different from the “assistance information analytics” comprising expected QoE for the candidate QoS parameters / values discussed above.

Note: the two types of analytics support different Use Cases (#1 vs #2) described in the TR

NOTE 5:	Possible enhancements to the existing analytics and/or introducing new analytics will be determined in the normative phase.

NOTE 6	Details of the input data and corresponding data sources for analytics will be determined in normative stage.

NOTE 7:	To provide the assistance for policy control and QoS, NWDAF shall not be aware of PCF internal logic.

NOTE 8: NWDAF can monitor the accuracy performance of the assistance information analytics based on a request from PCF. Whether to reuse the existing procedures for Analytics Accuracy Monitoring (defined in clause 6.2D of TS 23.288 [5]) or not can be decided in the normative phase.

Observation 6: The following are not supported by the TR conclusions and therefore should not be considered for normative work:

- The NWDAF provides recommended QoS parameters to the PCF.

- The PCF provides a “desired” QoE to the NWDAF.

- The PCF provides a range for a QoS parameter rather than candidate QoS parameters / values to the NWDAF

Proposal l: To algin with the TR conclusion, the following analytics may be separately supported in normative work:

- Assistance information analytics comprising expected service experience (i.e. QoE) for each set of candidate QoS parameters (with candidate values) provided by the PCF.

- Observed service experience, QoS sustainability, Network Performance analytics, analytics of the duration and usage of the established QoS Flows for an analytics target period specified by the PCF. It is TBD whether enhancements to these existing analytics and/or introducing new analytics are needed.

Proposal 2: The following are not supported by the TR conclusions and therefore should not be considered for normative work:

-	The NWDAF provides recommended QoS parameters to the PCF.

-	The PCF provides a “desired” QoE to the NWDAF.

-	The PCF provides a range for a QoS parameter rather than candidate QoS parameters / values to the NWDAF

Proposal 3: A new analytics ID is used for the new Assistance information analytics comprising expected service experience (i.e. QoE). The new analytics may be based on the current Observed Service Experience analytics, modified to provide analytics for each candidate set of QoS parameters provided by the PCF.

The above proposal for QoE Assistance information analytics are reflected in the S2-240xxx CR for TS 23.288.