3GPP TSG-SA2 Meeting #165	S2-2409871

# Hyderabad, India, October 14 – 18, 2024	(revision of S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5033 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G Femto Hosting Party |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LS from SA3 on Clarification regarding definition of 5G NR femto ownership (S2-2409604/S3-243518) includes the following recommendation to SA2. As per TS 33.320, Hosting party: The party hosting the H(e)NB and having a contract with the PLMN operator. SA3 considers that hosting party terminology should be inherited in 5G, and 5G NR Femto owner should be considered same as hosting party. Also, we recommend using hosting party as the terminology for normative work in 5G SA2 as well as SA3. It is considered that using "hosting party" instead of "CAG owner" is appropriate and aligned with SA3 terminology. However, just "hosting party" may be not crystal clear to express 5G Femto owner and TS 22.220 has the following term. H(e)NB Hosting Party: A H(e)NB Hosting Party has a contractual relationship with the operator, related to the provision of access to the operator’s network via one or more H(e)NBs. NOTE: A H(e)NB Hosting Party is likely to have the billing relationship with the operator. A H(e)NB Hosting Party will typically be the “lead” user in a household but could be e.g. the corporate IT manager in an enterprise context. Therefore, defining new term "5G Femto Hosting Party" in TS 23.501 and using it are proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §4.15.6.3h - Replace "5G Femto CAG owner" with "5G Femto Hosting Party" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G Femto related term is not aligned with other WGs, e.g. SA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3h |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 5666 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.15.6.3h	5G Femto CAG provisioning information parameters

The 5G Femto information provisioning parameters are the parameters sent from an AF by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2. The AF request can be driven by the 5G Femto Hosting Party or an authorized administrator. The 5G Femto information provisioning includes the parameters listed in Table 4.15.6.3h-1. According to the received 5G Femto information provisioning parameters at NEF, the NEF updates the UDM with the provided information. The UDM updates the allowed CAG list of the given UE(s) and notifies AMF via Nudm_SDM_Notification Notify message as defined in clause 4.15.6.2. Then, AMF initiates UE Configuration Update procedure to the UE(s) to update the allowed CAG list of the UE(s) along with the provided validity timer as described in clause 4.2.4.2.

NOTE:	The existing CAG concept defined for PNI-NPN in 5.30.3.3 of TS 23.501 [2] is re-used for 5G Femto information provisioning.

Table 4.15.6.3h-1: Description of 5G Femto information provisioning parameters

| 5G Femto information provisioning Parameters | Description |
| --- | --- |
| UE ID(s) | Indicates the unique identifier for the user to be updated with provided CAG information. It can be GPSI or MSISDN. |
| CAG ID(s) | Indicates the CAG ID(s) to be included to the existing UE Allowed CAG list. |
| Validity time | Indicates the expiry time if the CAG information is only valid temporarily. If no expiry time is present, the provided CAG information will be valid until UE leaves the CAG cells or the next update is performed. |

* * * * End of Changes * * * *