3GPP TSG-SA2 Meeting #165	S2-2409873

# 14 – 18 October 2024, Hyderabad, India							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1221 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General Vertical Federated Learning Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Toyota |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for general Vertical Federated Learning procedure taking into account the agreed Principles in clause 8.2 of 3GPP TR 23.700-84, namely P#2.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made taking into account the agreed principles. P#2.4: For VFL training process: P#2.4.1: Either the NWDAF or the AF can act as VFL server and initiate VFL training process with the VFL client(s). The general procedure supports the VFL server being either NWDAF or AF. P#2.4.2: If an untrusted AF is involved in the VFL training process, their interactions with the NWDAF(s) are via NEF. When the NWDAF acts as VFL Server, the NWDAF can receive labels from an AF. Step 2 indicates that if VFL server is AF the request may be sent via NEF. P#2.4.3: An identifier is allocated by a VFL server, which is used to correlate the participants during the VFL training and subsequent VFL inference processes and it is associated with the distributed ML Models in the VFL joint model training process. Step 2: Introduces the VFL training identifier in step 2 that is assigned by the VFL server. P#2.4.4: VFL Clients compute the intermediate results for their local ML models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server. Steps 2 through 5 provide the details P#2.4.5: VFL clients may also provide intermediate results (e.g. gradient information, loss information) to other VFL clients as instructed by the VFL server. This principle is not covered in this CR. P#2.4.6: An AF or NWDAF acting as VFL server aggregates intermediate results from VFL client(s), trains a local model, computes intermediate results based on its local ML model, and sends the intermediate results towards VFL clients involved in the joint VFL training process. Step 5 provides the details. P#2.4.7: VFL server may compute different intermediate training information (e.g. gradient information, loss information) for updating its own local model and the models of VFL clients during the VFL training process after processing the received intermediate results (that may include convergence reports), sends the updates to the VFL client(s), and the VFL server/client(s) update their local ML model based on the received information. The VFL server determines when the VFL training process terminates, then it will inform the VFL Clients that the training ends. Steps 6 7 provide the details NOTE 6: Whether further specification on how the AF provides the label to NWDAF is needed in P#2.4.2 will be discussed in normative work. It is suggested that there is no need for the AF to provide the labels to the NWDAF. NOTE 7: Whether and how P#2.4.5 is supported will be determined in normative phase. NOTE 8: How the initial model is provided to VFL clients is to be discussed in normative phase. It is suggested that there is no need for the AF to provide the labels to the NWDAF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on vertical federated learning procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE ****************************************

#### 6.2H.2.x	General Vertical Federated Learning procedure among NWDAFs and/or AFs

Figure 6.2H.2.x-1: General procedure for Federated Learning among Multiple NWDAF where VFL Server is NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to VFL server (i.e. NWDAF) to retrieve an ML Model, using Nnwdaf_MLModelProvision including Analytics ID, ML Model Monitoring information as defined in clause 7.5.2, desired ML Model metric (e.g. ML Model Accuracy).

If the consumer (i.e. the NWDAF containing AnLF or NWDAF containing MTLF) provides the Time when the ML Model is needed, the VFL Server NWDAF can take this information into account to decide the maximum response time for its VFL Clients (NWDAF(s)/AF).

1.	VFL Server (NWDAF or AF) selects NWDAF(s) containing MTLF (VFL Client NWDAF(s)) and VFL Clients from AF as described in clause 6.2H.2.1).

2.	VFL Server sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) and AF(s). The request includes the VFL training ID and may include the maximum response time, the VFL Clients has to report the interim VFL parameters (e.g. Loss) to the VFL Server NWDAF before the maximum response time elapses.

If the VFL Server is an AF the request may be sent via the NEF.

If the VFL Server is an NWDAF the request to VFL Clients AF may be sent via the NEF.

NOTE X:	The procedure for the VLF Server AF interfacing with VFL Client(s) AF is out of scope of 3GPP

3.	[Optional] Each VFL Client NWDAF/AF collects its local data by using the current mechanism in clause 6.2 if the Client NWDAF has not local data available already. The VFL Server may also collect local data to train its own model if local data are available.

NOTE Υ:	The procedure for data collection by the VFL Client AF is out of scope of 3GPP

4.	During Vertical Federated Learning training procedure, each VFL client trains its local model based on the Analytic ID and VFL training identifier. Each VFL Client NWDAF and/or VFL Client AF further trains its own ML model based on its local data and reports interim VFL parameters (e.g. Loss) to the VFL Server NWDAF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response. EachVFL Client may also include a local accuracy metric providing an indication of the quality of the local data (i.e. an indication of the data distribution has changed).

Editor’s Note:	Details of the procedure where each VFL client does not provide results within a maximum response time are FFS.

5.	The VFL Server NWDAF trains its own ML model based on interim VFL parameters provided by VFL clients and local data information. The VFL Server NWDAF computes contribution weight for eachVFL client and the ML model of the VFL server based on factors such as importance of feature, number of intermediate results provided by each VFL client. The VFL server also computes a global accuracy metric of the VFL training process taking into account derived contribution weights.

6a.	[Optional] Based on the consumer request in step 0, theVFL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

6b.	[Optional] The consumer decides whether the current model can fulfil the requirement, and determines to stop or continue the training process. The consumer re-invokes Nnwdaf_MLModelProvision_Subscribe service operation as used in step 0 to continue the training process or invokes Nnwdaf_MLModelProvision_Unsubscribe service operation to stop the training process.

6c.	[Optional] Based on the subscription request sent from the consumer in step 6b, theVFL Server NWDAF updates or terminates the currentVFL training process.

If theVFL Server NWDAF received a request in step 6b to stop the Federated Training process, steps 7 and 8 are skipped.

7.	If the VFL procedure continues, VFL Server NWDAF may determine VFL Client NWDAF/AF as described in clause 6.2C.2.3 and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to selectedVFL Client NWDAF(s)/AF(s) for next round of Federated Training. The request includes VFL parameters that will be used by eachVFL Client to train its own ML model.

8.	Each VFL Client NWDAF/AF updates its own ML Model based on VFL parameters provided by theVFL Server NWDAF at step 7.

NOTE G:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

9.	After the training process is complete, the VFL Server NWDAF may send Nnwdaf_MLModelProvision_Notify that the ML model using VFL training is complete.

************************************ END OF CHANGES ****************************************