3GPP TSG-SA2 Meeting #165	S2-2409886

# 14 – 18 October 2024, Hyderabad, India							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1222 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of distributed inference procedure for a model trained using VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Toyota |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for Vertical Federated Learning inference taking into account Principle#2.5 in clause 8.2 of 3GPP TR 23.700-84. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are added according to agreed principles: P#2.5: For VFL inference process: P#2.5.1: NWDAF acting as VFL server scenario, the NF consumer of analytics will obtain the required output based on the VFL inference process coordinated by VFL server, which is generated via the VFL inference process between VFL sever and corresponding AFs or NWDAF acting as VFL client(s): - For NWDAF acting as VFL server, NWDAF triggers the VFL inference phase after receiving the subscription or request for analytics and delivers the analytics to the NF consumer. Already agreed as part of steps 0 1. P#2.5.2: For the AF acting as a VFL server, the AF acting as VFL server can also start an inference process with corresponding NWDAF acting as VFL client(s). The AF may be triggered to start the inference by a 5GC consumer (i.e. NWDAF containing AnLF). Interactions are via NEF if AF is untrusted. Clarifying in step 4 the procedure where the AF is the VFL server. Adding in a note that the procedure between VFL Server AF and VFL client AF is out of scope of 3GPP. Removing relevant EN P#2.5.3: Before performing the VFL inference, the NWDAF acting as VFL server determines corresponding AF(s) and/or NWDAF(s) as VFL client(s) for the inference process based on the same identifier used in the VFL training process. Related details will be specified in the normative phase. For discovering FL clients adding the following description in step 2. For the case where VFL Server is the NWDAF containing AnLF, if the VFL server already has a trained ML model the VFL server retrieves ML model information internally. Otherwise, the VFL Server retrieves ML model information from the ADRF. ML model information includes the address of VFL Client NWDAF(s)/AF(s), contributions weights, VFL training ID. For the case where the VFL Server is the AF details of ML model information retrieval are out of scope of 3GPP. P#2.5.4: The VFL inference process may be controlled by a set of requirements, i.e. whether the determination if all VFL participants associated with the same identifier are needed in the VFL inference process may be based e.g. on accuracy requirements, the VFL signalling and load cost, contribution weights of each client, and temporal availability of output from VFL participants. For selecting FL clients adding the following description in new step 3 The VFL server selects the VFL clients that were used during ML training process based on the ML model information received. The VFL server select candicate VFL clients taking into account accuracy information/contribution weights of each VFL client. P#2.5.5: When performing VFL model performance monitoring, inference data can be used for model retraining, which is aligned with R18. Not covered in this CR. In addition the following clarifications are made: - In step 1 clarifying that if the NWDAF containing AnLF does not support VFL inference then it discovers a VFL server - Renaming VFL Model Correlation ID to VFL training ID aligned with principle 2.5.3 - Adding description on the procedure for the VFL Server to aggregate inference results taking into account accuracy/contribution weight information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on vertical federated learning inference procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2, 6.2G.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

## 6.2G	Vertical Federated Learning

### 6.2G.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL inference.

### 6.2G.2	Procedures

#### 6.2G.2.1	General inference procedure for vertical federated learning

Figure 6.2G.2.1-1: General inference procedure for vertical federated learning

Editor's note	Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable.

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF discovers the VFL server NWDAF used for training the ML model and sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

Editor's note:	Which service used between the consumer and the VFL server is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to iniate the VFL inference procedure with the VFL clients.

For the case where VFL Server is the NWDAF containing AnLF, if the VFL server already has a trained ML model the VFL server retrieves ML model information internally. Otherwise, the VFL Server retrieves ML model information from the ADRF. ML model information includes the address of VFL Client NWDAF(s)/AF(s), contributions weights, VFL training ID.

For the case where the VFL Server is the AF details of ML model information retrieval are out of scope of 3GPP.

3.	The VFL server selects the VFL clients that were used during ML training process based on the ML model information received. The VFL server select candicate VFL clients taking into account accuracy information/contribution weights of each VFL client.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

4.	If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL training ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the NWDAF and the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL training ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

If the VFL server is the AF and the VFL client is the NWDAF the request may be sent via the NEF.

NOTE X:	The procedure where the VFL Server is the AF and the VFL client is the AF is out of scope of 3GPP>

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

Editor's note:	Additional parameters in the request are FFS, e.g. UE ID(s) which will need to be sent to VFL clients.

5.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

6.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

7.	Each VFL Client sends the intermediate local inference results to the VFL server.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

8.	The VFL server may collect its local data and generate the intermediate local inference results for its own ML model. The VFL Server then aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL training ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the aggregate inference output.

9.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

10.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

************************************ END OF CHANGES ****************************************