3GPP TSG-SA2 Meeting #165	 S2-2409887

# 14 – 18 October 2024, Hyderabad, India							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1223 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of VFL server registration and discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR addresses editor’s notes for the registration and discovery of VFL training procedure taking into account principles agreed in TR 23700-84. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor's note: Whether the FL capability will be extended or a new VFL capability will be defined is FFS. It is proposed to extend the FL capabilty type by including VFL Server/VFL client capability. Editor's note: Whether the VFL server/client needs to register into NRF any interoperability information is FFS. Editor's note: Whether to use Vendor ID(s) or introduce new identifiers for interoperability among NWDAF and AFs is FFS. For both editor’s note it is proposed to introduce a single identifier (VFL interoperability identifier) that allows VFL server to discover interoperable VFL clients to participate in VFL training. The VFL interoperability identifier is per Analytics ID. Editor's note: The content of the AF VFL-related capability is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on Registration, Discovery, Sample Alignment and Selection of FL clients for vertical federated learning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

## 6.2H	Vertical Federated Learning among NWDAFs and AFs

### 6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF.

### 6.2H.2	Procedures

#### 6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning among NWDAFs and/or AFs with NWDAF as the VFL server

Figure 6.2H.2.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is the VFL server and NWDAF(s) and/or AF(s) are the VFL client

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1-3.	VFL Server/Client NWDAF and VFL Client AF register to NRF with its NF profile, which includes NF Type (as defined in clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), VFL interoperability identifier(s) per Analytics ID, VFL server and/or VFL client capability within the FL Capability Type, Address information of NWDAF/AF, Service Area, Time interval supporting FL as described in clause 5.2.

For an untrusted AF, the NEF registers at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes VFL Server and/or VFL client capability within the FL capability type.

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF as the VFL server determines that the ML Model requires VFL based on operator policy, Analytics ID, and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1-1 may be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from a consumer.

If the NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, Time Period of Interest, and Service Area.

Once the VFL Server NWDAF is determined, the VFL Server NWDAF discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, VFL interoperability identifier for the requested Analytic ID and Service Area.

.************************************ END OF CHANGES***************************************