3GPP TSG-SA2 Meeting #165	S2-2409888

# 14 – 18 October 2024, Hyderabad, India							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1224 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical Federated Learning Sample Alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for Vertical Federated Learning Sample Alignment in vertical federated learning taking into account the agreed Principles P#2.3 in clause 8.2 of 3GPP TR 23.700-84 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made according to the agreed principles. P#2.3: For sample alignment for VFL: P#2.3.1: For NWDAF acting as VFL Server, NWDAF triggers sample alignment, may query NWDAFs or AFs acting as VFL clients to query availability of samples, and generates the interclause of samples. Introduce description in a new clause where sample alignment is carried out as part of VFL preparation procedure P#2.3.2 In case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information) , then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training. This principle is not addressed P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples. and NOTE 4: How NEF assists sample alignment in P#2.3.3 will be determined in normative phase. Clarifying in step 6 that the NEF may assist in the sample alignment when the VFL server is the AF. It is clarified that the NEF NEF may optionally carry out sample alignment by selecting samples supported by all VFL client based on the available data provided by each VFL client and the VFL server. P#2.3.4: Feature description information may be registered to the NRF or locally configured in the NWDAF or the AF, negotiated between VFL server and VFL client when performing feature alignment. Feature alignment is optional in a VFL process. This principle is not addressed in this CR NOTE 2: Whether and how the feature alignment is supported will be determined in normative phase. It is proposed feature alignment to carried out during the VFL preparation procedure where the Feature is mapped to an Event ID. NOTE 3: The procedure to perform sample alignment is not agreed yet, i.e. it can be done in a standalone procedure or part of the preparation phase, which will be determined in normative phase. It is proposed sample alignment to be carried out during preparation phase NOTE 5: For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase. It is proposed the NEF to perform sample alignment only for the case where the VFL server is an AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on Registration, Discovery, Sample Alignment and Selection of FL clients for vertical federated learning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

##### 6.2H.2.x	Vertical Federated Learning Preparation Procedure

Figure 6.2H.2.x-1: Vertical Federated Learning Preparation Procedure

1.	VFL server/clients registers to NRF with its NF profile as described in steps 1-3 Figure 6.2H.2.1-1.

2	The VFL Server discovers VFL clients as described in steps 3-6 of Figure 6.2H.2.1-1.

3.	VFL server sends a VFL Preparation Request using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, including Analytic ID and Data Set Requirement which includes Features (Event IDs) and Samples needed to train a model.

NOTE X:	If the VFL Server is an AF the request may be sent via the NEF

4.	The candidate VFL clients identify common samples and decide to join or not in the VFL process. Example criteria used by VFL Client(s) may be based on its data availability and time availability, computation and communication capability and VFL Interoperability information.

5.	VFL Client(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response service operation or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the VFL Server whether the VFL client will join the FL procedure and may include the reason in the response message if it cannot join the FL process. The VFL client include in the response the available data (i.e. Event IDs and corresponding Samples) if they decide to join the VFL process.

6.	This step may be carried out if the VFL Server is an AF. If NEF is involved the NEF may optionally carry out sample alignment by selecting samples supported by all VFL client based on the available data provided by each VFL client in step 5 and the Data Set Requirements provided by the VFL server in step 3.

7.	If NEF is involved the NEF sends an VFL Preparation Request response including a list of VFL Clients and corresponding features and aligned samples.

8.	VFL Server determines the final list of VFL Clients to be involved in the VFL procedures. The VFL Server assigns a VFL training ID corresponding to the VFL process and determines initial contribution weights from each VFL participant according to the data availability and/or importance of the feature for the VFL process.

************************************ END OF CHANGES ****************************************