3GPP TSG-WG SA2 Meeting #165	S2-2409891

# Hyderabad, IN, 14th Oct – 18th Oct, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1127 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MFAF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 sends LS (C3-243483) to SA2 asking details about Nmfaf_ContextManagement service which does not have definitions in TS 23.288. Actually, the Nmfaf_ContextManagement service is not needed. When DCCF needs to transfer the data collection context from source MFAF to the target MFAF, it can use the Nmfaf_3daDataManagement_Configure service operation to provide the data collection information to the target MFAF and use Nmfaf_3daDataManagement_Deconfigure to inform the source MFAF to stop the corresponding data collections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure in clause 6.2.6.3.8 by removing Nmfaf_ContextManagement service in the procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MFAF relocation cannot be correctly implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2.6.3.8	DCCF and MFAF relocation initiated by DCCF

The procedure depicted in Figure 6.2.6.3.8-1 is used support DCCF and MFAF reselection by DCCF when the source DCCF or MFAF can no longer serve the UE.

Figure 6.2.6.3.8-1: Procedure for DCCF relocation initiated by DCCF

0.	The data consumer subscribes to source DCCF. The data consumer may indicate in the subscription request with an indicator that the DCCF may execute the relocation procedure.

NOTE 1:	If the source DCCF or target DCCF does not support relocation, the consumer may execute DCCF relocation based on internal logic.

1.	Source DCCF subscribes UE mobility events from AMF. The UE ID is provided by the data consumer in step 0.

2.	If UE moves out of the service area of the source DCCF, source DCCF determines UE DCCF subscription context to be transferred to target DCCF, e.g., triggered by a UE mobility event notification from AMF. If UE moves out of the service area of the source MFAF, source DCCF determines UE MFAF data subscription to be transferred to target MFAF.

3.	Source DCCF may query the NRF to discover and select the target DCCF and/or MFAF, e.g., based on the UE location information received from AMF.

Conditional on MFAF transfer:

4.	Source DCCF sends the Nmfaf_3daDataManagement_Configure requestto the target MFAF, so that the target MFAF is ready to receives UE data from the data source and send the data to the data consumer according to the instructions provided in the Nmfaf_3daDataManagement_Configure request.

5.	Target MFAF sends Nmfaf_3daDataManagement_Configure response to the DCCF with MFAF Notification Information if not provided before.

6.	DCCF updates the data subscription to the data source by changing the Notification Target Address and Notification Correlation ID to the MFAF Notification Address and the MFAF Notification Correlation ID that received in Step 5.

7.	DCCF informs the source MFAF to stop the data collection from the data source by using Nmfaf_3daDataManagement_Deconfigure service operation.

Conditional on DCCF transfer:

8.	Source DCCF using Ndccf_DataManagement_Transfer Request service operation to transfer of UE data subscription context to the target DCCF.

9.	Target DCCF accepts the data subscription(s) context transfer.

10.	If an MFAF is being used, the Target DCCF uses the Nmfaf_3daDataManagement_Configure service to configure the target MFAF.

11.	Target DCCF confirms UE data subscription context transfer to the source DCCF. The confirmation includes the Subscription Correlation ID used by the Target DCCF.

12.	[Optional] Target DCCF subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription context and repeat step 1 to subscribes for UE mobility events from AMF.

13.	Source DCCF informs the data consumer about the successful UE DCCF data subscription context transfer using a Ndccf_DataManagement_Notify message. The notification may contain a Subscription Correlation ID provided by the target DCCF.

14.	[Optional] Source DCCF unsubscribes with the data source(s) that are no longer needed for the remaining UE data subscriptions.

NOTE 2:	At this point, UE data subscription transfer is deemed completed.

For DCCF or MFAF transfer:

15-17.	Target DCCF or MFAF collects data from the data source(s) and notifies the data consumer using a Ndccf_DataManagement_Notify or Nmfaf_3caDataManagement_Notify message.

# * * * * End of changes * * * *