3GPP SA WG2 Meeting #165	S2-2409907

# Hyderabad, India, 14-18 October 2024									                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1466 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting of IMS Avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#8 on " supporting IMS Avatar communication " are agreed. This CR implements the approved conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for Avatar communication, including architecture and procedures for both UE centeric and network centric based models. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS Avatar communication is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, AC.X (new), AC.X.1, AC.X.2, AC.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES (All new)<<<<

# AC.X	Support of Avatar Communication

## AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication. Two modes for Avatar communication rendering are supported: UE centric and network centric.

The rendering of Avatar animation is performed based on Avatar assets, animation data and scene description. The Avatar assets are retrieved from the Digital Assets Repository, which includes the base Avatar model and any personalized accessories. The animation data can be generated by UE or the network from cameras, microphones, specialized motion capturing devices or captured audio/videos streams. The scene description is a document that is generated by the DC AS as a result of composing/updating the 3D scene for all participants in an Avatar communication session.

## AC.X.2	Architecture

### AC.X.2.1	General

AR Application Server

NEF

DC Signaling Function (DCSF)

DC Application Repository (DCAR)

IMS HSS

IMS AS

I/S-CSCF

P-CSCF

UE

MF

IBCF

TrGW

Remote IMS

IMS-AGW

Digital Asset Repository (DAR)

DC4

N33

DC3

DC5

DC1

N72

N71

N70

ISC

Mw

Gm

Iq

Mb

Mb

Mb

Ix

MDC2

MDC1

DC2

DAR1

Mx

DAR2

The following figure depicts the enhanced architecture for AR calls with Avatars.

Figure AC.X.2.1-1: Enhanced architecture for AR calls with Avatars

To support Avatar communication, the data channel architecture supporting AR communication is enhanced as follows:

The Digital Assets Repository (DAR):

-	The DAR stores Avatar assets, including the base Avatar model and any personalized accessories. Base Avatar is a personalized avatar representation. The assets are associated with a particular user and access to them is restricted. The owner can update the assets that are stored in the DAR. Participants in an AR call are authorized to access the base Avatar and a selected subset of accessories during the lifetime of the call.

AR Application Server (AR AS):

-	The AR Application Server is enhanced to support Avatar service control, including Avatar session media control, Avatar media capability negotiation with the UE.

MF:

-	The MF supports Avatar media processing. It receives and stores Avatar service media handling logic from the AR AS, and provides Avatar MF capability, including the generation of animation data, animation and rendering Avatar representation.

-	The MF supports retrieval of Avatar assets from DAR.

The following reference points are used for Avatar communication related signalling.

-	DAR1: Reference point between the UE and the DAR. It is used by the UE to upload and update the base Avatar model and its associated accessories.

-	DAR2: Reference point for MF to access the base Avatar model at the start or during an AR call.

NOTE:	The DAR1 and DAR2 reference points are not specified in the specification.

### AC.X.2.2 UE Centric Avatar Communication

#### AC. X.2.2.1	Sending UE based animation and rendering

For the UE centric case with sending UE based rendering, the sending UE animates the Avatar locally based on the Avatar base model, personalized accessories and the generated animation data. The sending UE uses its input data, e.g. the camera feeds and the user voice to generate the animation data. The animated Avatar traffic is transparent to the IMS network and is sent to the peer UEs via the deployed media functions (e.g., IMS AGW/TrGW).

The rendered Avatar representation is shared with the other participants using traditional IMS media, such as 2D video streams. No changes are expected to the IMS service. Note that this rendering is agnostic to the receiver’s pose and the rendered media is the same for all participants of the Avatar communication session.

#### AC. X.2.2.2	Receiving UE based animation and rendering

For the UE centric case with receiving UE based rendering, The receiving UE animates the Avatar locally based on the received Avatar base model, personalized accessories and the animation data. The animation data can be generated by the sending UE using its input data or generated by MF from received audio/video streams.

### AC.X.2.3 Network Centric Avatar Communication

Network assisted Avatar communication is available depending on device capability and the requirements of the Avatar application on the media transport protocol used between the UE and the MF. Avatar services can be delivered via the following mechanisms:

-	An IMS Media Function (MF) having the Avatar Media Function (AvMF) capability provides required network assisted Avatar media processing. In this case, the MF is tasked with the animation of the Avatar of the participants and the network rendering of the 3D scene for all participants. Note that in this version of the specification, network animation and rendering may not be separated, i.o.w. if Avatar animation is performed by the MF, then network rendering shall also be performed by the AvMF.

## AC.X.3	Procedures

### AC.X.3.1 UE Centric Avatar Communication

#### AC.X.3.1.1	Sending UE based rendering

Figure AC.X.3.1.1-1: Establishment of Sending UE rendered Avatar communication session

Figure AC.X.3.1.1-1 depicts a typical call flow to establish a Sending UE (UE-A) rendered Avatar communication. The main steps in the call flow are as follows:

0a.	UE-A generates or updates a conformant Avatar representation of the user and uploads it to the DAR. The representation may include the Avatar base model as well as personalized accessories, such as garments. UE-A may upgrade each of the components independently.

0b.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels.

1.	The user of UE-A decides to initiate Avatar communications and selects the digital asset (including Base Avatar and personalized accessories) locally.

2.	UE-A upgrades the IMS session for Avatar communication. UE-A initiates a re-INVITE adding media descriptors required for Avatar communication, e.g. RTP and/or application data channels.

3.	If UE-A does not have the required digital assets, it sends a request to the DAR to retrieve the required digital assets.

4.	Based on UE local configuration or network policy (e.g., for 3D Avatar communication), UE-A may request the scene description for service control instruction of the Avatar session from the AR Application Server.

5.	UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation data, then animates from the Base Avatar and scene description.

#### 6. UE-A sends the animated Avatar media to UE-B via RTP.

#### AC. X.3.1.2	Receiving UE based rendering

Figure AC.X.3.1.2-1: Establishment of Receiving UE rendered Avatar communication session

Figure AC.X.3.1.2-1 depicts a typical call flow to establish a Receiving UE (UE-B) rendered Avatar communication. The main steps in the call flow are as follows:

0a.	UE-A generates or updates a conformant Avatar representation of the user and uploads it to the DAR. The representation may include the Base Avatar base as well as personalized accessories, such as garments. UE-A may upgrade each of the components independently.

0b.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels that is used to distribute the initial scene description to both participants.

1.	The user of UE-A decides to initiate Avatar communications and selects the digital assets (including Base Avatar and personalized accessories) locally. Considering the local media capability and/or power status, UE-A triggers the request of receiving UE based rendering and sends the chosen digital assets.

2.	UE-A initiates media negotiation with AR AS to perform receiving UE rendered Avatar communication. The AR AS further negotiates the requested media with UE-B.

3.	Based on the network policy of Avatar rendering (e.g., for 3D Avatar Communication), UE-B may retrieve via MF the scene description generated by AR AS.

4.	UE-B retrieves the required digital assets from DAR according to the received digital assets information (e.g., Avatar ID, personalized accessories indication) chosen by UE-A.

5.	The IMS session is updated to establish the required application data channel used to transmit animation data and /or RTP media used to transimt video streams. The MF is instructed by IMS As on how to handle the received media.

6a.	UE-A uses its input data, e.g., the camera feeds and the user voice to generate the animation data, then sends the animation data stream to MF via the established data channel.

6b.	UE-A sends the media streams that are used to generate the animation data to the MF. These streams may include video streams from the user’s cameras and/or the user’s captured audio streams. The MF generates the animation data from the received media streams.

NOTE:	Whether alternative 6a or 6b is performed is determined by media negotiation with AR AS and instructed by scene description.

7.	 The MF then sends the animation data, which it received from UE-A or generated by itself, to UE-B.

8.	UE-B animates UE-A’s Avatar based on the downloaded digital assets and the received animation data. If UE-B receives the scene description, the animated Avatar may be further rendered in the scene environment.

### AC.X.3.2 Network Centric Avatar Communication

Figure AC.X.3.2-1: Establishment of Network Centric Avatar communication session

Figure AC.X.3.2-1 depicts a typical call flow to establish a network rendered Avatar communication. The main steps in the call flow are as follows:

0a.	UE-A generates or updates a conformant Avatar representation of the user and uploads it to the DAR. The representation may include the Avatar base model as well as personalized accessories, such as garments. UE-A may upgrade each of the components independently.

0b.	UE-A and UE-B establish an IMS session that includes audio, video, and bootstrap data channels that is used to distribute the initial scene description to both participants.

- The user of UE-A decides to initiate Avatar communications and selects the digital assets (including Base Avatar and personalized accessories) locally. Considering the local media capability and/or power status, UE-A triggers the request of network based rendering and sends the a description of the chosen digital assets in the request.

2.	UE-A initiates media negotiation with AR AS to perform avatar animation and network rendered Avatar communication.

3.	Based on the network policy of Avatar rendering (e.g., for 3D Avatar Communication), the MF may retrieve the scene description generated by AR AS.

4.	The MF retrieves the required digital assets from DAR according to the received digital assets information (e.g., Avatar ID, personalized accessories indication) chosen by UE-A.

5.	The IMS session is updated to establish the required application data channel used to transmit animation data and /or RTP media used to transimt video streams. The MF is instructed by IMS As on how to handle the received edia.

6a.	UE-A uses its input data, e.g., the camera feeds and the user's voice to generate the animation data, then sends the animation data stream to MF via established data channel.

6b.	UE-A sends the media streams that are used to generate the animation data to the MF. These streams may include video streams from the user’s cameras and/or the user’s captured audio streams. The MF generates the animation data from the received media streams.

NOTE:	Whether alternative 6a or 6b is performed is determined by media negotiation with AR AS and instructed by scene description.

7.	 The MF animates UE-A’s Avatar based on the downloaded Avatar base model and the received animation data and then renders it as part of the scene.

8.	 The MF sends the animated Avatar media to UE-B via RTP.

# >>>>END OF CHANGES<<<<