3GPP TSG-SA2 Meeting #165	S2-2409924

14 - 18 October, 2024, Hyderabad, India

Source:	Nokia

Title:	KI#2: Proposals to address open issues for NWDAF discovery and selection for VFL

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution discusses open issues for NWDAF discovery and selection for VFL as identified by ENs. Related Changes are in S2-2409922.

# Discussion

The following ENs need to be resolved:

Editor's note:	Whether we can use the existing FL capability information IE, or a new VFL capability information IE is introduced, is FFS.

Editor's note:	Whether additional VFL capability information needs to be registered in NRF is required or new IEs are needed in the VFL capability information, is FFS.

Editor's note:	Details of ML Model handling, supported features, Feature alignment and Sample alignment are FFS.

Issue 1: VFL indicator in NF profile

The following options have been discussed at the last SA2 meeting:

Issue 1, Option 1: Make existing FL capability indicator applicable only for HFL, and introduce a new capability indicator for VFL

Issue 1, Option 2: Make existing capability indicator applicable for VFL and HFL, and combine it with a new indicator to express whether the capability indicator relates to VFL or HFL

Observation 1: In Rel-18, only HFL was supported and the FL capability indicator thus related to HFL.

Observation 1 leads to backward compatibility issues for Option 2: A Rel-18 client will ignore unknown elements in an NF profile it receives and thus also the new IE to discriminate between HFL and VFL. But a Rel-19 NWDAF may desire to advertise that it supports only VFL in its NF profile. A Rel-18 client will still mistakenly read from the NF profile that the Rel-19 NWDAF supports HFL.

Observation 2: Option 2 has backward compatibility issues.

Proposal 1: Adopt Option 1 for issue 1, i.e. make existing FL capability indicator applicable only for HFL, and introduce a new capability indicator for VFL

Issue 2: Possible standardisation of VFL features

The support of a feature requires the ability to collect related input data, to handle related models, to receive intermediate results from the server, and to compute intermediate results/gradients for the feature and provide them to the server. The intermediate results/gradients received from the server and sent to the server a strongly related to models. SA2 so far assumes that the content of models is not standardized but rather vendor specific.

Observation 3: A full standardisation of features would likely require a standardisation of related models.

For each analytics ID, several features may be used if VFL is applied and the effort define such features would also become probative considering the number of Application IDs that have been standardized.

Proposal 2: VFL features are not standardized but assumed to be defined by vendors.

Issue 3: Discovery and selection of NF clients supporting different features

The following options have been discussed at the last SA2 meeting:

Issue 3, Option 1: Express feature information in NF profile

Issue 3, Option 2: Use a dynamic feature negotiation

As the support of features requires ability to collect related input data, to handle related models, to receive intermediate results from the server, and to compute intermediate results/gradients for the feature and provide them to the server. It can therefore be assumed that in many implementations specific code would be required to support a feature and the capability of an VFL client to support a feature would be relatively static.

Observation 4: The capability of an VFL client to support a feature is in many cases relatively static.

For such features, an omission of feature information in the NRF and a subsequent dynamic feature negotiation (option 2) could lead to the selection of an excessive number of candidate NFs (with the need to transfer a high number of extensive NF profiles to the VFL server)

Observation 5: Issue 3. Option 2, may lead to significantly higher signalling load than Issue 3, Option 1.

On the other hand, feature information may be deemed sensitive and protection may be deemed desirable. Both Option 1 and Option 2 involve an exchange of feature information between NFs. In both options feature information would be vendor specific and could be encrypted. However, Option 2 offers in addition the possibility to restrict the feature negotiation to NFs of the same vendor.

Observation 6: Issue 3. Option 2, may offer better privacy for feature information than Issue 3, Option 1.

Proposal 3: Standardize support both for Issue 3, Option 1 (Express feature information in NF profile) and Issue 3, Option 2 (Use a dynamic feature negotiation).

Summary of Proposals

Proposal 1: Adopt Option 1 for issue 1, i.e. make existing FL capability indicator applicable only for HFL, and introduce a new capability indicator for VFL

Proposal 2: VFL features are not standardized but assumed to be defined by vendors.

Proposal 3: Standardize support both for Issue 3, Option 1 (Express feature information in NF profile) and Issue 3, Option 2 (Use a dynamic feature negotiation).