3GPP TSG-SA2 Meeting #165	S2-2409932

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1237 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL training procedures with NWDAF as server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per conclusion of 23.288 key issue #2 Vertical Federated Learning with NWDAF as server should be supported. Related smaple allignment and training procedures are still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added New Clauses related to Vertical Federated Learning Procedures for sample allignment and model training with NWDAF as server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Call flows for VFL |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.X, 6.2H.2Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.2H.2.X	Vertical Federated Learning (VFL) sample allignment procedures with NWDAF as VFL server

Figure 6.2H.X.1-2: VFL sample alignment

- Before starting a vertical federated learning, the NWDAF/MTLF acting as VFL training server determines an initial list of samples, e.g. of target UEs to be used as sample for vertical federated learning.

If samples are UEs, for each candidate UE, steps 2 to 11 are performed

- The NWDAF/MTLF acting as VFL training server inquiries at the UDM whether an AMF is assigned to the UE.

- The UDM returns the assigned AMF ID (if any).

- If no AMF is assigned to a UE, the MTLF acting as VFL training server removes this UE from the list of target UEs.

- The NWDAF/MTLF acting as VFL training server inquiries at the UDM whether an there is user consent for UE data collection for model training for the UE

- The UDM returns the user consent.

- If there is no suitable user consent for a UE, the MTLF acting as VFL training server removes this UE from the list of target UEs.

8.-9.	The NWDAF/MTLF discovers the AMF address using the NRF.

10.-11.	The NWDAF/MTLF inquiries the UE location from the AMF.

- If the UE is located outside the desired target area, the NWDAF/MTLF acting as VFL training server removes this UE from the list of target UEs.

For each AF acting as VFL training client, steps 13 to 15 are performed

- The MTLF acting as VFL training server sends a request to the AF to check the availability of samples, e.g. target UEs and provides the list of target UEs.

- The AF checks for each sample, e.g.target UE, whether it can retrieve related input data and otherwise removes the sample from the list of target samples.

- The AF replies with a possibly reduced list of target samples.

For each NWDAF/MTLF acting as VFL training client, steps 16 to 18 are performed if samples are not UEs

- The MTLF acting as VFL training server sends a request to the NWDAF/MTLF acting as VFL training client to check the availability of samples and provides the list of samples.

- The NWDAF/MTLF acting as VFL training client checks for sample whether it can retrieve related input data and otherwise removes the sample from the list of target samples.

- The NWDAF/MTLF acting as VFL training client replies with a possibly reduced list of target samples.

- The MTLF acting as VFL training server computes the intersection of the possibly reduced list of target samples provided by the AFs and NWDAF/MTLFs acting as VFL training clients and uses the resulting intersection list as target samples as sample for the VFL model training.

# * * * * 2nd change * * * *

## 6.2H.2.Y	Vertical Federated Learning (VFL) training procedures with NWDAF as VFL server

Figure 6.2H.2.Y-1: VFL model training with NWDAF as VFL server

- The NWDAF/MTLF acting as VFL training server assigns a unique VFL Model Correlation ID.

Steps 2 to 4 are repeated until the MTLF acting as VFL training server determines that the training is complete ID.

- The NWDAF/MTLF acting as VFL training server sends a request to start a VFL training iteration to each MTLF and AF acting as VFL training client and provides the Analytics ID, Feature ID, and VFL Model Correlation ID. If intermediate results are available, they are provided.

- Each NWDAF/MTLF or AF acting as VFL training client collect input data, updates the own model based on the input data, and calculates so-called “gradients” as intermediate results based on the updated model.

- Each NWDAF/MTLF or AF acting as VFL training client sends the intermediate results to the MTLF acting as VFL training server

- The NWDAF/MTLF acting as VFL training server updates the own model based on the received intermediate results. The NWDAF/MTLF may in addition collect own input data and also use those data to update the model. For each VFL client, it determines intermediate results based on the updated model and provides those intermediate results to the VFL clients in the request to start the next training iteration (step 2).

- Once the NWDAF/MTLF acting as VFL training server determines that the trained model is sufficiently stable to terminate the training, it informs each AF and NWDAF/MTLFs acting as VFL training clients that the VFL training is completed.

- The NWDAF/MTLF acting as VFL training server may store the model it trained in the ADRF together with the Analytics ID, VFL Model Correlation ID, and Feature IDs of all features used to train the model and an indication that the model is for an VFL server.

- Each NWDAF/MTLF acting as VFL training client may store the model it trained in the ADRF together with the Analytics ID, VFL Model Correlation ID and the Feature ID of the feature the model relates to and an indication that the model is for an VFL client.

# * * * * End of changes * * * *