3GPP TSG-SA2 Meeting #165	S2-2409942

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5038 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: SCP Event Exposure to detect signaling storm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | See considerations in S2-2409941 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define SCP Event Exposure service to enable SCP to help detecting a signaling storm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing input dat may impede an NWDAF from detecting a signalling storm |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 5.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.X	SCP Services

#### 5.2.X.1	General

The following table shows the SCP Services and SCP Service Operations.

Table 5.2.26.1-1: NF services provided by the SCP

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nscp_EventExposure | Notify | Subscribe/Notify | NWDAF, DCCF |
|  | Subscribe | Subscribe/Notify | NWDAF, DCCF |
|  | Unsubscribe | Subscribe/Notify | NWDAF, DCCF |

#### 5.2.X.2	Nscp_EventExposure Service

##### 5.2.X.2.1	General

Service description: This service can expose information collected by the SCP to other NFs..

The following events can be notified to a NF consumer:

-	Abnormal Signalling characteristics: An NF, NF set, Service Type, may be provided as target. Areporting threshold (in terms of absolute transaction or deviation from average, or in terms of a minimum percentage failed transactions) and time window may be provided as filters. The SCP reports the observed rate (i.e. number per time intevall) of different types of signalling messages received and sent by different NF instance(s), the percentage of failed transactions, and when an NF is observed to fail to reply to received requests.

NOTE:	The “Abnormal Signalling characteristics” event is expected to be subscribed by an NWDAF to generate analytics related to abnormal signalling (see TS 23.288 [50]).

##### 5.2.X.2.2	Nscp_EventExposure_Notify service operation

Service operation name: Nscp_EventExposure_Notify

Description: This service operation reports the event and related information to the consumer that has subscribed.

Input Required: Event ID, Notification Correlation Information.

Input, Optional:, Event specific parameters as described in clause 5.2.X.2.1, time stamps for the measurements.

Output Required: Result Indication.

Output, Optional: None.

##### 5.2.X.2.3	Nscp_EventExposure_Subscribe service operation

Service operation name: Nscp_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to SCP event exposure notifications.

Input, Required: Event ID(s) defined in clause 5.2.X.2.1, Notification Target Address (+ Notification Correlation ID),.

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, Event specific target amd filtering information defined in clause 5.2.X.2.1

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SCP with this subscription.

##### 5.2.X.2.4	Nscp_EventExposure_UnSubscribe service operation

Service operation name: Nscp_EventExposure_UnSubscribe

Description: The NF consumer uses this service operation to unsubscribe from an existing subscription.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.