SA WG2 Meeting #165 	S2-2409951

# 14 - 18 Oct 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5041 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the UPF event exposure service for any UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 4.15.4.5.3-1 step 2 is performed only if the Analytic Filter Information does not contain DNN/S-NSSAI but only include application server IP address/FQDN. However according to clause 6.8 of TS 23.548, this step may also include the DNN/S-NSSAI, therefore this step may also be triggered if the Analytic Filter information include both the DNN/S-NSSAI and the application server IP address/FQDN. The condition needs to be corrected. According to clause 5.8.2.17 of TS 23.501, if the UPF event subscription is not including any of the following parameters: AoI, BSSID/SSID and DNAI, the NF consumer shall subscribe indirectly via SMF. Therefore the Analytic Filter Information include the DNAI, the UPF event consumer shall subscribe the UPF event via SMF of the PDU Session associated with the DNN/S-NSSAI. If the I-SMF is used for the PDU session the SMF may not support the DNAI, but rather the I-SMF support the DNAI. Step 4 needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct step 2 to cover the case that the subscription include the DNN/S-NSSAI Clarify that the SMF may not support the DNAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consumer may not be able to subscribe the event for UPF associated with a DNAI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

4.15.4.5.3	Information flow for UPF event exposure service for any UE

Figure 4.15.4.5.3-1: UPF Information Exposure to the UPF event consumer (e.g. NWDAF) of any UE scenario

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer sends a request to the NWDAF for analytics on any UE. The analytics consumer provides the value "any UE" in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, Application ID, DNAI, etc.

2.	(Optional and only when the UPF event consumer is NWDAF) If the Analytic Filter Information includes application server IP address/FQDN, the NWDAF should first obtain the DNAI from NEF. The NWDAF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range and/or FQDN.

3.	(Conditional, if step 2 took place) The NEF determines the suitable DNAI(s) and provides them to NWDAF.

4.	The UPF event consumer triggers the SMF(s)/UPF(s) discovery to NRF by Nnrf_NFDiscovery_Request providing the DNN, S-NSSAI, DNAI etc. This procedure is to discover the related SMF(s)/UPF(s) associated with any UE. Either SMF(s) or UPF(s) are discovered depending on whether the subscription request to UPF events meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]).

5.	The NRF provides Nnrf_NFDiscovery_Response that may refer to several SMFs/UPFs.

6.	(Option 1) If the subscribed UPF events needs the SMF(s) to do a third-party subscription onto UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the same procedure as Indirect subscription via several SMFs (steps 3 - 5 in Figure 4.15.4.5.2-1(for single UE)) takes place via each discovered SMF.

7.	(Option 2) If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to all discovered UPFs. The information included in the subscription is:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting: any UE.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

NOTE:	As the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact.

8.	Each of the UPFs invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

* * * End of Change * * *