SA WG2 Meeting #165 	S2-2409954

# 14 - 18 Oct 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0263 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-49 clause 8.1 concluded that I-SMF based approach to reduce the impact on central SMF for discovery EAS is selected for the Local Offloading Management service in key issue 1. This CR propose the update to capture Local Offloading Management service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support of Local Offloading Management service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local Offloading Management service is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.X	Support of the Local Offloading Management

6.X.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. The AMF is locally configured whether the Local Offloading Management can be used for PDU session associated with S-NSSAI and DNN. If Local Offloading Management is used, AMF select/re-select an I-SMF supporting Local Offloading Management. The I-SMF controls the offloaded session traffic to the local part of DN. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message is handled by the I-SMF instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF.

-	Management and retrieval of EDI from NEF to configure EASDF.

NOTE X:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Policy from SMF, which indicates the DNAI, IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

-	Management of N4 association towards the Local UPF.

NOTE Y:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, the SMF does not discover, select and configure EASDF as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The reference architecture supporting this scenario is depicted in Figure 4.2-X in clause 4.2.

6.X.2	Procedure

6.X.2.1	General

This clause describes the procedures for Local Offloading Management.

6.X.2.2	Registration procedure

Registration procedure described in clause 4.23.3 of TS 23.502 applies with the following difference:

-	In Step 10: The following cases are applied for the PDU Session is configured with Local Offloading Management in the target AMF:

-	if the received SMF information includes only SMF information, the (target) AMF selects an I-SMF supporting Local Offloading Management. The I-SMF insertion process is triggered; or

-	if the received SMF information includes both I-SMF and SMF information and the old I-SMF doesn’t support the Local Offloading Management, the (target) AMF selects a new I-SMF supporting the Local Offloading Management. The change of I-SMF process is triggered.

-	In step 17, for I-SMF insertion or change, the (target) AMF invokes Nsmf_PDUSession_CreateSMContext (PDU Session ID, SM Context ID at SMF) towards the new I-SMF. Steps from step 3 onwards as described in clause 4.23.4.3 are executed with the following enhancements:

-	Step 8a, the I-SMF selects Local EASDF and provides the Local EASDF address to the SMF. If the Local Offloading Management is authorized for the PDU Session, in step 8c the SMF may provide the Local Offloading information to the I-SMF.

After the registration procedure, if the I-SMF is provided the Local Offloading information from the SMF, the I-SMF may use the Local Offloading information to select Local UPF and establish N4 Session towards the Local UPF.

6.X.2.3	PDU Session Establishment procedure

When the UE is in non-roaming scenario, the PDU Session establishment for Local Offloading Management reuses step 2-4 as clause 6.7.2.2 by replacing V-SMF, H-SMF, V-UPF, V-EASDF, VPLMN Specific Offloading Policy/Information and HR-SBO with I-SMF, SMF, I-UPF, Local EASDF, local offloading control Policy/Information and Local Offloading Management accordingly, and with following differences:

-	Step 2: If the PDU Session is configured for Local Offloading Management the AMF selects an I-SMF that supports the Local Offloading Management and serves the area where UE camps. The I-SMF selects an Local EASDF and provides the Local EASDF address to the SMF. If the Local Offloading Management is authorized for the PDU Session the SMF then provide local offloading support indication to the PCF. The SMF may receive the Local Offloading policy from the PCF that indicates the IP range(s) and/or FQDN(s) allowed to be routed to the local part of a DN. The SMF generates the Local Offloading information based on local configuration or the Local Offload policy if received from the PCF. The SMF provides the Local Offloading information to the I-SMF. The I-SMF may use the Local Offloading information to select Local UPF and establish N4 Session towards the Local UPF.

-	Step 3: The I-SMF configures the Local EASDF with the DNS handling rules using the local offloading control information received from the SMF.

* * * End of Change * * *