3GPP TSG-SA WG2 Meeting #165	S2-2409967

# Hyderabad, India, October 14 – 18, 2024 									 (revision of S2-2409091)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1121 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Flows Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following principle is concluded for KI#3 in clause 8.3 of TR 23.700-84 - The PCF may request one or more analytics from the NWDAF that are used within the same analytics target period, i.e. Observed service experience, QoS sustainability, Network Performance analytics, analytics of the duration and usage of the established QoS Flows. In the current specifications, the "analytics of the duration and usage of the established QoS Flows" is not supported. This is a new analytics ID introduced in the study phase. This CR provides the specification of the QoS Flows Analytics ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new sub-clause for QoS Flows Analytics ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concluded principles in the study phase are not implemented in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4849 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change (All New Text) **********

## 6.X	QoS Flows analytics

### 6.X.1	General

This clause specifies the procedure for an NWDAF to provide analytics of established QoS Flows. The analytics may be used by PCF in combination with other analytics IDs such as Observed Service Experience and QoS Sustainability Analytics, etc. in determining QoS parameters.

NOTE 1: This analytics ID by itself does not provide QoS profile recommendation to PCF. The PCF may take the outputs of this analytics ID besides other analytics to determine QoS parameters.

Editor's Note: How the PCF uses these analytics ID is to be defined in clause X of TS 23.502 [3] and/or clause X of TS 23.503 [4]; and the reference to TS 23.502 and/or TS 23.503 will be updated.

The NWDAF collects information of established QoS Flows from SMF and then provides analytics (including the duration and number of usage) of QoS Flows.

The service consumer may be a PCF.

The consumer of the analytics includes in the request:

-	Analytics ID = "QoS Flows".

-	Target of Analytics Reporting: a single UE (SUPI).

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Area of Interest;

-	Optional Traffic Descriptor: Application Identifier, IP Descriptions or Domain Descriptors are applicable.

-	An optional list of candidate QoS Profiles (including QoS characteristics and QoS parameters) to filter out the output statistics and/or analytics

-	An optional list of analytics subsets that are requested.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optional Preferred level of accuracy of the analytics.

-	Optional Preferred level of accuracy per analytics subset;

-	Optional preferred order of results for the list of QoS Flows:

-	ordering criterion: "duration" or "number of usages",

-	order: ascending or descending.

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.X.2	Input Data

NWDAF collects input data from the SMF, AF, and AMF to derive the analytics. The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1: Input data from 5GC related to QoS Flow

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | SMF | A time stamp associated with the collected information. |
| UE ID | SMF, AF | SUPI in the case of SMF, external UE ID (i.e. GPSI) in the case of AF |
| UE Location | AMF | The UE location information, e.g. cell ID or TAI. |
| QoS Flows Information (1..max) | SMF | List of QoS flow information |
| > Event type | SMF | The type of QFI change, i.e., QoS Flow establishment (i.e., QFI Change) or QoS Flow termination (i.e., QFI deallocation) or Traffic binding of QoS Flow event |
| > QFI | SMF | QoS Flow identifier |
| > Traff descriptor | SMF | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set |
| > PDU Session ID | SMF | PDU session ID containing the QoS Flow |
| > DNN | SMF | DNN corresponding to the PDU Session |
| > S-NSSAI | SMF | S-NSSAI corresponding to the PDU Session |
| > 5QI | SMF | 5QI of the QoS Flow |
| > QoS parameters | SMF | The value of the QoS parameters including ARP, GRBR, MFBR, AMBR of the QoS Flow |

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.X.2-1 by invoking Nnf_EventExposure_Subscribe service operation with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI Change).

### 6.X.3	Output Analytics

The NWDAF gets the events of QoS Flows establishment and termination from the SMF and uses the information to provide the analytics (i.e. statistics and predictions) of QoS flow information including the duration, number of usage and QoS related information. The output analytics are shown in Table 6.X.3-1 and Table 6.X.3-2.

Table 6.X.3-1: Output statistics of QoS Flow Analytics

| Information | Description |
| --- | --- |
| UE ID | UE ID |
| UE Location | UE Location |
| QoS Flows Information (1..max) (NOTE 2) | Ordered list of QoS Flows information |
| > QoS Flow attributes | A list of QoS Flow attributes |
| >> Traff descriptor | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set |
| >> DNN | DNN corresponding to the PDU Session |
| >> S-NSSAI | S-NSSAI corresponding to the PDU Session |
| >> 5QI | 5QI of the QoS Flow |
| >> QoS parameters | The value of the QoS parameters including ARP, GRBR, MFBR, AMBR of the QoS Flow |
| > Duration information (NOTE 3) | Maximum/Minimum/Average duration of QoS Flows with the specified attributes |
| > Number of usage (NOTE 3) | The number of times that the QoS Flows with the specified attributes are used |
| NOTE 2: This list contains only the information of QoS Flows bounded to the Candidate QoS Profiles if the optional input provided in the request by the PCF. NOTE 3: The duration and number of usage of QoS Flow is determined by NWDAF using the SMF Events QoS Flow establishment (i.e., QFI Change) or QoS Flow termination (i.e., QFI deallocation) or Traffic binding of QoS Flow events; i.e., the duration equals to different between the timestamp of QoS Flow establishment and QoS Flow termination events. |  |

Table 6.X.3-2: Output prediction of QoS Flow Analytics

| Information | Description |
| --- | --- |
| UE ID | UE ID |
| UE Location | UE Location |
| QoS Flows Information (1..max) (NOTE 2) | Ordered list of QoS Flows information |
| > QoS Flow attributes | A list of QoS Flow attributes |
| >> Traff descriptor | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set |
| >> DNN | DNN corresponding to the PDU Session |
| >> S-NSSAI | S-NSSAI corresponding to the PDU Session |
| >> 5QI | 5QI of the QoS Flow |
| >> QoS parameters | The value of the QoS parameters including ARP, GRBR, MFBR, AMBR of the QoS Flow |
| > Duration information (NOTE 3) | Maximum/Minimum/Average duration of QoS Flows with the specified attributes |
| > Number of usage (NOTE 3) | The number of times that the QoS Flows with the specified attributes are used |
| > Confidence | Confidence of the prediction |
| ntains only the information of QoS Flows bounded to the Candidate QoS Profiles if the optional input provided in the request by the PCF. NOTE 3: The duration and number of usage of QoS Flow is determined by NWDAF using the SMF Events QoS Flow establishment (i.e., QFI Change) or QoS Flow termination (i.e., QFI deallocation) or Traffic binding of QoS Flow events; i.e., the duration equals to different between the timestamp of QoS Flow establishment and QoS Flow termination events. |  |

### 6.X.4	Procedures

Figure 6.X.4-1 shows the procedure where the NWDAF provides QoS Flows analytics, in the form of statistics or predictions or both, to PCF.

Figure 6.X.4-1: Procedure for QoS Flow analytics

1a-b. [Optional] To provide the requested analytics, the NWDAF may subscribe per application communication information, which is identified by Application ID, from AFs for the UE. The Event ID "UE Communication information" as defined in TS 23.502 [3] is used, which indicates communication report for the UE which is requested by the 5GC NF in the step 1.

1c.	The NWDAF subscribes to SMF(s) for QoS Flows information via Nsmf_EventExposure_Subscribe (Event ID = QFI Change) service operation.

NOTE 1: The Target of Event Reporting may be e.g., any UE or a specific SUPI based on the implementation.

1d.	The SMF(s) record(s) the events (establishment, release, change) of QoS Flows. The SMF(s) also record(s) the corresponding information of the QoS Flows specified in Table 6.X.2-1.

1e.	The SMF(s) provide(s) the requested input data to NWDAF.

1f-g. The NWDAF retrieves one or more of Type Allocation code and UE location trends from AMF(s).

2.	The NWDAF trains the ML model using the QoS Flows information obtained from the SMF(s) and AMF(s).

3.	PCF sends a request to the NWDAF for the analytics on a specific UE(s), using either Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription_Subscribe service. The analytics type indicated by "Analytics ID" is set to "QoS Flows". The Target of Analytics Reporting is set to SUPI. PCF may also specify a desired ordering e.g. "Duration" to get a list of QoS parameters used for most of the time.

4.	The NWDAF derives requested analytics based on the inputs provided by the PCF, in the form of statistics or predictions or both.

5.	The NWDAF provides requested QoS Flow analytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 3. The output includes a list of QoS information which are ordered based on the ordering specified by PCF in step 3.

6.	If the PCF subscribed QoS Flow analytics at step 3, based on new events from SMF, NWDAF may update the analytics, and then notifies the new generated analytics to the PCF.

7.	Using on the ordered list of QoS information provided by NWDAF, PCF knows the most commonly used QoS information for the UE in the specified time and location; based on this information and considering other influencing factors (e.g. network resources, operator's policy, etc.) and also other Analytics for the same location and the same Analytics target period such as Observed QoS Experience, the PCF determines the QoS parameters in QoS Flow establishment/modification time.

********** End of Changes **********