3GPP TSG SA WG2#165	            S2-2409973

Hyderabad, India, October 14-18 2024 	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5684 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of regulatory service via MWAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This CR introduces the descriptions on how the MWAB supports emergency calls for the UEs it serves |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added to describe how the MWAB is supporting emergency calls for the UEs it serves |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MWAB feature cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.49.8	Support of regulatory service via MWAB

A MWAB-gNB, if configured to do so according to regulations applicable for the MWAB Broadcasted PLMN/SNPN, shall support the emergency and regulatory services for the UEs it serves as specified here below. Location services applicable to MWABs are specified in clause 5.49.7.

When MWAB-gNB determines it serves an emergency session for a UE accessing the MWAB (a PDU session context includes ARP value dedicated to emergency services) the MWAB-gNB, if configured to do so, can request the MWAB UE to adjust the backhaul handling by providing the necessary information over the MWAB-gNB to MWAB-UE interface:

-	The MWAB-UE can be requested, if this is not yet established, to establish new BH PDU Session(s) with specific S-NSSAI(s) and DNN(s) for N2 and N3 connections that is associated to the ARP for emergency services in the BH PLMN. The HPLMN of MWAB-UE provisions specific S-NSSAI(s) and DNN(s) in the MWAB-UE subscription which are associated to BH PDU sessions of N2 and N3 connections used to support emergency services for the UEs served by the MWAB-gNB. These S-NSSAIs are associated with the ARP for emergency services.

-	If the MWAB-UE has PDU sessions established for the S-NSSAI for the BH PDU sessions used to support emergency services, these are not released by the MWAB until it continues serving UEs with ongoing emergency calls.

-	AMFs in the BH PLMN shall not deregister a MWAB-UE which is registered with a S-NSSAI that is dedicated to emergency sessions backhauling, unless there is no more any BH-PDU session with the related S-NSSAI.

-	SMFs in the BH PLMN shall not initiate the release a BH PDU session associated with a S-NSSAI and DNN for emergency, unless the BH PDU session remained inactive for a configurable sufficiently large time (which may be subject to regulations or operator policy).

-	When OAM commands a MWAB-gNB to shut down, the MWAB-gNB attempts to handover UEs it serves to other gNBs before shutting down, but if it is unable to do so, it continues serving the UEs with emergency sessions ongoing and does not release the BH PDU sessions for the support of the emergency calls. Other established PDU sessions (not used for emergency services) can be released in this case.

When MWAB-gNB determines it no longer serves any emergency session(s) for UEs accessing the MWAB, the MWAB-gNB shall request the MWAB UE to release the BH PDU sessions which were established to support the emergency session(s) of the UE(s) served by the MWAB-gNB.

End of CHANGES