3GPP TSG-WG SA2 Meeting #165	S2-2409980

# Hyderabad, India, 14 – 18 October 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5045 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Energy related information collection and exposure procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the conclusion for KI#1 of TR 23.700-66, the support of network energy related information exposure is agreed. The procedures and services for supporting collection and exposure of the energy related information need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure for supporting network energy related information collection and exposure is added. The new services of the new network function are also added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network energy related information collection and exposure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X.Y (new), 5.2.X (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 5689 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text new) * * * *

4.X.Y	Energy related information collection and exposure

The procedure is used by the EESF to collect the energy related information and expose the energy information to the Consumer NF.

Figure 4.X.Y-1: Energy related information collection and exposure

- The Consumer NF subscribes to the energy information exposure and provides the associated notification endpoint of the Consumer NF by sending Neesf_EnergyInfoExposure_Subscribe.

If the Consumer is AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, the NEF sends the event subscription to the EESF via Neesf_EnergyInfoExposure_Subscribe.

The Reporting mode defines the type of reporting requested (e.g., periodic reporting or threshold-based reporting, for Energy related parameters as defined in clause 5.X.Y of TS 23.501 [2]). Target of Event Reporting defines the granularities of the energy related parameters, e.g., SUPI, S-NSSAI, Application Identifier.

2.	The EESF invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID, DNN, S-NSSAI and NF type = SMF and AMF.

The UDM provides a Nudm_UECM_Get response with the corresponding SMF ID and AMF ID.

3.  The EESF sends the Nsmf_EventExposure Subscription request to the SMF to subscribe to UPF data, including the following information:

-	Notification Target Address (EESF address), Subscription Correlation ID.

-	UPF Event ID (i.e. User Data Usage Measures).

-	Event Filter Information, e.g., one or more of the following parameters: S-NSSAI, DNN, DNAI, UPF Id, Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting

4.	The SMF selects the PDU session(s) and the UPFs it has to send the request to. The SMF sends the request to the UPF with Nupf_EventExposure_Subscribe (as defined in clause 5.2.26.2.3). The request includes the EESF address, UPF Event ID, Subsription Correlation Information, Event Filter Information, Target of Event Reporting.

5.	The UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the EESF.

6.  The EESF invokes Nsmf_EventExposure service operation to retrieve the NG-RAN node identifier serving the UE by providing UE ID.

7.   The EESF sends the NG-RAN node identifier and UPF ID(s) serving the UE to the OAM, and collects the energy related information from OAM, i.e.,   DVUPF ,   ECUPF,   ECgNB.

8.  After the calculation and determination of the energy information in the EESF, the EESF sends the information to the Consumer NF via Neesf_EnergyInfoExposure_Notify.

# * * * * Second change (all text new) * * * *

5.2.X	EESF Services

5.2.X.1	General

The following table shows the EESF Services and EESF Service Operations.

Table 5.2.X.1-1: NF services provided by the EESF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neesf_EnergyInfoExposure | Subscribe | Subscribe/Notify | NEF, AMF, SMF |
|  | Unsubscribe | Subscribe/Notify | NEF, AMF, SMF |
|  | Notify | Subscribe/Notify | NEF, AMF, SMF |

5.2.X.2	Neesf_EnergyInfoExposure service

5.2.X.2.1	General

The service provides the energy related information to the consumer NFs.

5.2.X.2.2	Neesf_EnergyInfoExposure_Subscribe service operation

Service operation name: Neesf_EnergyInfoExposure _Subscribe

Description: the consumer subscribes to receive the energy related information from the network, or the subscription is updated if the same subscription is already existing in EESF.

Inputs, Required: Energy related parameter(s) as defined in clause 5.X.Y of TS 23.501 [2], Reporting mode (e.g., periodic reporting or threshold based reporting), Target of Event Reporting (e.g., SUPI, S-NSSAI, Application Identifier, PDU session), Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: External Application Identifier(s), Reporting periodicity, Reporting Threshold, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding energy related information report is included, if available. External Identifier (representing an AF specific UE Identifier).

5.2.X.2.3	Neesf_EnergyInfoExposure_Unsubscribe service operation

Service operation name: Neesf_EnergyInfoExposure_Unsubscribe

Description: the NF consumer deletes a subscription if already defined in EESF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

5.2.X.2.4	Neesf_EnergyInfoExposure_Notify service operation

Service operation name: Neesf_EnergyInfoExposure_Notify

Description: EESF reports the energy related information to the consumer that has previously subscribed.

Inputs, Required: Energy related parameter(s), Notification Correlation Information, time stamp.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * End of changes * * *