3GPP TSG-SA WG2 Meeting #165	S2-2409985

# Hyderabad, India 14-18 October 2024													(was S2-24xywz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1474 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6: Update to Support for Standalone IMS Data Channel feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedures for stand-alone IMS DC are incorrect. In addition, they specify interaction with the MRF although the MRF impacts are removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MRF removed from the figures and steps descriptions. Updated P2P procedure adding application DC to standalone IMS DC session with bootstrap DC. Add new figure AC.10.2.3-1 describing the P2P combined standalone bootstrap DC and application DC setup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.10.1, AC.10.2, AC.10.2.1, AC.10.2.2, AC.10.2.3, AC.10.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

## AC.10.1	General

Data channels can also be established in a standalone IMS data channel session. A standalone IMS data channel session with only standalone bootstrap DC is used for downloading application list and applications from the DCSF. A standalone IMS data channel session with a combination including both bootstrap DC and application DC is used to establish application DC for a downloaded application to the remote UE. This allows the remote UE to download the application if not yet downloaded after accepting the bootstrap DC, and to update the IMS data channel session to use the application in the same session.

NOTE:	The behaviour of the remote UE responding to the standalone IMS data channel session is not mandated and is up to the UE's own implementation, i.e. the remote UE can reject the IMS data channel session and does not download the application.

If a standalone IMS data channel session is supported, the UE can establish standalone IMS data channel session only with bootstrap DC, or with combination of standalone bootstrap DC and application DC. Additionally, a standalone IMS data channel session only with bootstrap DC can be updated by adding application DC to the session.

Editor's note:	Whether the configuration of standalone IMS data channel session is required is FFS.

When standalone IMS data channel session is supported, based on operator policy, additional subscription information for standalone IMS data channel service may be required to be included in service subscription for IMS Data Channel. This is used by the IMS AS to determine whether to accept or reject a standalone IMS data channel session.

When subscription to standalone IMS data channel session is needed, and if a UE that has not subscribed to standalone IMS data channel session establishes a standalone IMS data channel session, the IMS AS rejects the session establishment.

Standalone IMS data channel sessions use SIP session timer as per RFC 4028 [106] to avoid hanging resources in the UE and the network. Usage of SIP session timer in IMS is specified in TS 24.229 [10a].

The standalone IMS data channel session is terminated by the existing session release procedure specified in clause AC.7.6. The session release may be triggered by the expiry of the session timer or by sending a SIP BYE request.

# *** Next Change ***

## AC.10.2	Procedures

# *** Next Change ***

### AC.10.2.1	Originating Standalone Bootstrap DC Setup using PSI

The standalone IMS data channel session setup with only bootstrap DC is triggered by a UE or the user of a UE for downloading application list and/or applications. In this procedure, the UE initiates the IMS data channel towards a PSI configured in the UE for that purpose.

After the standalone bootstrap DC is established, the UE downloads the application list via the bootstrap DC, and may display the list of application information to the user for selection, downloading, and/or initiating standalone application DC. The UE terminates the standalone IMS data channel session when the session timer expired.

Figure AC.10.2.1-1depicts a signalling flow diagram for establishing a standalone bootstrap data channel using PSI.

AC.10.2.1-1: Originating Standalone Bootstrap DC Setup using PSI

1.	UE#1 sends the SIP INVITE request with an initial SDP offer to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is a specific target URI, such as a specific PSI indicating standalone bootstrap DC. The initial SDP offer includes only the bootstrap data channel establishment request with bootstrap DC stream ID.

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure.

2-4.	Steps 2-4 of clause AC.7.1 apply.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered only local bootstrap media, the DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP offer, determines that the UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which is used to receive the UE request for application downloading from the MF.

6.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) service operation based on its policies instructing the IMS AS how to set up the bootstrap data channel with the MF for originating side and not to forward the initial INVITE request i.e. to terminate the initial INVITE request. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct the MF to allocate required data channel media resources for originating side only. The IMS AS requests creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	The IMS AS does not forward initial INVITE request based on the PSI and sends 18X response with the SDP answer through S-CSCF and P-CSCF to the UE#1.

12-13.	The standalone IMS data channel session with only bootstrap data channel is established.

14.	The bootstrap data channel has been established between the originating MF and the UE#1. The UE#1 sends application request messages to the MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF replaces the root URL with the replacement URL received in step 8 and forwards the message to the received media point of DCSF. The DCSF provides the application list and proper data channel applications further to the UE#1 based on their data channel capabilities and their choices through MF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), the UE#1 sends SIP BYE request to release the bootstrap DC to save power.

# *** Next Change ***

### AC.10.2.2	Adding Application DC to Standalone IMS DC Session with Bootstrap DC

This case applies to the scenario where the user selected DC application has not been downloaded yet. The originating UE initiates standalone bootstrap DC setup during the IMS session setup procedure. The standalone bootstrap DC setup is performed separately by the originating UE and terminating UE with the DCSF. The originating UE downloads the DC application selected by the user via the bootstrap DC, updates the IMS session to add application DC with the terminating UE and indicates the selected DC application to the terminating UE. If the indicated DC application is not downloaded yet, the terminating UE downloads the indicated DC application via the bootstrap DC.

In this procedure, the assumption is that the alert to terminating user will only happen after the terminating UE downloaded the application and updated the application DC, i.e. when the indicated DC application is downloaded, the terminating UE alerts the user and displays the application information including the required rights to the user.

When the user of the terminating UE accepts to use the DC application for the communication, the IMS session is answered by the terminating UE.

The procedure specified in clause AC.7.1 applies with the following differences:

-	In step 1, UE#1 sends the initial INVITE request with an SDP offer only including media component for bootstrap data channel for originating side and optional for terminating side. The initial INVITE request additionally includes the P-Early-Media header field to indicate support of early media authorization.

-	In step 2, the IMS AS determines whether the IMS session is a standalone IMS data channel session, and determines if the session is allowed to be established based on local configuration or subscription data. If standalone IMS data channel session is not allowed, the IMS AS rejects the request.

-	In step 11, the media component for bootstrap data channel for terminating side may be included in an SDP early offer to enable the terminating side downloading the DC Application selected by the user of UE#1 before ringing.

-	In step 14 the provisional response includes the P-Early-Media header field to enable the download of DC application by the UE#2, prior to alerting the user.

-	Steps 23-24 may be skipped.

-	In step 25, the UE#2 alerts the user of UE#2 after the corresponding DC Application is downloaded. If UE#2 did not successfully download the DC Application, the UE#2, as an example, may not alert the user at all and tear down the entire IMS session.

-	Steps 10-19 occur after step 25.

NOTE 1:	It is assumed that that UE#2 autonomously is able to download the list of applications and the same application downloaded by UE#1 during the early media phase.

NOTE 2:	The time between alerting UE#2, and the download of the application need to conform to SIP Session timers.

Subsequent to a successful session set up and user alert, the UE#1 upgrades the IMS DC session to add an application  DC. The UE#1 follows procedures from clause AC.7.2 starting at step 1. The user of the UE#2 is alerted only if the UE#2 successfully downloaded the requested application.

# *** Next Change ***

### AC.10.2.3	Combined Standalone Bootstrap DC and Application DC Setup

This case applies to the scenario where the user selected DC application has been downloaded, the originating UE initiates combined standalone bootstrap DC and application DC setup during the IMS session setup procedure. The standalone bootstrap DC setup is performed separately by the originating UE and terminating UE with the DCSF, the application DC is established between the originating UE and terminating UE. If the selected DC application is not downloaded yet by the terminating UE, the terminating UE downloads the selected DC application via the bootstrap DC.

In this procedure, the assumption is that the alert to terminating user will only happen after the terminating UE successfully downloaded the selected application autonomously by UE#2. It is also assumed early media is used during that process. When the terminating UE alerts the user it displays the application information including the required rights to the user.

When the user of the terminating UE accepts to use the downloaded application for communication, the IMS data channel session is answered by the terminating UE.

If the UE#2 did not successfully download the DC Application, the UE#2, as an example, does not alert the user at all and tear down the entire IMS session.

The call flows in figure AC.10.2.3-1 depicts the above procedure.

Figure AC.10.2.3-1: P2P Combined Standalone Bootstrap DC and Application DC Setup

Editor's note:	The steps descriptions in the call flow depicting the above are FFS.

# *** Next Change ***

### AC.10.2.5	Removing MMTel Media from IMS Data Channel Session

When a non-stand alone IMS data channel session between two UEs is ongoing, any of the UE may be triggered by the corresponding user to remove audio/video media from the IMS session, which turns the IMS data channel session to standalone IMS data channel session.

Figure AC.10.2.5-1depicts a signalling flow diagram for removing audio/video media from IMS data channel session between two UEs.

Figure AC.10.2.5-1: Removing Audio/Video/Messaging Media from IMS data channel session

1.	An IMS DC session has been established between the UE-A and UE-B, which include audio/video and IMS DC media.

2.	The UE-A sends SIP re-INVITE request with an SDP offer containing media information for audio/video media and data channel, which sets the port number of audio/video media stream to zero, as specified in RFC 3264 [72].

3.	The IMS AS, the DCSF and the UE-B handles the SIP re-INVITE request.

4.	The UE-B sends 200 OK response to the UE-A through the IMS AS, indicating the audio/video media has been removed. The IMS AS sends notify message to the DCSF.

# *** End of Changes ***