3GPP TSG-WG2 Meeting #165	S2-2409993

# Hyderabad, India, October 14-18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0139 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing the ENs on geographical description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are two ENs in TS 23.256 about representation of geographical locations (i.e., flight path and a border-crossing point). -In clause 5.12 on “Pre-flight planning and in-flight monitoring”, there is the following EN: “what parameter e.g. polygon, is provided by the USS/UTM, to identify the UAV flight path, and how the network maps the parameter to the 3GPP defined area are FFS” that needs to be addressed. -In clause 4.4.1.1.3.3 (description of the Nnef_UAVFlightAssistance_Create service operation), there is an EN “Representation of the destination point and border crossing point format is FFS” that needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In regards of the first note (i.e., polygon for the representation of flight paths), polygon, as per definition in clause 6.1.6.2.9 of 3GPP TS 29.572, contain 2D information without height/altitude component. Therefore, it is not suitable for this purpose, instead it can be used to represent a border-crossing point. Considering the above, cuboids (or a set on cuboids) are more suitable (than, for instance, changing the definition of the polygon data type to include the height component) for representing a flight path. Hence, to address the EN on flight path representation, -adding a note to clarifty that: It is expected that the USS/UTM provides flight path information using a set of cuboids and, if available, corresponding timestamps/time schedule. Each of the cuboids is identified, e.g., by a set of the lowest/highest latitude, lowest/highest longitude and lowest/highest height above the sea level in which the UAV is supposed to fly from its starting point to its next point in the assigned flight path. In order to address the EN about border-crossing point representation: - adding a NOTE stating that: (i) destination point (or a next point in the flight path) can be represented using the exisiting data type, specifically, PointAltitudeUncertainty data type as specified in clause 6.1.6.2.11 of 3GPP TS 29.572; (ii) border-crossing point can be represented using using the Polygon data type (clause 6.1.6.2.9 of 3GPP TS 29.572); no height information needs to be considered/included. Additionally, -there is a need to add clarification/restriction that, when it comes to bordering tracking areas and border-crossing points, not more than one border-crossing point per TA is considered, otherwise, the 5GC will not be able to determine which border-crossing point will be used for the USS changeover when the UAV enters a bordering TA, imposng additional mechanisms to be added (or existing to be reused) to 5GC. Therefore, the proposal is to add a NOTE clarifying that, for the USS changeover purposes, there is not more than one border-crossing point per tracking area in this realease of the specification. -bullet#2 in clause 5.12 needs to be rephrased/corrected to emphasize that NEF does not derive a flight plan for a UAV, but it exploits the existing network analytics to collect the data and expose them to the USS/UTM for pre-flight planning. The USS/UTM makes the fligh planning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN remains unresolved in the TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.12.1, 5.13.2, 4.4.1.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[16]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[17]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[18]	3GPP TS 26.502: "5G multicast-broadcast services; User Service architecture".

[19]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[20]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[R2]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

>>>> Next Change <<<<

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a destination point (even when the destination point is located in geographical area served by different USS).

Input, Required: UAV's identifier (e.g. GPSI).

Input, Optional: Flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells).

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

>>>> Next Change <<<<

### 5.12.1	General

5G network may support pre-flight planning and in-flight monitoring for UAVs via the NEF service exposure. The USS/UTM may request assistance information to NEF of pre-flight planning and in-flight monitoring for UAV, after the UAV establishes user plane connection with the USS/UTM.

In the case of pre-flight planning, USS/UTM may request assistance information from the NEF for the following purposes:

(1)	To determine the most suitable flight path among several candidate flight paths:

USS/UTM provides multiple planned flight paths to the network, and network leverages the network analytics, e.g. Movement Behaviour Analytics and/or QoS Sustainability Analytics, to determine the most suitable (in terms of the provided criteria, e.g., fastest, shortest) planned flight path among the provided candidates.

NOTE X1:	It is expected that the USS/UTM provides flight path information using a set of cuboids and, if available, corresponding timestamps/time schedule. Each of the cuboids is identified, e.g., by a set of the lowest/highest latitude, lowest/highest longitude, and lowest/highest height above the sea level in which the UAV is supposed to fly from its starting point to its next point in the assigned flight path.

(2)	To derive a flight path(s) for the requested starting and destination points for a UAV’s flight:

USS/UTM provides to the network information about the UAV’s starting and destination points for the flight, and the network leverages the network analytics, e.g. Movement Behaviour Analytics and/or QoS Sustainability Analytics that can be exposed via NEF, to be used by the USS/UTM to determine a detailed flight path(s) for the UAV. NEF responds to the USS/UTM with the collected network analytics outputs to assist with the pre-flight planning.

NOTE X2:	Destination and/or a next point in the flight path can be described using the PointAltitudeUncertainty data type as specified in clause 6.1.6.2.11 of 3GPP TS 29.572 [R2].

(3)	Output QoS information along the flight path:

USS/UTM provides multiple planned flight paths to the network, and network leverages QoS Sustainability Analytics to output QoS information for each of the provided flight paths.

In the case of in-flight monitoring, in addition to QoS Sustainability Analytics request, USS/UTM can provide the UAV flight path and corresponding (arriving) time to the network, in order for the network to verify whether the UAV's exact location matches the scheduled location represented by the waypoint.

>>>> Next Change <<<<

### 5.13.2	Procedure for USS changeover during a UAV flight

Procedure for USS changeover in cases when a UAV moves over geographical areas served by different USS is shown in Figure 5.13.2-1.

NOTE 1:	This procedure can be used in conjunction with the NEF-assisted pre-flight planning as specified in clause 5.12.

NOTE 2:	USS changeover is independent of AMF(s) or NEF(s) changes, i.e. both a serving USS and a target USS can be served by the same 5GC NFs (e.g. AMF, NEF/UAS NF) during the entire UAV flight from the starting point to the next point in the flight path via the USS-determined flight path(s).

Figure 5.13.2-1: Procedure for UAV changeover from one USS to another serving different geographical areas

1.	The UAV establishes a PDU Session for communication with a serving USS (shown as USS 1 in Figure 5.13.2-1) as described in clause 5.2.3.

2a.	The UAV requests the pre-flight planning service from the serving USS, if needed. The request message includes an identifier of the UAV (e.g. GPSI, CAA-Level UAV ID), information of the starting and next point for the flight, requirements on the flight route (e.g. on time, shortest, highest, the farthest from a no-transmit zone) and may include candidate flight path(s) if available.

2b.	The serving USS determines the need for the changeover to another USS based on the information about UAV's next point in the flight path or notification from the core network (e.g. UAV location reporting events from AMF/GMLC/LMF when serving USS has subscribed to such events).

3.	The serving USS determines neighbouring USSs (one or more) that can serve as a target USS and triggers communication with all suitable target USSs (USS 2 in Figure 5.13.2-1) to determine candidate border-crossing point(s) for the UAV.

NOTE 3:	The serving USS determines suitable target USSs as well as a candidate border-crossing points between the serving USS and potential target USSs by means outside the scope of the 3GPP; for this purpose several criteria/metrics can be used, e.g. location of the next point in the flight path, area that a specific USS serves, a number of UAVs in the area served by a specific USS, proximity to no-transmit zones, USS load, etc.

Editor's note:	A term “border-crossing point” might need the revision to be more self-explanatory and is FFS.

NOTE X4:	A border-crossing point can be described using the Polygon data type described in clause 6.1.6.2.9 of 3GPP TS 29.572 [R2].

NOTE X5:	It is assumed not more than one border-crossing point per tracking area.

The serving USS in cooperation with the UTM derives any other information (e.g. acceptable deviations from the assigned flight plan/route) that can be used by 5GC to assist with the USS changeover and/or to provide flight planning assistance information.

NOTE 4:	The exact information that a USS can use for this purpose is outside the scope of the 3GPP, however, it can contain, for instance, information about UAV capability, geographical coordinates of the candidate border-crossing point(s), indication to reconnect to the network once the UAV enters a TA served by another USS, information about no-transmit zone (NTZ), etc.

4.	The USS invokes a Nnef_UAVFlightAssistance_Get request to the NEF/UAS NF to collect from the core network information for the changeover. Inside the request, the serving USS includes the information derived in step 3 (e.g. indication about USS changeover, list of addresses for suitable target USS, a list of candidate border-crossing point(s), acceptable deviations for flight plan/route), UAV's identifier (e.g. GPSI), and other parameters such as requirements on the flight path, candidate flight path(s), accuracy level of predictions relevant to the flight planning.

Editor's note:	Whether Nnef_UAVFlightAssistance_Get operation is needed or not is FFS.

NOTE 5:	In this procedure mentioned service operations can be reused for pre-flight planning procedure as described in clause 5.12.

5.	When the NEF/UAS NF receives the Nnef_UAVFlightAssistance_Get request from the serving USS with the indication about the USS changeover, the NEF/UAS NF translates/maps, if required, the parameters included in the USS request to 3GPP identifiers. For instance, the NEF determines a cell ID/tracking area identifier (TAI) of the cell/TA where the UAV-requested starting and next point in the flight path are located; similarly, if the request includes an indication that UAV will cross the USS-border and/or a list of the candidate border-crossing point(s) in form of geographical coordinates, the NEF maps them to a list of border cell IDs/TAIs.

The NEF determines the relevant NFs and specific service operations it needs to invoke to collect the required information for the serving USS for the purpose of UAV flight planning (e.g. NWDAF analytics service for Movement Behaviour analytics, GMLC service for Ranging/Sidelink Positioning location, AMF for UAV's presence in bordering TAs/cells, AMF service for UAV's deviation for the expected/assigned trajectory, UDM service for expected UE behaviour parameter provision).

6.	The NEF/UAS NF invokes service operations towards the identified NFs as described in Steps 8-12 of the procedure for the NEF-assisted pre-flight planning, see clause 5.12.

7.	If the NEF/UAS NF receives a list of candidate border-crossing points, the NEF/UAS NF identifies the AMFs serving NG-RAN nodes in all identified border TA(s)/cell(s); for that the NEF/UAS NF uses the Nnrf_NFDiscovery service from NRF. Once the AMF(s) information is retrieved, the NEF/UAS NF invokes an Namf_EventExposure_Subscribe request to subscribe to the UE/UAV's presence in area of interest wherein the area of interest is set to cell ID(s)/TAI(s) of the border cells(s)/TA(s).

8.	The NEF/UAS NF responds to the Nnef_UAVFlightAssistance_Get request with the collected information from the 5GC NFs for UAV's flight path between the starting point and all candidate USS border-crossing point(s).

9.	Based on the received information, the serving USS selects the target USS from the list of pre-selected suitable USSs (step 3) and starts communication to request the target USS (USS 2 in Figure 5.13.2-1) to prepare for the USS changeover and, if required, to perform the flight planning for the UAV from the border-crossing point(s) to the next point in the flight path that is located in the geographical area served by the target USS.

10.	The target USS (USS 2) performs Steps 4 - 8 to plan the UAV flight across its geographical area towards the UAV's next point in the flight path.

11.	The target USS (USS 2) provides the serving USS (USS 1) information about UAV's flight path(s) from a border cell(s)/TA(s) to the next point in the flight path (e.g. primary flight path, secondary/alternative flight path etc.).

12.	If the UAV requests a pre-mission flight planning in step 2a, the serving USS responds to this request with the planned flight routes and time schedule for the entire flight from the starting point to the next point in the flight path.

13.	The serving USS sends a Nnef_UAVFlightAssistance_Create request to the NEF/UAS NF with the information about the planned UAV flight path(s), see clause 4.4.1.1.3.3. In this request, the serving USS may also include additional flight path information for each of the segment of the paths (served by USS 1 and USS 2), for instance, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells.

14.	The NEF/UAS NF may invoke additional services with 5GC NFs: with AMF to determine the UAV deviation from the expected UAV's flight trajectory (e.g. primary/secondary flight paths); with UDM to update the Expected UE Behaviour parameters via the parameter provision; with the NWDAF for the Movement Behaviour analytics of the UE/UAV to determine whether the UAV will likely leave the USS area and continue following the flight path or not.

The expected UAV's flight trajectory may possibly consist of multiple segments where each of the segments are identified by geographical areas that are served by a different USSs (e.g. the currently serving USS and the target USS for the changeover).

14a.	The NEF/UAS NF may use the AMF event exposure service to get notified when the UAV does not follow (i.e. deviates from) the assigned flight plan. In this case, the NEF/UAS NF sends a Namf_EventExposure_Subscribe request to the serving AMF with the planned flight path information, which consists of a list of 3GPP locations (i.e. TAIs or cell IDs), a time schedule describing when the UE shall be present at these locations, height/altitude range and, optionally, acceptable deviations, e.g. the UAV not arriving on time.

Editor's note:	A CR needs to be prepared for TS 23.502 [3] to add a new AMF event exposure filter in Table 5.2.2.3.1-1 of TS 23.502 [3]. The new event filter needs to support reporting when a UAV moves between different trajectory segments of the UAV.

14b.	The NEF/UAS NF invokes the parameter provisioning service from the UDM (i.e. Nudm_ParameterProvision_Update, see clause 4.15.6.3 of TS 23.502 [3]) to provision the planned flight path information, which consists of a list of 3GPP locations (i.e. TAIs or cell IDs and, optionally, height/altitude information) together with the time schedule describing when the UE shall be present at these locations, optionally, acceptable deviations, e.g. UAV is not arriving on time or UAV flies below/above the assigned flight altitude.

14c.	The NEF/UAS NF invokes a Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe service operation towards the NWDAF in order to obtain the Movement Behaviour analytics of the UE/UAV as described in clause 6.21 of TS 23.288 [20].

15.	If the NEF/UAS NF subscribes (in step 7) to UAV's presence in the borders cell(s)/TA(s)s, the AMF notifies the NEF/UAS NF once the UAV enters a border cell/TA of the serving USS about the event (i.e. UAV enters the Area of Interest). Similarly, if the NEF/UAS NF subscribes for event reporting when the UAV deviating from the expected/assigned trajectory and/or moving across different trajectory segments, the AMF notifies the NEF/UAS NF when either of the events are detected.

16.	When the NEF/UAS NF receives a report from the AMF indicating the UAV is deviating the assigned flight plan, the NEF/UAS NF may invoke an Ngmlc_Location_ProvideLocation service request to a GMLC to get a accurate position of the UAV. In such case, the GMLC performs the 5G-MT-LR procedure to retrieve the accurate UAV location via AMF/LMF (as specified in TS 23.273 [8]) and provide the UAV location to the NEF/UAS NF in an Ngmlc_Location_ProvideLocation service response.

17.	If the NEF/UAS NF determines that the UAV is leaving the geographical area served by the serving USS (USS 1), the NEF/UAS NF sends a Nnef_UAVFlightAssistance_Notify (as described in clause 4.4.1.1.3.5) request to the serving USS in which it includes information about which/when border-crossing point will be used by the UAV so that the USS timely triggers the changeover.

18.	The serving USS communicates with the target USS to execute the changeover for the UAV; this communication and details are outside the scope of the 3GPP specification, however, it is expected that a serving USS pass the target USS the information on which exposure services/notifications are of relevance for the UAV (e.g. subscribed events and UAS NF address), UAV's identifiers (e.g. GPSI, CAA-Level UAV ID), and other information required by the target USS (or UAV itself) to establish the connection.

19.	The AMF serving a border TA/cell on the target USS's side (USS 2) notifies a serving NEF/UAS NF about the UAV's presence in the border TA/cell, and the NEF/UAS NF invokes the Nnef_UAVFlightAssistance_Notify service operation to further notify the target USS.

20.	The serving USS (USS 1) informs the UAV about the changeover to the new USS (USS 2) and, if required, may instruct the UAV to execute the UUAA procedure with the new USS (USS 2) as well as to trigger the exposure services towards 5GC NFs similar to what the previous USS had before the changeover.

NOTE 6:	Steps 19 and 20 can continue in parallel and not intended to imply sequential processing.

21.	After receiving the required information from the serving USS (the exact details are outside the scope of the 3GPP specification), the target USS responds to the Nnef_UAVFlightAssistance_Notify request from the NEF/UAS NF in such a way completing the changeover for the UAV.

>>>> End of Changes <<<<