3GPP TSG-WG2 Meeting #165	S2-2409995

# Hyderabad, India, October 14-18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0141 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing the EN on Nnef_UAVFlightAssistance_Get service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 4.4.1.1.3.1 on “Nnef_UAVFlightAssistance service”, there is the following EN: “Whether the GET operation is needed is FFS” that needs to be addressed. This EN concerns two service operations: (1) Nnef_UAVFlightAssistance_Get and (2) Nnef_UAVFlightAssistance_Create that both are used in the USS changeover procedure (clause 5.13.2) but for different purposes. -In step 4, the USS invokes a Nnef_UAVFlightAssistance_Get request to the NEF/UAS NF to collect from the 5GC NFs information. The information will be delivered to the USS in Nnef_UAVFlightAssistance_Get response and used by the serving USS to selects the target USS from the list of pre-selected suitable USSs and to start communication in order to request from the target USS to prepare for the USS changeover and, if required, to perform the flight planning for the UAV from the border-crossing point(s) to the next point in the flight path that is located in the geographical area served by the target USS. Basically, this _Get service information is just used to retrieve information for the USS. -In step 13, the serving USS uses a Nnef_UAVFlightAssistance_Create request to the NEF/UAS NF to send the information about the planned UAV flight path(s) and, if determined, to include some additional flight path information for each of the segment of the paths, for instance, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells. Moreover, the create request creates the explicit subscription to event that USS is expected to get notified via Nnef_UAVFlightAssistance_Notify service operation (steps 17, 19, 21 in clause 5.13.2). -Consequently, these two service operations are used completely for different purposes (and to deliver/request different parameters). Thus, instead of having the same service (Nnef_UAVFlightAssistance_), Nnef_UAVFlightAssistance_Get needs to be separated out and a new service created (Nnef_RetrieveInfoUAVFlight_Get) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following (main) chages have been made by this CR: 1) replacing “Nnef_UAVFlightAssistance_Get” with “Nnef_RetrieveInfoUAVFlight_Get” in such as way creating a new NEF service (incl. updating the service description and USS changeover procedure accordingly) 2) Clarifying for the Nnef_UAVFlightAssistance_Create service operation that the request creates the explicit subscriptions to get notified about the following events, when applicable, (1) the UAS NF/NEF determines that UAV will leave a geographical area served by the USS, (2) the UAV enters a TA/Cell with a border-crossing point to another USS, and (3) the UAV deviates from the assigned flight plan. 3) Adding new service operation, i.e., Nnef_RetrieveInfoUAVFlight_Delete; 4) clarifying in the description of the USS changeover procedure (clause 5.13.2) that step 2a and Step 12 are over the application layer; 5) Adding Step 22 in the USS changeover procedure so that the new USS could inform the UAS NF about new Notification Target Address; 6) deleted the EN about a new AMF event filter, as the corresponding CR has been agreed at SA2#164 (see S2-2409514) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN remains unresolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3, 4.4.1.1.2, 4.4.1.1.X (new), 5.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

##### 4.4.1.1.1	General

In addition to those defined in TS 23.501 [2] clause 7.2.8 and TS 23.502 [3] clause 5.2.6, the following table illustrates additional NEF services to support UAS.

Table 4.4.2.1.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_Authentication | AuthenticateAuthorize | Request/Response | AMF, SMF |
|  | Notification | Subscribe/Notify | AMF, SMF |
| Nnef_UAVFlightAssistance | Delete | Request/Response | AF/USS |
|  | Create | Request/Response | AF/USS |
|  | Update | Request/Response | AF/USS |
|  | Notify | Request/Response | AF/USS |
| Nnef_RetrieveInfoUAVFlight | Get | Request/Response | AF/USS |

>>>> Next Change <<<<

##### 4.4.1.1.3	Nnef_UAVFlightAssistance service

4.4.1.1.3.1	General

Service Description: This service enables the consumer to request assistance for USS changeover if/when a UAV/UE moves from a geographical area served by one USS to an area served by another USS. For this purpose, a USS requests a core network to provide the assistance with the USS changeover, to inform the core network about the flight trajectory assigned and time schedule for the UAV/UE, and to receive notifications when the UAV/UE moves out/in the geographical area served by a USS triggering and completing the changeover.

4.4.1.1.3.2	Nnef_UAVFlightAssistance_Delete service operation

Service operation name: Nnef_UAVFlightAssistance_Delete

Description: Used by the service consumer to delete the previously provided information about UAV flight path(s) and/or time schedule and to cancel the associated subscriptions. The consumer can use this service operation after a USS changeover in cases when a new USS do not reuse the flight context created by the old USS or when the serving USS determines that it does not need the assistance from the 5GS.

Input, Required: Flight plan configuration ID.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a next point in the flight plath (even when the next point is located in geographical area served by different USS). The request creates the explicit subscriptions to get notified about the following events, when applicable, (1) the UAS NF/NEF determines that UAV will leave a geographical area served by the USS, (2) the UAV enters a TA/Cell with a border-crossing point to another USS, and (3) the UAV deviates from the assigned flight plan.

Editor's note:	Representation of the destination point and border crossing point format is FFS.

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the request (i.e., USS changeover, pre-flight planning, in-flight monitoring).

Input, Optional: Flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells), Notification Target Address.

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

4.4.1.1.3.4	Nnef_UAVFlightAssistance_Update service operation

Service operation name: Nnef_UAVFlightAssistance_Update

Description: Used by the service consumer to update the core network about changes in the previously provided UAV flight path(s) and/or time schedule.

Input, Required: Flight plan configuration ID.

Input, Optional: Flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells), Notification Target Address.

Output, Required: None.

Output, Optional: None.

4.4.1.1.3.5	Nnef_UAVFlightAssistance_Notify service operation

Service operation name: Nnef_UAVFlightAssistance_Notify

Description: Notifying a USS about UAV leaving its serviced area and which border crossing point will be used for this purpose, informing a target USS about the UAV entering its serviced area, and confirming the completion of the changeover.

Input, Required: Flight plan configuration ID, status of the UAV according to the assigned flight path (leaving, entering, deviating).

Input, Optional: Information about border-crossing point that will be used for USS changeover.

Output, Required: Indication whether the USS changeover is completed successfully when used by the target USS when responding to the notification.

Output, Optional: None.

>>>> Next Change <<<<

##### 4.4.1.1.X	Nnef_RetrieveInfoUAVFlight_Get service

4.4.1.1.X.1	General

Service Description: This service enables the consumer (i.e., USS) to request the 5GC to collect data from NFs to be used by the USS for UAV’s flight planning and assistance purposes, for instance, USS changeover and/or pre-flight planning.

4.4.1.1.X.2	Nnef_RetrieveInfoUAVFlight_Get service operation

Service operation name: Nnef_ RetrieveInfoUAVFlight_Get

Description: Requesting from a NEF/UAS NF to retrieve data from 5GC NFs that can be used by USS for UAV’s flight purposes.

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the retrieved information (i.e., USS changeover, pre-fligh planning).

Input, Optional: list of suitable target USSes, a list of candidate border-crossing point(s), acceptable deviations for flight plan/route, information about candidate flight path(s), requirements on the flight path as described in clause 5.12.

Output, Required: Report containing a Report ID and the retrieved information.

Output, Optional: None.

>>>> Next Change <<<<

### 5.13.2	Procedure for USS changeover during a UAV flight

Procedure for USS changeover in cases when a UAV moves over geographical areas served by different USS is shown in Figure 5.13.2-1.

NOTE 1:	This procedure can be used in conjunction with the NEF-assisted pre-flight planning as specified in clause 5.12.

NOTE 2:	USS changeover is independent of AMF(s) or NEF(s) changes, i.e. both a serving USS and a target USS can be served by the same 5GC NFs (e.g. AMF, NEF/UAS NF) during the entire UAV flight from the starting point to the next point in the flight path via the USS-determined flight path(s).

AMF

AMF

GMLC

GMLC

NEF / UAS NF

NEF / UAS NF

USS2

USS2

UAV

UAV

NWDAF

NWDAF

USS1

USS1

1. PDU session establishment

1. PDU session establishment

2a. Pre-flight planning request as specified in clause 5.12.2

2a. Pre-flight planning request as specified in clause 5.12.2

2b. Determining the need for a changeover to another USS

2b. Determining the need for a changeover to another USS

3. Determines information to request from the 5GC, pre-selects suitable target USS and triggers communication with them

3. Determines information to request from the 5GC, pre-selects suitable target USS and triggers communication with them

4. Nnef_RetrieveInfoUAVFlight_Get request as specified in clause 4.4.1.1.X.2

4. Nnef_RetrieveInfoUAVFlight_Get request as specified in clause 4.4.1.1.X.2

5. Determining services and NFs required to provide the assistance with the USS changeover and/or flight planning

5. Determining services and NFs required to provide the assistance with the USS changeover and/or flight planning

6. NEF-assisted pre-flight planning as specified in clause 5.12.2

6. NEF-assisted pre-flight planning as specified in clause 5.12.2

7. Namf_EventExposure_Subscribe request for UAV presence in the border TAs/cells

7. Namf_EventExposure_Subscribe request for UAV presence in the border TAs/cells

8. Nnef_RetrieveInfoUAVFlight_Get response as specified in clause 4.4.1.1.X.2

8. Nnef_RetrieveInfoUAVFlight_Get response as specified in clause 4.4.1.1.X.2

9. Selection of the target USS and flight planning by the target USS to the next point in the flight path

9. Selection of the target USS and flight planning by the target USS to the next point in the flight path

10. Steps 4 – 8 by the target USS

10. Steps 4 – 8 by the target USS

11. The target USS provides the serving USS information about its leg of the UAV flight path

11. The target USS provides the serving USS information about its leg of the UAV flight path

12. Response to the UAV’s request for the pre-flight planning

12. Response to the UAV’s request for the pre-flight planning

13. Nnef_UAVFlightAssistance_Create request with information about the planned flight routes

13. Nnef_UAVFlightAssistance_Create request with information about the planned flight routes

14. Additional services with 5GC NFs.

14a. Namf_EventExposure_Subscribe request for UAV deviation from the expected trajectory;

14b. UDM parameter provision for “Expected UE Behaviour” update as specified in clause 4.15.6.3 of TS 23.502 [3];

14c. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe for the Movement Behaviour analytics of the UE/UAV from NWDAF as described in clause 6.21 of TS 23.288 [20]

14. Additional services with 5GC NFs.

14a. Namf_EventExposure_Subscribe request for UAV deviation from the expected trajectory;

14b. UDM parameter provision for “Expected UE Behaviour” update as specified in clause 4.15.6.3 of TS 23.502 [3];

14c. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe for the Movement Behaviour analytics of the UE/UAV from NWDAF as described in clause 6.21 of TS 23.288 [20]

15. Namf_EventExposure_Notify about an event

15. Namf_EventExposure_Notify about an event

16. Ngmlc_Location_ProvideLocation request/response

16. Ngmlc_Location_ProvideLocation request/response

17. Determining whether the UAV leaves the area served by the USS and sending the notification about the event

17. Determining whether the UAV leaves the area served by the USS and sending the notification about the event

18. The serving USS triggers the changeover to the target USS

18. The serving USS triggers the changeover to the target USS

19. Notifying when the UAV enters the geographic areas served by the target USS

19. Notifying when the UAV enters the geographic areas served by the target USS

20. Informing the UAV about a new USS and, if required, re-invoking exposure services from the relevant NFs

20. Informing the UAV about a new USS and, if required, re-invoking exposure services from the relevant NFs

21. Completion of the changeover between the serving USS and the target USS

21. Completion of the changeover between the serving USS and the target USS

22. Nnef_UAVFlightAssistance_Update request with information about Notification Target Address

22. Nnef_UAVFlightAssistance_Update request with information about Notification Target Address

Figure 5.13.2-1: Procedure for UAV changeover from one USS to another serving different geographical areas

1.	The UAV establishes a PDU Session for communication with a serving USS (shown as USS 1 in Figure 5.13.2-1) as described in clause 5.2.3.

2a.	(via application layer) The UAV requests the pre-flight planning service from the serving USS, if needed. The request message includes an identifier of the UAV (e.g. GPSI, CAA-Level UAV ID), information of the starting and next point for the flight, requirements on the flight route (e.g. on time, shortest, highest, the farthest from a no-transmit zone) and may include candidate flight path(s) if available.

2b.	The serving USS determines the need for the changeover to another USS based on the information about UAV's next point in the flight path or notification from the core network (e.g. UAV location reporting events from AMF/GMLC/LMF when serving USS has subscribed to such events).

3.	The serving USS determines neighbouring USSs (one or more) that can serve as a target USS and triggers communication with all suitable target USSs (USS 2 in Figure 5.13.2-1) to determine candidate border-crossing point(s) for the UAV.

NOTE 3:	The serving USS determines suitable target USSs as well as a candidate border-crossing points between the serving USS and potential target USSs by means outside the scope of the 3GPP; for this purpose several criteria/metrics can be used, e.g. location of the next point in the flight path, area that a specific USS serves, a number of UAVs in the area served by a specific USS, proximity to no-transmit zones, USS load, etc.

The serving USS in cooperation with the UTM derives any other information (e.g. acceptable deviations from the assigned flight plan/route) that can be used by 5GC to assist with the USS changeover and/or to provide flight planning assistance information.

NOTE 4:	The exact information that a USS can use for this purpose is outside the scope of the 3GPP, however, it can contain, for instance, information about UAV capability, geographical coordinates of the candidate border-crossing point(s), indication to reconnect to the network once the UAV enters a TA served by another USS, information about no-transmit zone (NTZ), etc.

4.	The USS invokes an Nnef_RetrieveInfoUAVFlight_Get request to the NEF/UAS NF to collect from the core network information for the changeover. Inside the request, the serving USS includes the information derived in step 3 (e.g. indication about USS changeover, list of addresses for suitable target USS, a list of candidate border-crossing point(s), acceptable deviations for flight plan/route), UAV's identifier (e.g. GPSI), and other parameters such as requirements on the flight path, candidate flight path(s), accuracy level of predictions relevant to the flight planning.

NOTE 5:	In this procedure mentioned service operations can be reused for pre-flight planning procedure as described in clause 5.12.

5.	When the NEF/UAS NF receives the Nnef_UAVFlightAssistance_Get request from the serving USS with the indication about the USS changeover, the NEF/UAS NF translates/maps, if required, the parameters included in the USS request to 3GPP identifiers. For instance, the NEF determines a cell ID/tracking area identifier (TAI) of the cell/TA where the UAV-requested starting and next point in the flight path are located; similarly, if the request includes an indication that UAV will cross the USS-border and/or a list of the candidate border-crossing point(s) in form of geographical coordinates, the NEF maps them to a list of border cell IDs/TAIs.

The NEF determines the relevant NFs and specific service operations it needs to invoke to collect the required information for the serving USS for the purpose of UAV flight planning (e.g. NWDAF analytics service for Movement Behaviour analytics, GMLC service for Ranging/Sidelink Positioning location, AMF for UAV's presence in bordering TAs/cells, AMF service for UAV's deviation for the expected/assigned trajectory, UDM service for expected UE behaviour parameter provision).

6.	The NEF/UAS NF invokes service operations towards the identified NFs as described in Steps 8-12 of the procedure for the NEF-assisted pre-flight planning, see clause 5.12.

7.	If the NEF/UAS NF receives a list of candidate border-crossing points, the NEF/UAS NF identifies the AMFs serving NG-RAN nodes in all identified border TA(s)/cell(s); for that the NEF/UAS NF uses the Nnrf_NFDiscovery service from NRF. Once the AMF(s) information is retrieved, the NEF/UAS NF invokes an Namf_EventExposure_Subscribe request to subscribe to the UE/UAV's presence in area of interest wherein the area of interest is set to cell ID(s)/TAI(s) of the border cells(s)/TA(s).

8.	The NEF/UAS NF responds to the Nnef_RetrieveInfoUAVFlight_Get request with the collected information from the 5GC NFs for UAV's flight path between the starting point and all candidate USS border-crossing point(s).

9.	Based on the received information, the serving USS selects the target USS from the list of pre-selected suitable USSs (step 3) and starts communication to request the target USS (USS 2 in Figure 5.13.2-1) to prepare for the USS changeover and, if required, to perform the flight planning for the UAV from the border-crossing point(s) to the next point in the flight path that is located in the geographical area served by the target USS.

10.	The target USS (USS 2) performs Steps 4 - 8 to plan the UAV flight across its geographical area towards the UAV's next point in the flight path.

11.	The target USS (USS 2) provides the serving USS (USS 1) information about UAV's flight path(s) from a border cell(s)/TA(s) to the next point in the flight path (e.g. primary flight path, secondary/alternative flight path etc.).

12.	(via application layer) If the UAV requests a pre-mission flight planning in step 2a, the serving USS responds to this request with the planned flight routes and time schedule for the entire flight from the starting point to the next point in the flight path.

13.	The serving USS sends an Nnef_UAVFlightAssistance_Create request to the NEF/UAS NF with the information about the planned UAV flight path(s), see clause 4.4.1.1.3.3. In this request, the serving USS may also include additional flight path information for each of the segment of the paths (served by USS 1 and USS 2), for instance, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells.

14.	The NEF/UAS NF may invoke additional services with 5GC NFs: with AMF to determine the UAV deviation from the expected UAV's flight trajectory (e.g. primary/secondary flight paths); with UDM to update the Expected UE Behaviour parameters via the parameter provision; with the NWDAF for the Movement Behaviour analytics of the UE/UAV to determine whether the UAV will likely leave the USS area and continue following the flight path or not.

The expected UAV's flight trajectory may possibly consist of multiple segments where each of the segments are identified by geographical areas that are served by a different USSs (e.g. the currently serving USS and the target USS for the changeover).

14a.	The NEF/UAS NF may use the AMF event exposure service to get notified when the UAV does not follow (i.e. deviates from) the assigned flight plan. In this case, the NEF/UAS NF sends a Namf_EventExposure_Subscribe request to the serving AMF with the planned flight path information, which consists of a list of 3GPP locations (i.e. TAIs or cell IDs), a time schedule describing when the UE shall be present at these locations, height/altitude range and, optionally, acceptable deviations, e.g. the UAV not arriving on time.

14b.	The NEF/UAS NF invokes the parameter provisioning service from the UDM (i.e. Nudm_ParameterProvision_Update, see clause 4.15.6.3 of TS 23.502 [3]) to provision the planned flight path information, which consists of a list of 3GPP locations (i.e. TAIs or cell IDs and, optionally, height/altitude information) together with the time schedule describing when the UE shall be present at these locations, optionally, acceptable deviations, e.g. UAV is not arriving on time or UAV flies below/above the assigned flight altitude.

14c.	The NEF/UAS NF invokes a Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe service operation towards the NWDAF in order to obtain the Movement Behaviour analytics of the UE/UAV as described in clause 6.21 of TS 23.288 [20].

15.	If the NEF/UAS NF subscribes (in step 7) to UAV's presence in the borders cell(s)/TA(s)s, the AMF notifies the NEF/UAS NF once the UAV enters a border cell/TA of the serving USS about the event (i.e. UAV enters the Area of Interest). Similarly, if the NEF/UAS NF subscribes for event reporting when the UAV deviating from the expected/assigned trajectory and/or moving across different trajectory segments, the AMF notifies the NEF/UAS NF when either of the events are detected.

16.	When the NEF/UAS NF receives a report from the AMF indicating the UAV is deviating the assigned flight plan, the NEF/UAS NF may invoke an Ngmlc_Location_ProvideLocation service request to a GMLC to get a accurate position of the UAV. In such case, the GMLC performs the 5G-MT-LR procedure to retrieve the accurate UAV location via AMF/LMF (as specified in TS 23.273 [8]) and provide the UAV location to the NEF/UAS NF in an Ngmlc_Location_ProvideLocation service response.

17.	If the NEF/UAS NF determines that the UAV is leaving the geographical area served by the serving USS (USS 1), the NEF/UAS NF sends a Nnef_UAVFlightAssistance_Notify (as described in clause 4.4.1.1.3.5) request to the serving USS in which it includes information about which/when border-crossing point will be used by the UAV so that the USS timely triggers the changeover.

18.	The serving USS communicates with the target USS to execute the changeover for the UAV; this communication and details are outside the scope of the 3GPP specification, however, it is expected that a serving USS pass the target USS the information on which exposure services/notifications are of relevance for the UAV (e.g. subscribed events and UAS NF address), UAV's identifiers (e.g. GPSI, CAA-Level UAV ID), and other information required by the target USS (or UAV itself) to establish the connection.

19.	The AMF serving a border TA/cell on the target USS's side (USS 2) notifies a serving NEF/UAS NF about the UAV's presence in the border TA/cell, and the NEF/UAS NF invokes the Nnef_UAVFlightAssistance_Notify service operation to further notify the target USS.

20.	The serving USS (USS 1) informs the UAV about the changeover to the new USS (USS 2) and, if required, may instruct the UAV to execute the UUAA procedure with the new USS (USS 2) as well as to trigger the exposure services towards 5GC NFs similar to what the previous USS had before the changeover.

NOTE 6:	Steps 19 and 20 can continue in parallel and not intended to imply sequential processing.

21.	After receiving the required information from the serving USS (the exact details are outside the scope of the 3GPP specification), the target USS responds to the Nnef_UAVFlightAssistance_Notify request from the NEF/UAS NF in such a way completing the changeover for the UAV.

22. The new USS (i.e., the target USS) sends an Nnef_UAVFlightAssistance_Update request to the NEF/UAS NF with the information about a new Notification Target Address to receive notifications from the subscription invoked by the previous serving USS.

>>>> End of Changes <<<<